C64DX SYSTEM SPECIFICATION

o Design Concepts

o Hardware Specifications

o Software Specifications

Requires ROM Version 0.9A.910228 or later.

COPYRIGHT 1991 COMMODORE BUSINESS MACNINES, INC.

ALL RIGHTS RESERVED.

INFORMATION  CONTAINED  HEREIN  IS  THE  UNPUBLISHED  AND CONFIDENTIAL

PROPERTY  OF  COMMODORE  BUSINESS MACHINES, INC. USE, REPRODUCTION, OR

DISCLOSURE OF THIS INFORMATION WITHOUT THE PRIOR WRITTEN PERMISSION OF

COMMODORE IS PROHIBITED.

CCCC       666     555555

C    C     6        5

C         6         5

C         6         55555

C         66666     5    5

C         6    6         5

C         6    6         5

C    C    6    6    5    5

CCCC      6666      5555

Copyright 1991 Commodore Business Machines, Inc.

All Rights Reserved.

This documentation contains confidential, proprietary, and unpublished

information  of  Commodore  Business  Machines, Inc. The reproduction,

dissemination, disclosure or translation of this information to others

without the prior written consent of Commodore Business Machines, Inc.

is strictly prohibited.

Notice  is  hereby given that the works of authorship contained herein

are  owned  by  Commodore  Business  Machines,  Inc.  pursuant to U.S.

Copyright Law, Title 17 U.S.C. 3101 et. seq.

This system specification reflects the latest information available at

this  time.  Updates  will  occur  as  the  system  evolves. Commodore

Business Machines, Inc. makes no warranties, expressed or implied with

regard  to  the  information  contained  herein including the quality,

performance,  merchantability,  or  fitness of this information or the

system as described.

This system specification contains the contributions of several people

including: Fred Bowen, Paul Lassa, Bill Gardei, and Victor Andrade.

Portions of the BASIC ROM code are Copyright 1977 Microsoft.

PPPP   RRRR   EEEE  L     I  M   M  I  N   N    A    RRRR  Y   Y

P   P  R   R  E     L     I  MM MM  I  NN  N   A A   R   R  Y Y

PPPP   RRRR   EEE   L     I  M M M  I  N N N  AAAAA  RRRR    Y

P      R   R  E     L     I  M   M  I  N  NN  A   A  R   R   Y

P      R   R  EEEE  LLLL  I  M   M  I  N   N  A   A  R   R   Y

Revision 0.2   (pilot release)         January 31, 1991

At  this  time,  Pilot  Production,  the C65 system consists of either

revision 2A or 2B PCB, 4510R3, 4567R5 (PAL only), F011B/C FDC, and 018

DMAgic  chips.  There  will  be  changes  to  all  these  chips before

Production Release.

This work is by:

Fred Bowen              System Software -   C65

Paul Lassa              Hardware engineer - C65, DMagic

Bill Gardei             LSI engineer -      4567, FDC

Victor Andrade          LSI engineer -      4510

Included  are  contributions by contractors hired by Commodore for the

C65 project. These contributions include the DOS, Graphics, Audio, and

Memory management areas.

Several 4502 assembler systems are available:

VAX, Amiga, and PC based BSO-compatible cross assemblers.

PC based custom cross assembler by Memocom, compatible

with Memocom 4502 emulator and Mem-ulator systems.

C128-based BSO compatible cross assembler by Commodore.

Custom   software   support  is  available  for  the  following  logic

analyzers:

Hewlett Packard HP655x A and B logic analyzers.

Table of Contents

-----------------

1.0. Introduction

1.1. System Concept

1.2. System Overview

1.3. System Components

1.4. System Concerns

1.4.1. C64 Compatibility

1.4.1.1. Software

1.4.1.2. Hardware

1.4.2. 1581 DOS Compatibility

1.4.3. Modes of Operation

1.5. System Maps

1.5.1. Composite System Memory Map

1.5.2. C65 System Memory Map

1.5.3. C65 System Memory Layout

1.5.4. C65 I/O Memory Map

2.0. System Hardware

2.1. Keyboard

2.1.1. Keyboard Layout

2.1.2. Keyboard Matrix

2.2. External Ports & Form-Factor

2.3. Microcontroller

2.3.1. Description

2.3.2. Configuration

2.3.3. Functional Description

2.3.3.1. Pin Description

2.3.3.2. Timing Description

2.3.3.3. Register Description

2.3.4. Mapper

2.3.5. Peripheral Control

2.3.5.1. I/O Ports

2.3.5.2. Handshaking

2.3.5.3. Timers

2.3.5.4. TOD Clocks

2.3.5.5. Serial Ports

2.3.5.6. Fast Serial Ports

2.3.5.7. Interrupt Control

2.3.5.8. Control Registers

2.3.6. UART

2.3.6.1. Control Registers

2.3.6.2. Status Register

2.3.6.3. Character Configuration

2.3.6.4. Register Map

2.3.7. CPU

2.3.7.1. Introduction

2.3.7.2. CPU Operation

2.3.7.3. Interrupt Handling

2.3.7.4. Addressing Modes

2.3.7.5. Instruction Set

2.3.7.6. Opcode Table

2.4. Video Controller

2.4.1. Description

2.4.2. Configuration

2.4.3. Functional Description

2.4.4. Programming

2.4.5. Registers

2.5. Disk Controller

2.5.1. Description

2.5.2. Configuration

2.5.3. Registers

2.5.4. Functional Description

2.5.5. Expansion port protocol

2.5.6. Timing diagrams

2.6. Expansion Disk Controller (option)

2.6.1. Description

2.6.2. Expansion port protocol

2.7. DMAgic Controller

2.7.1. Description

2.7.2. Registers

2.8. RAM Expansion Controller (option)

2.8.1. Description

2.9. Audio Controller

3.0. System Software

3.1. BASIC 10.0

3.1.1. Introduction

3.1.2. List of Commands

3.1.3. Command Descriptions

3.1.4. Variables

3.1.5. Operators

3.1.6. Error Messages

3.1.6.1. BASIC Error Messages

3.1.6.2. DOS Error Messages

3.2. Monitor

3.2.1. Introduction

3.2.2. Commands and Conventions

3.2.3. Command Descriptions

3.3. Editor

3.3.1. Escape Sequences

3.3.2. Control Characters

3.4. Kernel

3.4.1. Kernel Jump Table

3.4.2. BASIC Jump Table

3.4.3. Editor Jump Table

3.4.4. Indirect Vectors

3.4.5. Kernel Documentation

3.4.6. BASIC Math Package Documentation

3.4.7. I/O Devices

3.5. DOS

3.6. RS-232

4.0. Development Support

1.0. Introduction

This  specification  describes  the  requirements for a low-cost 8-bit

microcomputer system with excellent graphic capabilities.

1.1. System Concept

The  C65  microcomputer  is a low-cost, versatile, competitive product

designed for the international home computer and game market.

The C65 is well suited for first time computer buyers, and provides an

excellent  upgrade  path  for  owners  of  the commercially successful

C64. The C65 is composed of concepts inherent in the C64 and C128.

The  purpose of the C65 is to modernize and revitalize the 10 year old

C64  market  while still taking advantage of the developed base of C64

software.  To  accomplish  this,  the  C65  will provide a C64 mode of

operation,  offering a reasonable degree of C64 software compatibility

and a moderate degree of add-on hardware and peripheral compatibility.

Compatibility can be sacrificed when it impedes enhanced functionality

and expandability, much as the C64 sacrificed VIC-20 compatibility.

It  is  anticipated that the many features and capabilities of the new

C65  mode  will  quickly  attract  the  attention  of  developers  and

consumers  alike,  thereby  revitalizing  the  low-end  home  computer

market.  The  C65  incorporates  features  that  are normally found on

today's more expensive machines, continuing the Commodore tradition of

maximizing  performance  for  the price. The C65 will provide many new

opportunities  for  third  party  software  and  hardware  developers,

including  telecommunications,  video,  instrument  control (including

MIDI), and productivity as well as entertainment software.

1.2. System Overview

o  CPU -- Commodore CSG4510 running at 1.02 or 3.5 Mhz

o  New instructions, including Rockwell and GTE extensions

o  Memory Mapper supporting up to 1 Megabyte address space

o  R6511-type UART (3-wire RS-232) device, programmable baud

rate (50-56K baud, MIDI-capable), parity, word size, sync

and async. modes.  XD/RD wire ORed/ANDed with user port.

o  Two CSG6526-type CIA devices, each with 2 I/O ports

programmable TOD clocks, interval timers, interrupt control

o  Memory

o  RAM -- 128K bytes (DRAM)

Externally expandable from additional 512K bytes to 4MB

using dedicated RAM expansion port.

o  ROM -- 128K bytes

C64 Kernel and BASIC 2.2

C65 Kernel, Editor, BASIC 10.0, ML Monitor (like C128)

DOS v10 (1581 subset)

Multiple character sets: 40 and 80 column versions

National keyboards/charsets for foreign language systems

Externally expandable by conventional C64 ROM cartridges

via cartridge/expansion port using C64 decodes.

Externally expandable by additional 128K bytes or more

via cartridge/expansion port using new system decodes.

o  DMA -- Custom DMAgic controller chip built-in

Absolute address access to entire 8MB system map

including I/O devices, both ROM & RAM expansion ports.

List-based DMA structures can be chained together

Copy (up,down,invert), Fill, Swap, Mix (boolean Minterms)

Hold, Modulus (window), Interrupt, and Resume modes,

Block operations from 1 byte to 64K bytes

DRQ handshaking for I/O devices

Built-in support for (optional) expansion RAM controller

o  Video -- Commodore CSG 4567 enhanced VIC chip

o   RGBA with sync on all colors or digital sync

o   Composite NTSC or PAL video, separate chroma/luma

o   Composite NTSC or PAL digital monochrome

o   RF TV output via NTSC or PAL modulator

o   Digital foreground/background control (genlock)

o   All original C64 video modes:

40x25 standard character mode

Extended background color mode

320x200 bitmap mode

Multi-color mode

16 colors

8 sprites, 24x21

o   40 and 80 character columns by 25 rows:

Color, blink, bold, inverse video, underline attributes

o   True bitplane graphics:

320 x 200 x 256  (8-bitplane)  non-interlaced

640 x 200 x 16*  (4-bitplane)  non-interlaced

1280 x 200 x 4*   (2-bitplane)  non-interlaced

320 x 400 x 256  (8-bitplane)  interlaced

640 x 400 x 16*  (4-bitplane)  interlaced

1280 x 400 x 4*   (2-bitplane)  interlaced

*plus sprite and border colors

o   Color palettes:

Standard 16-color C64 ROM palette

Programmable 256-color RAM palette, with 16 intensity

levels per primary color (yielding 4096 colors)

o   Horizontal and vertical screen positioning verniers

o   Display Address Translator (DAT) allows programmer to

access bitplanes easily and directly.

o   Access to optional expansion RAM

o   Operates at either clock speed without blanking

o  Audio -- Commodore CSG8580 SID chips

o  Stereo SID chips:

Total of 6 voices, 3 per channel

Programmable ADSR envelope for each voice

Filter, modulation, audio inputs, potentiometer

Separate left/right volume, filter, modulation control

o  Disk, Printer support --

o  FDC custom MFM controller chip built in, with 512-byte

buffer, sector or full track read/write/format, LED and

motor control, copy protection.

o  Built-in 3.5" double sided, 1MB MFM capacity drive

o  Media & file system compatible with 1581 disk drive

o  Supports one additional "dumb" drive externally.

o  Standard CBM bus serial (all modes, about 4800 baud)

o  Fast serial bus (C65 mode only, about 20K baud)

o  Burst serial (C65 mode only, about 50K baud)

o  External ports --

o  50-pin Cartridge/expansion port (ROM cartridges, etc.)

o  24-pin User/parallel port (modem (1670), RS-232 serial)

o  Composite video/audio port (8-pin DIN)

o  Analog RGB video port (DB-9)

o  RF video output jack

o  Serial bus port (disks (1541/1571/1581), printers, etc.)

o  External floppy drive port (mini DIN8)

o  2 DB9 control ports (joystick, mouse, tablets, lightpen)

o  Left and right stereo audio output jacks

o  RAM expansion port, built-in support for RAM controller

o  Keyboard -- 77 keys, including standard C64 keyboard plus:

o  Total of 8 function keys, F1-F16, shifted and nonshifted

o  TAB, escape, ALT, CAPS lock, no scroll, help (F15/16)

o  Power, disk activity LEDs

o  Power supply -- external, brick type

o  +5VDC at 2.2A  and  +12VDC at .85A

1.3. System Components

Microcontroller:     4510 (65CE02, 2x6526, 6511 UART, Mapper,

Fast serial)

Memory:              4464   DRAM (128K bytes)

271001 ROM  (128K bytes)

Video controller:    4567 (extended VIC, DAT, PLA)

Audio controllers:   6581 (SID)

Memory control:      41xx-F018 (DMA)

Disk controller:     41xx-F011 (FDC, supports 2 DSDD drives, MFM,

RAM buffer)

KEYS

+ USER PORT

| + CONTROL PORTS                      EXPANSION PORT

| | +                                      + + +  +---+

| | |                                      | | |  |   | +MOD-> RFOUT

++-+-++                                     | | |  |   +-+----> COMP,CHROMA/LUMA

|     |                                     | | |  |   +------> RGBA

|     +-------------------------------------+------+   |      +---+

|     +--------------------------------------------+   +--...-+ R +--+

|     |                                       | |  |   |        E    | EXPANSION

|     |     +---+  +---+  +---+  +---+  +---+ | |  |   +--...-+ C +--+   MEMORY

|  4  |     |   |  |   |  |   |  |   |  |   | | |  | 4 |      +---+

|  5  +-----+ D +--+ F +--+ S +--+ S +--+ R +-+----+ 5 |  +--+  +--+  +--+  +--+

|  1  +-----+ M +--+ D +--+ I +--+ I +--+ O +------+ 6 |  |  |  |  |  |  |  |  |

|  0  | ADR | A |  | C |  | D |  | D |  | M |   |  | 7 +--+  +--+  +--+  +--+  |

|     |     | G |  |   |  |   |  |   |  |   |   |  |   +--+  +--+  +--+  +--+  |

|     +-----+ I +--+   +--+   +--+   +--+   +---+--+   +--+  +--+  +--+  +--+  |

|     +-----+ C +--+   +--+   +--+   +--+   +------+   +--+  +--+  +--+  +--+  |

|     | DAT |   |  |   |  |   |  |   |  |   |      |   |  |  |  |  |  |  |  |  |

+--+--+     +---+  +++-+  +-+-+  +-+-+  +---+      +---+  +--+  +--+  +--+  +--+

|                ||      |      |                               128K

+                ||      R      L                           RAM INTERNAL

SERIAL BUS          ||      SPEAKERS

++

FLOPPY PORT

1.4. System Concerns

1.4.1. C64 Compatibility Issues

1.4.1.1. Software

C64 software compatibility is an important goal. To this end, when the

system  is in "C64 mode" the processor will operate at average 1.02MHz

speed  and  dummy "dead" cycles are emulated by the processor. The C64

ROM  is  the  same  except  for  patches to serial bus routines in the

kernel  (to  interface  built-in  drive), the removal of cassette code

(there  is  no  cassette  port), and patches to the C64 initialization

routines  to  boot C65 mode if there is no reason (eg., cartridges) to

stay in C64 mode.

Compatibility  with  C64  software  that uses previously unimplemented

6502  opcodes  (often associated with many copy-protection schemes) or

that implements extremely timing dependent "fast loaders" is iherently

impossible. Because the VIC-III timing is slightly different, programs

that  are  extremely  timing  dependant  may  not  work properly. Also

because  the  VIC-III does not change display modes until the end of a

character  line, programs that change displays based strictly upon the

raster  position  may not display things properly. The aspect ratio of

the  VIC-III display is slightly different than the VIC-II. The use of

a  1541-II disk drive (optional) will improve compatibility. C64 BASIC

2.2  compatibility  will be 100%  (within hardware constraints).  C128

BASIC   10  compatibility  will  be  moderate  (graphic  commands  are

different,  some  command parameters different, and there are many new

commands).

1.4.1.2. Hardware

C64  hardware  compatibility  is  limited. Serial bus and control port

devices  (mouse, joysticks, etc.) are fully supported.  Some user port

devices  are  supported  such as the newer (4-DIP switch) 1670 modems,

but  there's  no  9VAC  so  devices  which require 9VAC won't function

correctly.  The expansion port has additional pins (50 total), and the

pin  spacing is closer than the C64 (it's like the PLUS/4). An adaptor

("WIDGET")  will  be necessary to utilize C64 cartridges and expansion

port devices. Furthermore, timing differences between some C64 and C65

expansion port signals will affect many C64 expansion devices (such as

the 1764).

1.4.2. DOS Compatibility

The  built-in  C65  DOS is a subset of Commodore 1581 DOS. There is no

track  cache,  index  sensor, etc. To load and run existing 1541-based

applications,  the  consumer must add a 1541 drive to the system. Many

commercial  applications cannot be easily ported from 1541/5.25" media

to 1581/3.5" media, due to copy protection or "fast loaders". Most C64

applications  that  directly  address DOS memory, specific disk tracks

or  sectors, or rely on DOS job queues and timing characteristics will

not work with the built-in drive and new DOS.

1.4.3. Operating Modes

The  C65 powers up in the C64 mode. If there are no conditions present

which  indicate that C64 mode is desired, such as the C= key depressed

or   a   C64   cartridge  signature  found,  then  C65  mode  will  be

automatically  brought  into  context. Unlike the C128, "C6 4 mode" is

escapable.  Like  the  C128,  all  of the extended features of the C65

system  are  accessible  from "C64 mode" via custom software. Whenever

the  system initiates C64 mode, new VIC mode is always disabled except

when the DOS is required.

1.5. System Maps

1.5.1. Composite System Memory Map

C64 CARTRIDGES     C64           C65          RAM-LO        RAM-HI

$FFFF+-----------+ +-----------+ +-----------+ +-----------+ +-----------+

|           | |           | |           | |           | |COLOR NYBS |

$F800|   GAME    | |  KERNEL   | |  KERNEL   | |           | +-----------+

|           | |    &      | |    &      | |           | |           |

|   CARD    | |  EDITOR   | |  EDITOR   | |           | |           |

|           | |           | |           | |.......... | | ......... |

$E000+-----------+ +-----------+ +-----------+ | C65 EVEN  | | C65 ODD   |

|COLOR NYBS | |COLOR NYBS | | BITPLANES | | BITPLANES |

|I/O & CHARS| |I/O & CHARS| |.......... | | ......... |

$D000 ------------ +-----------+ +-----------+ |           | |           |

|           | |           | |           |

|  KERNEL   | |           | |           |

|           | | C65 BASIC | | C65 VARS &|

$C000+-----------+ +-----------+ +-----------+ |   TEXT    | |   STRINGS |

|           | |           | |           | |$2000-$FEFF| |$2000-$F7FF|

|APPLICATION| |           | |           | |           | |           |

|           | |   BASIC   | |           | |           | |           |

| CARD _ HI | |           | |   BASIC   | |           | |           |

|           | |           | |  GRAPHICS | |           | |           |

$A000+-----------+ +-----------+ |           | +-----------+ |           |

|           |               |           | |           | |           |

|APPLICATION|               |    DOS    | |           | |           |

|           |               |  (MAPPED) | |           | |           |

| CARD _ LOW|               |           | |           | |           |

|           |               |           | | C64 VARS &| |           |

$8000+-----------+ ------------- +-----------+ |   STRINGS | |           |

|COLOR NYBS | | TEXT-$BFFF| |           |

|I/O & CHARS| |           | |           |

$6000 -------------------------- +-----------+ | C64 BASIC | |           |

|           | |   TEXT    | |           |

|           | |$0800-VARS | |           |

|           | |           | |           |

|           | |           | |           |

|   BASIC   | |           | |           |

|           | |           | |           |

|           | |           | |           |

|           | |           | |           |

$2000 -------------------------- +-----------+ +-----------+ +-----------+

| C65 SYSTEM| | C64 & C65 |

|TEXTSCREENS| |    DOS    |

$0000 ---------------------------------------- +-----------+ +-----------+

1.5.2. C65 System Memory Map

MAPPER BANK

-----+-----

|

|

1M    $F,FFFF  +-------------+ ----------

|             |

+-           -+

|     RAM     |     512K BLOCK APPEARING

768K  $C,0000  +-           -+     HERE IS DETERMINED BY

|  EXPANSION  |     THE RAM EXPANDER CTLR

+-           -+     (UP TO 8MB TOTAL MAP)

|             |

512K  $8,0000  +-------------+ ----------

|             |

+-  RESERVED -+     FUTURE CARTRIDGES

|             |

256K  $4,0000  +-------------+ ----------

| SYSTEM ROMS |

128K  $2,0000  +-------------+     SEE SYSTEM MEMORY

| SYSTEM ROMS |     LAYOUT, BELOW

$0,0000  +-------------+ ----------

1.5.3. C65 System Memory Layout

BANK 0          BANK 1          BANK 2          BANK 3

RAM-LO          RAM-HI          ROM-LO          ROM-HI

$FFFF  +-------------+ +-------------+ +-------------+ +-------------+

$F800  |             | |  COLOR NYBS | |     C64     | |     C65     |

|             | +-------------+ |    KERNEL   | |    KERNEL   |

$E000  |  BITPLANES  | |             | +-------------+ +-------------+

|    (EVEN)   | |             | |  C64 CHRSET | |             |

$D000  |             | |  BITPLANES  | +-------------+ |   RESERVED  |

|             | |    (ODD)    | |  INTERFACE  | |             |

$C000  +.............+ +.............+ +-------------+ +-------------+

|             | |             | |     C64     | |             |

|             | |             | |    BASIC    | |             |

$A000  | STRUCTURES  | |   STRINGS   | +-------------+ |   GRAPHICS  |

|    ???      | |             | |     C65     | |             |

|             | |             | |    CHRSET   | |             |

$8000  +.............+ +.............+ +-------------+ +-------------+

|             | |             | |             | |             |

|             | |             | |             | |             |

|             | |             | |             | |             |

|             | |             | |             | |             |

|    BASIC    | |    BASIC    | |   RESERVED  | |  C65 BASIC  |

|    TEXT     | |  VARIABLES  | |             | |             |

|             | |             | |             | |             |

|             | |             | |             | |             |

$4000  |             | |             | +-------------+ |             |

|             | |             | |             | |             |

|             | |             | |             | |             |

|             | |             | |             | |             |

$2000  +-------------+ +-------------+ |             | +-------------+

| TEXT SCREEN | |     DOS     | |     DOS     | |   MONITOR   |

+-------------+ |             | |             | |             |

|             | |   BUFFERS   | |  (MAPS TO   | |  (MAPS TO   |

| SYSTEM VARS | |    & VARS   | |   $8000)    | |   $6000)    |

|             | |             | |             | |             |

$0000  +-------------+ +-------------+ +-------------+ +-------------+

What  does  this mean?   Here is what the 64K memory map looks like in

various configurations (i.e., as seen by the processor):

C64 mode:      $E000-$FFFF     Kernel, Editor, Basic overflow area

---------      $D000-$DFFF     I/O and Color Nybbles, Character ROM

$C000-$CFFF     Application RAM

$A000-$BFFF     BASIC 2.2

$0002-$9FFF     RAMLO.  VIC screen at $0400-$07FF

BASIC program & vars from $0800-$9FFF

C65 mode:      $E000-$FFFF     Kernel, Editor ROM code

---------      $D000-$DFFF     I/O and Color Bytes (CHRROM at $29000)

$C000-$CFFF     Kernel Interface, DOS ROM overflow area

$8000-$BFFF     BASIC 10.0 Graphics & Sprite ROM code

$2000-$7FFF     BASIC 10.0 ROM code

$0002-$1FFF     RAMLO.  VIC screen at $0800-$0FFF

BASIC prgs mapped from $02000-$0FF00

BASIC vars mapped from $12000-$1F7FF

C65 DOS mode:  $E000-$FFFF     Kernel, Editor ROM code

-------------  $D000-$DFFF     I/O (CIA's mapped out), Color Bytes

$C800-$CFFF     Kernel Interface

$8000-$C3FF     DOS ROM code

$2000-$7FFF      (don't care)

$0000-$1FFF     DOS RAMHI

C65 Monitor:   $E000-$FFFF     Kernel, Editor ROM code

------------   $D000-$DFFF     I/O and Color Bytes

$C000-$CFFF     Kernel Interface

$8000-$BFFF      (don't care)

$6000-$7FFF     Monitor ROM code

$0002-$5FFF     RAMLO

It's  done  this  way  for  a  reason.  The  CPU  MAPPER restricts the

programmer  to  one offset for each 32Kbyte half of a 64Kbyte segment.

For  one chunk of ROM to MAP in another chunk with a different offset,

it  must  do  so  into  the  other  half  of  memory  from which it is

executing.  The  OS  does  this  by  never mapping the chunk of ROM at

$C000-$DFFF, which allows this chunk to contain the Interface/MAP code

and I/O (having I/O in context is usually desirable, and you can't map

I/O  anyhow).  The  Interface/MAP ROM can be turned on and off via VIC

register $30,  bit 5 (ROM @ $C000),  and therefore does not need to be

mapped itself.  Generally,  OS functions  (such as the Kernel, Editor,

and  DOS)  live in the upper 32K half of memory, and applications such

as BASIC  or  the  Monitor)  live in the lower 32K half.  For example,

when  Monitor  mode is entered,  the OS maps out BASIC and maps in the

Monitor.  Each  has ready access to the OS,  but no built-in access to

each other.  When a DOS call is made,  the OS overlays itself with the

DOS  (except  for  the  magical  $C000 code)  in the upper 32K half of

memory,  and  overlays  the application area with DOS RAM in the lower

32K half of memory.

1.5.4. C65 System I/O Memory Map

+-------------+

$DF00   |    I/O-2    |     EXTERNAL I/O SELECT

$DE00   |    I/O-1    |     EXTERNAL I/O SELECT

+-------------+

$DD00   |    CIA-2    |     SERIAL, USER PORT

$DC00   |    CIA-1    |     KEYBOARD, JOYSTICK, MOUSE CONTROL

+-------------+

$D800   |  COLOR NYB  |     COLOR MATRIX (*FROM $1F800-$1FFFF)

+-------------+

$D700   |     DMA     |     *DMA CONTROLLER

+-------------+

$D600   |    UART     |     *RS-232, FAST SERIAL, NEW KEY LINES

+-------------+

$D440   |   SID (L)   |     AUDIO CONTROLLER (LEFT)

$D400   |   SID (R)   |     AUDIO CONTROLLER (RIGHT)

+-------------+

$D300   | BLU PALETTE |

$D200   | GRN PALETTE |     *COLOR PALETTES (NYBBLES)

$D100   | RED PALETTE |

+-------------+

$D0A0   |     REC     |     *RAM EXPANSION CTRL (OPTIONAL)

+-------------+

$D080   |     FDC     |     *DISK CONTROLLER

+-------------+

$D000   |  VIC-4567   |     VIDEO CONTROLLER

+-------------+

.

.

.

+-------------+

$0000   |    4510     |     MEMORY CONTROL FOR C64 MODE

+-------------+    (this register is actually in

the VIC-4567 in the C65)

*NOTE:  VIC must be in "new" mode to address these devices

2.0. C65 System Hardware

2.1.1. Keyboard Layout

+----+   +----+----+----+----+   +----+----+----+----+   +----+----+----+----+

|RUN |   |ESC |ALT |ASC | NO |   | F1 | F3 | F5 | F7 |   | F9 | F11| F13|HELP|

|STOP|   |    |    |DIN |SCRL|   | F2 | F4 | F6 | F8 |   | F10| F12| F14|    |

+----+   +----+----+----+----+   +----+----+----+----+   +----+----+----+----+

+----+----+----+----+----+----+----+----+----+----+----+----+----+----+----+----+

| <- | !  | "  | #  | $  | %  | &  | '  | (  | )  |    |    |    |    |CLR |INST|

|    | 1  | 2  | 3  | 4  | 5  | 6  | 7  | 8  | 9  | 0  | +  | -  | ś  |HOME|DEL |

+----+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+----+

|  TAB  |    |    |    |    |    |    |    |    |    |    |    |    | ă  | RSTR |

|       | Q  | W  | E  | R  | T  | Y  | U  | I  | O  | P  | @  | *  | ^  |      |

+----+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+------+

|CTRL|SHFT|    |    |    |    |    |    |    |    |    | [  | ]  |    |  RETURN |

|    |LOCK| A  | S  | D  | F  | G  | H  | J  | K  | L  | :  | ;  | =  |         |

+----+----+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+----+----+----+

| C= | SHIFT |    |    |    |    |    |    |    | <  | >  | ?  | SHIFT|CRSR|

|    |       | Z  | X  | C  | V  | B  | N  | M  | ,  | .  | /  |      | UP |

+----+-------+-+--+----+----+----+----+----+----+----+----+-+--+-+----+----+----+

|                   SPACE                    |    |CRSR|CRSR|CRSR|

|                                            |    |LEFT|DOWN|RITE|

+--------------------------------------------+    +----+----+----+

Notes:

1/ The  cursor  keys  are special -- the shifted cursor keys appear as

separate  keys,  but  in  actuality pressing them generates a SHIFT

plus  the normal cursor code,  making them totally compatible with,

and therefore functional in, C64 mode.

2/ There are a total of 77 keys, two of which are locking keys.

3/ The NATIONAL keyboards are similar,  and their layout and operation

is identical to their C128 implementation.

2.1.2. Keyboard Matrix

+-----+-----+-----+-----+-----+-----+-----+-----+-----+            +-----+

| C0  | C1  | C2  | C3  | C4  | C5  | C6  | C7  | C8  |            | GND |

|PIN20|PIN19|PIN18|PIN17|PIN16|PIN15|PIN14|PIN13|PIN-4|            |PIN-1|

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+            +--+--+

|     |     |     |     |     |     |     |     |                  |

|     |     |     |     |     |     |     |     |                  |

V     V     V     V     V     V     V     V     V                  |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+               |

| R0  |<----+ INS |  #  |  %  |  '  |  )  |  +  | ś   |  !  | NO  |               |

|PIN12|     | DEL |  3  |  5  |  7  |  9  |     |     |  1  | SCRL|               |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+               |

| R1  |<----+ RET |  W  |  R  |  Y  |  I  |  P  |  *  | <-- | TAB |               |

|PIN11|     |     |     |     |     |     |     |     |     |     |               |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+               |

| R2  |<----+ HORZ|  A  |  D  |  G  |  J  |  L  |  ]  | CTRL| ALT |               |

|PIN10|     | CRSR|     |     |     |     |     |  ;  |     |     +----------+    |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+          |    |

| R3  |<----+ F8  |  $  |  &  |  {  |  0  |  -  | CLR |  "  | HELP|          |    |

|PIN-9|     | F7  |  4  |  6  |  8  |     |     | HOM |  2  |     |          |    |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+          |    |

| R4  |<----+ F2  |  Z  |  C  |  B  |  M  |  >  |RIGHT|SPACE| F10 |          |    |

|PIN-8|     | F1  |     |     |     |     |  .  |SHIFT| BAR | F9  |          |    |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+          |    |

| R5  |<----+ F4  |  S  |  F  |  H  |  K  |  [  |  =  | C=  | F12 |          |    |

|PIN-7|     | F3  |     |     |     |     |  :  |     |     | F11 |          |    |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+          |    |

| R6  |<----+ F6  |  E  |  T  |  U  |  O  |  @  |  ă  |  Q  | F14 |          |    |

|PIN-6|     | F5  |     |     |     |     |     |  ^  |     | F13 |          |    |

+-----+     +-----+-----+-----+-----+-----+-----+-----+-----+-----+          |    |

| R7  |<----+ VERT|LEFT |  X  |  V  |  N  |  <  |  ?  | RUN | ESC +------+   |    |

|PIN-5|     | CRSR|SHIFT|     |     |     |  ,  |  /  | STOP|     +--+   |   |    |

+-----+     +--+--+--+--+-----+-----+-----+-----+--+--+-----+-----+  |   |   |    |

|     |                             |                 |   |   |    |

|     |                             |                 |   |   |    |

|  +--+--+     /      (LOCKING)     |                 |   |   |    |

|  |SHIFT+----+  +------------------------------------+   |   |    |

|  | LOCK|                          |                     |   |    |

|  +-----+                          |                     |   |    |

|                             +-----+-----+               |   |    |

+--+--+                          |           |               |   |    |

|CRSR +------------+-------------+           +---------------+   |    |

| UP  |        K1 PIN-21         |           |                   |    |

+--+--+                          |   4066    |                   |    |

|                             |  DECODER  |                   |    |

+--+--+                          |           |                   |    |

|CRSR +------------+-------------+           +-------------------+    |

|LEFT |        K2 PIN-22         |           |                        |

+-----+                          +-----------+                        |

|

+-----+           +-----+     /                                                   |

| NMI | <---------+RESTR+----+  +-------------------------------------------------+

|PIN-3|           |     |                                                         |

+-----+           +-----+                                                         |

|

|

+-----+           +-----+     /      (LOCKING)                                    |

| R8  | <---------+CAPS +----+  +-------------------------------------------------+

|PIN-2|           |LOCK |

+-----+           +-----+

Keyboard Notes:

1/ The  64 keys under C0 through C7 occupy the same matrix position as

in the C/64,  as does the RESTORE key.  Including SHIFT-LOCK, there

are 66 such keys.

2/ The  extended  keyboard consists of the 8 keys under the C8 output.

Counting the CAPS-LOCK key, there are 9 new keys. The C/64 does not

scan these keys.

3/ The new CURSOR LEFT and CURSOR UP keys simulate a CURSOR plus RIGHT

SHIFT key combination.

4/ The  keyboard mechanism will be mechanically similar to that of the

C128.

2.2. Form Factor

EXPANSION  SERIAL  USER PORT    STEREO     RGBA    RF   COMPOSITE FAST DISK

PORT      BUS    (PARALLEL)   L    R    VIDEO   VIDEO   VIDEO     PORT

#########   ####   #########    #    #    #####    ###    #####     ####

|~         ~~~    ~~~         ~~~~ ~~~~ ~~~~     ~~~~   ~~~~     ~~~~~    ~~|

#                                                                            |

#    POWER CONNECTOR                                                         |

|                                                        +------------------+

##   POWER SWITCH                                         |                  |

|                                                        |                  |

#                                                         |                  |

#    CONTROL PORT #2                                      |                  |

#                                                         |       3.5"       |

|                        +--------------------------+    |                  |

#                         |                          |    |    DISK DRIVE    |

#    CONTROL PORT #1      |                          |    |                  |

#                         |  RAM EXPANSION (BOTTOM)  |    |                  |

|                        |                          |    |                  |

##   RESET                |                          |    |            EJECT |

|                        +--------------------------+    +------------+---+-+

|                                                                     +---+ |

|                                                                           |

+---------------------------------------------------------------------------+

NOTES:

1. Dimensions:  about 18" wide,  8" deep,  2" high.

2. Disk unit faces forward.

3.0. System Software

3.1. BASIC 10.0

C64DX BASIC 10.0

3.1.1. INTRODUCTION

This section lists BASIC 10.0 commands, statements, and functions

in alphabetical order.  It gives a complete list of the rules (syntax)

of BASIC 10.0, along with a concise description of each.

COMMAND AND STATEMENT FORMAT

The  commands  and  statements  presented  in  this  section  are

governed  by  consistent  format  conventions designed to make them as

clear as possible. In most cases, there are several actual examples to

illustrate  what the actual command looks like.  The following example

shows  some  of  the  format  conventions  that  are used in the BASIC

commands:

EXAMPLE:   DLOAD <"program name"|(file_name_var)> [,U#] [,D#]

|            |                         |     |

|            |                         |     |

keyword      argument (if any)       optional arguments

The  parts of the command or statement that the user must type in

exactly  as they appear are in capital letters.  Words that don't have

to  be  typed  exactly,  such  as  the  name  of  the program, are not

capitalized.  When  quote marks (" ") appear (usually around a program

or  file name),  the user should include them in the appropriate place

according, to the format example.

KEYWORDS,   also  called  RESERVED  WORDS,  appear  in  uppercase

letters.  THESE  KEYWORDS  MUST  BE  ENTERED  EXACTLY  AS THEY APPEAR.

However, many keywords have abbreviations that can also be used.

Keywords  are  words that are part of the BASIC language that the

computer  understands.  Keywords  are the central part of a command or

statement.  They tell the computer what kind of action to take.  These

words cannot be used as variable names.

ARGUMENTS   (also   called  parameters)  appear  in  lower  case.

Arguments  are  the  parts of a command or statement;  they complement

keywords  by  providing  specific  information  about  the  command or

statement.  For  example,  a  keyword  tells  the  computer  to load a

program,  while the argument tells the computer which specific program

to  load  and  a  second  argument  specifies  which  drive  the  disk

containing the program is in.  Arguments include filenames, variables,

line numbers, etc.

SQUARE BRACKETS [] show OPTIONAL arguments.  The user selects any

or none of the arguments listed, depending on the requirements.

ANGLE  BRACKETS <> indicates that the user MUST choose one of the

arguments listed.

VERTICAL  BAR  |  separates items in a list of arguments when the

choices are limited to those arguments listed,  and no other arguments

can  be  used.  Then  the  vertical  bar appears in a list enclosed in

SQUARE BRACKETS, the choices are limited to the items in the list, but

still have the option not to use any arguments.

ELLIPSIS  ...,  a sequence of three dots, means that an option or

argument can be repeated more than once.

QUOTATION  MARKS  "  "  enclose character strings, filenames, and

other expressions. When arguments are enclosed in quotation marks in a

format,  the  quotation  marks  must  be included in a command file or

statement.  Quotation  marks  are  not  conventions  used  to describe

formats; they are required parts of a command or statement.

PARENTHESES  ()  When  arguments are enclosed in parentheses in a

format,  they must be included in a command or statement.  Parentheses

are  not conventions used to describe formats; they are required parts

of a command or statement.

VARIABLE  refers to any valid BASIC variable name such as X,  A$,

or T%.

EXPRESSION means any valid BASIC expression,  such  as  A+B+2  or

.5*(X+3).

3.1.2. ALPHABETICAL LIST OF COMMANDS, FUNCTIONS, and OPERATORS

*           Token = AC        multiplication

+           Token = AA        addition

-           Token = AB        subtraction

/           Token = AD        division

<           Token = B3        less-than

=           Token = B2        equal

>           Token = B1        greater-than

^           Token = AE        exponentiation

(PI)        Token = FF        return value of PI

ABS         Token = B6        absolute function

AND         Token = AF        logical AND operator

APPEND      Token = FE,0E     append file

ASC         Token = C6        string to PETSCII function

ATN         Token = C1        trigonometric arctangent function

AUTO        Token = DC        auto line numbering

BACKGROUND  Token = FE,3B     background color

BACKUP      Token = F6        backup diskette

BANK        Token = FE,02     memory bank selection

BEGIN       Token = FE,18     start logical program block

BEND        Token = FE,19     end logical program block

BLOAD       Token = FE,11     binary load file from diskette

BOOT        Token = FE,1B     load & run ML, or BASIC autoboot

BORDER      Token = FE,3C     border color

BOX         Token = E1        draw graphic box

BSAVE       Token = FE,10     binary save to disk file

BUMP        Token = CE,03     sprite collision function

BVERIFY     Token = FE,28     verify memory to binary file

CATALOG     Token = FE,0C     disk directory

CHANGE      Token = FE,2C     edit program

CHAR        Token = E0        display characters on screen

CHR$        Token = C7        PETSCII to string function

CIRCLE      Token = E2        draw graphic circle

CLOSE       Token = A0        close channel or file

CLR         Token = 9C        clear BASIC variables, etc.

CMD         Token = 9D        set output channel

COLLECT     Token = F3        validate diskette (chkdsk)

COLLISION   Token = FE,17     enable BASIC event

COLOR       Token = E7        set screen colors

CONCAT      Token = FE,13     concatenate two disk files

CONT        Token = 9A        continue BASIC program execution

COPY        Token = F4        copy a disk file

COS         Token = BE        trigonometric cosine function

CUT         Token = E4        cut graphic area

DATA        Token = 83        pre-define BASIC program data

DCLEAR      Token = FE,15     mild reset of disk drive

DCLOSE      Token = FE,0F     close disk channel or file

DEC         Token = D1        decimal function

DEF         Token = 96        define user function

DELETE      Token = F7        delete BASIC lines or disk file

DIM         Token = 86        dimension BASIC array

DIR         Token = EE        disk directory

DISK        Token = FE,40     send disk special command

DLOAD       Token = F0        load BASIC program from disk

DMA         Token = FE,1F     define & execute DMA command

DMA         Token = FE,21     "

DMA         Token = FE,23     "

DMODE       Token = FE,35     set graphic draw mode

DO          Token = EB        start BASIC loop

DOPEN       Token = FE,0D     open channel to disk file

DPAT        Token = FE,36     set graphic draw pattern

DSAVE       Token = EF        save BASIC program to disk

DVERIFY     Token = FE,14     verify BASIC memory to file

ELLIPSE     Token = FE,30     draw graphic ellipse

ELSE        Token = D5        if/then/else clause

END         Token = 80        end of BASIC program

ENVELOPE    Token = FE,0A     define musical instrument

ERASE       Token = FE,2A     delete disk file

ERRS        Token = D3        BASIC error function

EXIT        Token = FD        exit BASIC loop

EXP         Token = BD        exponentiation function

FAST        Token = FE,25     set system speed to maximum

FILTER      Token = FE,03     set audio filter parameters

FIND        Token = FE,2B     hunt for string in BASIC program

FN          Token = A5        define user function

FOR         Token = 81        start BASIC for/next loop

FOREGROUND  Token = FE,39     set foreground color

FRE         Token = B8        available memory function

GCOPY       Token = FE,32     graphic copy

GENLOCK     Token = FE,38     set video sync mode

GET         Token = Al        receive a byte of input

GO          Token = CB        program branch

GOSUB       Token = 8D        program subroutine call

GOTO        Token = 89        program branch

GRAPHIC     Token = DE        set graphic mode

HEADER      Token = F1        format a diskette

HELP        Token = EA        display BASIC line causing error

HEX$        Token = D2        return hexadecimal string function

HIGHLIGHT   Token = FE,3D     set highlight color

IF          Token = 8B        if/then/else conditional

INPUT       Token = 85        receive input data from keyboard

INPUT#      Token = 84        receive input data from channel (file)

INSTR       Token = D4        locate a string within a string

INT         Token = B5        integer function

JOY         Token = CF        joystick position function

KEY         Token = F9        define or display function key

LEFT$       Token = C8        leftmost substring function

LEN         Token = C3        length of string function

LET         Token = 88        variable assignment

LINE        Token = E5        draw graphic line, input line

LIST        Token = 9B        list BASIC program

LOAD        Token = 93        load program from disk

LOCATE      Token = E6                (currently unimplemented)

LOG         Token = BC        natural log function

LOOP        Token = EC        end of do/loop

LPEN        Token = CE,04     lightpen position function

MID$        Token = CA        substring function

MONITOR     Token = FA        enter ML Monitor mode

MOUSE       Token = FE,3E     set mouse parameters

MOVSPR      Token = FE,06     set sprite position and speed

NEW         Token = A2        clear BASIC program area

NEXT        Token = 82        end of for-next loop

NOT         Token = A8        logical complement function

OFF         Token = FE,24             (subcommand)

ON          Token = 91        multiple branch or subcommand

OPEN        Token = 9F        open I/O channel

OR          Token = B0        logical or function

PAINT       Token = DF        graphic flood-fill

PALETTE     Token = FE,34     set palette color

PASTE       Token = E3        draw graphic area from cut buffer

PEEK        Token = C2        return memory byte function

PEN         Token = FE,33     set graphic pen color

PIC         Token = FE,37     graphic subcommand

PLAY        Token = FE,04     play musical notes from string

POINTER     Token = CE,0A     address of string var function

POKE        Token = 97        change memory byte

POLYGON     Token = FE,2F     draw graphic polygon

POS         Token = B9        text cursor position function

POT         Token = CE,02     return paddle position

PRINT       Token = 99        display data on text screen

PRINT#      Token = 98        send data to channel (file)

PUDEF       Token = DD        define print-using symbols

QUIT        Token = FE,1E             (currently unimplemented)

RCLR        Token = CD                (currently unimplemented)

RDOT        Token = D0                (currently unimplemented)

READ        Token = 87        read program pre-defined program data

RECORD      Token = FE,12     set relative disk file record pointer

REM         Token = 8F        BASIC program comment

RENAME      Token = F5        rename disk file

RENUMBER    Token = F8        renumber BASIC program lines

RESTORE     Token = 8C        set DATA pointer, subcommand

RESUME      Token = D6        resume BASIC program after trap

RETURN      Token = 8E        end of subroutine call

RGR         Token = CC                (currently unimplemented)

RIGHT$      Token = C9        rightmost substring function

RMOUSE      Token = FE,3F     read mouse position

RND         Token = BB        pseudo random number function

RREG        Token = FE,09     return processor registers after SYS

RSPCOLOR    Token = CE,07     return sprite color function

RSPPOS      Token = CE,05     return sprite position function

RSPRITE     Token = CE,06     return sprite parameter function

RUN         Token = 8A        run BASIC program from memory or disk

RWINDOW     Token = CE,09     return text window parameter function

SAVE        Token = 94        save BASIC program to disk

SCALE       Token = E9                (currently unimplemented)

SCNCLR      Token = E8        erase text or graphic display

SCRATCH     Token = F2        delete disk file

SCREEN      Token = FE,2E     set parameters or open graphic screen

SET         Token = FE,2D     set system parameter, subcommand

SGN         Token = B4        return sign of number function

SIN         Token = BF        trigonometric sine function

SLEEP       Token = FE,0B     pause BASIC program for time period

SLOW        Token = FE,26     set system speed to minimum

SOUND       Token = DA        perform sound effects

SPC         Token = A6        skip spaces in printed output

SPRCOLOR    Token = FE,08     set multicolor sprite colors

SPRDEF      Token = FE,1D             (currently unimplemented)

SPRITE      Token = FE,07     set sprite parameters

SPRSAV      Token = FE,16     set or copy sprite definition

SQR         Token = BA        Square root function

STEP        Token = A9        for-next step increment

STOP        Token = 90        halt BASIC program

STRS        Token = C4        string representation of number function

SYS         Token = 9E        call ML routine

TAB         Token = A3        tab position in printed output

TAN         Token = C0        trigonometric tangent function

TEMPO       Token = FE,05     set tempo (speed) of music play

THEN        Token = A7        if/then/else clause

TO          Token = A4                (subcommand)

TRAP        Token = D7        define BASIC error handler

TROFF       Token = D9        BASIC trace mode disable

TRON        Token = D8        BASIC trace mode enable

TYPE        Token = FE,27     display sequential disk file

UNTIL       Token = FC        do/loop conditional

USING       Token = FB        define print output format

USR         Token = B7        call user ML function

VAL         Token = C5        numeric value of a string function

VERIFY      Token = 95        compare memory to disk file

VIEWPORT    Token = FE,31             (currently unimplemented)

VOL         Token = DB        set audio volume

WAIT        Token = 92        pause program pending memory condition

WHILE       Token = ED        do/loop conditional

WIDTH       Token = FE,1C             (currently unimplemented)

WINDOW      Token = FE,1A     set text screen display window

XOR         Token = CE,08     logical xor function

3.1.3. BASIC 10.0 COMMAND AND FUNCTION DESCRIPTION

ABS -- Absolute value function

ABS (expression)

The  ABSolute value function returns the unsigned value of the numeric

expression.

X = ABS(1)              Result is X = 1

X = ABS(-1)             Result is X = 0

AND -- Boolean operator

expression AND expression

The  AND  operator returns a numeric value equal to the logical AND of

two  numeric  expressions,  operating  on  the  binary value of signed

16-bit  integers in the range (-32768 to 32767).  Numbers outside this

range result in an 'ILLEGAL QUANTITY' error.

X = 4 AND 12            Result is X=4

X = 8 AND 12            Result is X=8

X = 2 AND 12            Result is X=0

In  the  case  of  logical  comparisons,  the  numeric value of a true

situation  is  -1  (equivalent  to 65535 or $FFFF hex) and the numeric

value of a false situation is zero.

X = ("ABC"="ABC") AND ("DEF"="DEF") Result is X=-l (true)

X = ("ABC"="ABC") AND ("DEF"="XYZ") Result is X= 0 (false)

APPEND -- Open a disk file and prepare to append data to it

APPEND# logical_file_number, "filename" [,Ddrive] [<ON|,>Udevice]

Opens  filename for writing, and positions the file pointer at the end

of  the  file. Subsequent PRINT# statements to the logical_file_number

will  cause  data  to be appended to the end of this file. If the file

does not exist, it will be created.

APPEND#1, "filename"

APPEND#1, (file$), ON U(unit)

ASC -- PETSCII value function

ASC (string)

This   function  returns  the  PETSCII  numeric  value  of  the  first

character of a string.  The PETSCII value of an empty (null) string is

zero. This function is the opposite of the CHR$ function. Refer to the

Table of PETSCII Character Codes.

X = ASC("ABC")         Result is X=65

X = ASC("")            Result is X=0

ATN -- Arc tangent function

ATN (expression)

This  function  returns  the  angle  whose tangent is the value of the

numeric  expression,  measured  in radians. The result is in the range

of -PI/2 to PI/2 radians.

X = ATN(45)             Result is X=1.54050257

To  get  the arc tangent of an angle measured in degrees, multiply the

numeric expression by PI/180.

AUTO -- Enable or disable automatic line numbering

AUTO [increment]

Turns  on  the automatic line numbering feature which eases the job of

entering  programs  by  typing  the line numbers for the user. As each

program  line  is entered by pressing <RETURN> the next line number is

printed  on  the  screen,  with the cursor in position to begin typing

that  line.  The  increment  parameter refers to the increment between

line  numbers.  AUTO  with  no  increment  given  turns  off auto line

numbering.  AUTO  mode is also turned off automatically when a program

is RUN. This statement is executable only in direct mode.

AUTO 10   automatically numbers line in increments of ten.

AUTO 50   automatically numbers line in increments of fifty.

AUTO      turns off automatic line numbering.

BACKGROUND -- Set the background color of the display

BACKGROUND color

Sets  the  screen background color to the given color. The color given

must be in the range (0-15). See the Color Table.

BACKUP -- Backup an entire disk from one drive to another

BACKUP Dsource_drive TO Ddestination_drive [<ON|,>Udevice]

This  command  copies all the files on a diskette to another on a dual

drive  system  only.  It  cannot backup diskettes using CBM serial bus

type  drives, for example. If the destination diskette is unformatted,

BACKUP  will  automatically  format it. BACKUP copies every sector, so

any  data  already on the destination diskette will be overwritten. To

copy specific files from one drive to another, use the COPY command.

NOTE:  This  command  can only be used with a dual disk drive, such as

the  built-in  C64DX  drive and optional F016-type expansion drive. To

backup  diskettes  using  different drives, such as the built-in drive

and a 1581-type serial bus drive, use a utility program.

BACKUP D0 to D1              Copies all files from the disk in

drive 0 to the disk in drive 1.

BACKUP D0 TO D1, ON U9       Copies all files from drive 0 to

drive 1 in disk drive unit 9.

BANK -- Set the memory bank number for PEEK, POKE, SYS, WAIT, LOAD, SAVE

BANK memory_bank

[*** THIS COMMAND MIGHT CHANGE ***]

This  command  should  be  used  before  and BASIC command that has an

address  parameter.  The  address  parameters are limited to the range

(0-65535,  $0000-$FFFF hex). The BANK command tells the computer which

64K byte memory bank the location you want is in.

The  memory_bank  parameter  is number from 0-255. Refer to the System

memory  map  to  see  what is in each bank. A BANK number greater than

127  (i.e.,  has  its most significant bit set) means "use the current

system  configuration",  and  must  be used to access an I/O location.

BASIC defaults to BANK 128.

For examples, see PEEK, POKE, etc.

BEGIN/BEND -- Extend an IF clause over more than one line

BEGIN/BEND  are  used to define a block of code which is considered by

the IF statement to be one statement.

The normal usage of IF/THEN/ELSE would be along the following lines:

IF boolean THEN statement(s) : ELSE statement(s)

The  main  restriction  is  that  the  entire body of the IF/THEN/ELSE

construct  can  only  occupy  one  line.  BEGIN/BEND allows either the

'THEN' or the 'ELSE' clause to run on for more than one line.

IF boolean THEN BEGIN: statements...

statements...

statements...  BEND : ELSE BEGIN

statements...

statements...  BEND

Remember, however, that this is only a way to extend the body for more

than one line: all other 'IF/THEN' rules apply. For example:

100 IF x=1 THEN BEGIN : a=5

110 :  b=6

120 :  c=7

130 BEND : print "ah-ha!"

In the above example, "ah-ha!" would be printed ONLY if the expression

'x=1' is TRUE, because the print statement is on the same logical line

as the THEN clause.

It is bad practice to GOTO a line in the middle of a BEGIN-BEND block.

If  BEGIN or BEND is encountered outside of an active IF statement, it

is ignored.

BLOAD -- loads a binary disk file into memory

BLOAD "filename" [,Bbank] [,Paddress] [<ON|,>Udevice]

Used  to load a machine language program or other binary data (such as

display  pictures  or  sprite  data) into memory. If a load address is

not  given, the load address given in the disk file will be used. If a

bank  number  is  not given, the bank given in the last BANK statement

will be used.  If a load overflows a bank  (that is,  the load address

exceeds 65535 ($FFFF)), an 'OUT OF MEMORY' error is reported. Also see

the LOAD command.

BLOAD "sprites", P(dec("600")), B0

BOOT -- Load and execute a program

BOOT

BOOT SYS

BOOT filename [,Bbank] [,Paddress] [,Ddrive] [<ON|,>Udevice]

BOOT  without a filename given causes the computer to look for a BASIC

program  called  AUTOBOOT.C65* on the indicated diskette,  LOAD it and

RUN it (just like RUN "AUTOBOOT.C65*").

BOOT with a filename given will cause the executable binary file to be

BLOADed and executed beginning at the load address.  If a load address

is  not  given,  the  file  will  be loaded and execution begun at the

address stored on disk.

BOOT  SYS is a special command that copies the "home" sector (the very

track  and  sector) of the C64DX built-in drive into memory at address

$400 to $5FF  (one physical sector,  512 bytes)  and perform a machine

language  JSR  (Jump  SubRoutine)  to it.  It has the same function as

turning  on  your C64DX while holding down the ALT key.  It is used to

boot  an alternate operating system from either a CBM 3.5" diskette or

an  MSDOS (720K) diskette.  If used in a BASIC program,  and it fails,

the  system  can  be corrupted. BOOT SYS does *not* use the normal DOS

to access the disk.

BOOT            Loads & runs BASIC program called

AUTOBOOT.C65* on system disk.

BOOT U9         Loads & runs BASIC program called

AUTOBOOT.C65* on disk unit 9.

BOOT "ml"       Load & executes machine language

program called ML, starting at address

stored on disk.

BORDER -- Set the exterior border color of the display

BORDER color

Sets  the screen border color to given color. The color must be in the

range (0-15). See the Color Table.

BOX -- Draw a 4-sided graphical shape

BOX x0,y0, x1,y0, x0,y1, x1,y1 [,solid]

Requires  two  line  segments  to  be  specified,  the  order of which

determines  the  shape  drawn.  The  shape  is  drawn in the currently

specified  PEN  color,  on  the  currently specified SCREEN. The above

command will draw the following shape:

|0,<=0 +--------------------+ |1,<=0

|                    |

|                    |

|0,<=1 +--------------------+ |1,<=1

But if the order of the coordinates were given as:

BOX x0,y0, x1,y0, x1,y1, x0,y1

a "bowtie" shape would be drawn. See the sample program at SCREEN.

BSAVE -- Save an area of memory in binary disk file

BSAVE "[@]filename", Pstart_adr TO Pend_adr [,Bbank] [,Ddrive]

[<ON|,>Udevice]

BSAVE  copies  an  area  of  memory  into  a  binary  disk file called

"filename",  starting  at  start_adr  and  ending  at  end_adr-1 (i.e.

end_adr  must  be one more than actual last address saved).  If a bank

number  is  not given,  the bank given in the last BANK statement will

be used.  end_adr must be greater than start_adr, and area to be saved

must  be  limited  to the indicated memory bank.  You cannot save data

from  more than one bank at a time.  start_adr is saved on disk as the

load  address.  If filename already exists on the designated diskette,

memory  is NOT saved and a 'FILE EXISTS' error is reported.  Preceding

the  filename with an '@'-sign will allow you to overwrite an existing

file, but see the cautions at DSAVE.

BSAVE "sprites", P(dec("600")) TO P(dec("800")), B0

BUMP -- Sprite collision function

BUMP (type)

This   function   return   a  numeric  summary  of  sprite  collisions

accumulated since the last time the BUMP function was used.

You  can use the COLLISION command to set up a special routine in your

program to receive control whenever a sprite BUMPs into something, but

a  particular  COLLISION does not have to be enabled to use BUMP.  See

the COLLISION command.

To  evaluate  sprite  collisions,  where  a  BIT position (0-7) in the

numeric result corresponds to a sprite number (0-7):

BIT position:           7 6 5 4 3 2 1 0

| | | | | | | |

BUMP value in binary:   0 0 0 0 0 1 0 1   = 5 decimal

BUMP(1) returns a value representing sprite-to-sprite collisions.

BUMP(2) returns a value representing sprite-to-data collisions.

X = BUMP(1)     Result is X=3 if sprites 0 & 1 collided,

as shown above. (binary 101 = 5 decimal).

Note  that more than one collision can be recorded,  in which case you

should  evaluate  a  sprite's  position  using  the RSPPOS function to

figure  out  which  sprite  collided  with what. BUMP is reset to zero

after each use.

BVERIFY -- Compare a binary disk file to an area of memory

BVERIFY "filename" [,Paddress] [,Bbank] [,Ddrive] [<ON|,>Udevice]

BVERIFY  compares  a  binary disk file called "filename" to an area of

memory. In direct mode, if the areas contain the same data the message

"OK"  is displayed, and if the data differs the message 'VERIFY ERROR'

is displayed.

In  program  mode,  an  error  is  generated  if  a  mismatch is found

otherwise  the program continues normally.  The comparison starts with

the address given,  else it starts at the address stored on disk.  The

comparison ends when the last byte is read from the disk file.

If  a  bank  number  is  not  given,  the  bank given in the last BANK

statement  will  be  used.  The  ending  address  is determined by the

length of the disk file. The comparison halts on the first mismatch or

at  the  end of the file.  The area to be compared must be confined to

the indicated memory bank.

BVERIFY "sprites", P(dec("600")), B0

CATALOG -- see DIR (DIRECTORY) command

CHANGE -- Find text in a BASIC program and change it.

CHANGE :string1: TO :string2: [,line_range]

CHANGE "string1" TO "string2" [,line_range]

This is a direct (edit) mode command. CHANGE looks for all occurrences

of string1 in the program,  displays each line containing string1 with

the  target  string  highlighted,  and prompts the user for one of the

following:

Y<RETURN>       Yes, change it and look for more

N<RETURN>       No, don't change it, but look for more

*<RETURN>       Yes, change all occurrences from here on

<RETURN>        Exit command now, don't change anything

Any character can be used for the string delimiter, but there are side

effects: see comments at FIND command. If the line number range is not

given  (see  LIST  for  description  of  range parameter),  the entire

program is searched.

CHAR -- Draw a character string on a graphic screen

CHAR column, row, height, width, direction, "string" [,charsetadr]

[*** THIS IS SUBJECT TO CHANGE ***]

CHAR  displays  text  on  a  graphic  screen  at a given location. The

character   height,   width,   and  direction  are  programmable.  The

parameters are defined as:

column:         Character position:

For 320 wide screens, 0-39

For 640 wide screens, 0-79

row:            Pixel line:

For 200 line screens, 0-199

For 400 line screens, 0-399

height:                 Multiple of 8-bit character height:

1= 8 pixels high, 2= 16 pixels, etc.

width:          Multiple of 8-bit character width:

1= 8 pixels high, 2- 16 pixels, etc.

direction:      Bit mask:       B0= up

B1= right

B2= down

B3= left

The  string can consist of any printable character,  as defined by the

VIC  character  set.  Non-text characters are ignored.  If the address

of  the character set is not given,  the upper/lower ROM character set

is used ($29800).

CHAR 18,96, 1,1,2, "C65D", DEC("9000")

The  above  example will draw the characters "C65D" in the center of a

320x200  pixel  screen  using the system's uppercase/graphic character

set.

CHR$ -- Character string function

CHR$ (value)

This  function  returns  a  string of one character having the PETSCII

value  specified.  This  function is the opposite of the ASC function.

It's  often  used in PRINT strings to output data that is not visible,

such  as  control  codes  and escape sequences.  Refer to the Table of

PETSCII Character Codes.

PRINT CHR$(27)"Q";     CHR$(27) is the escape character.

This statement performs the

clear-to-end-of-line escape function.

CIRCLE -- Draw a circle on a graphic screen

CIRCLE x_center, y_center, radius [,solid]

The  CIRCLE  command will draw a circle with the given radius centered

at (x_center,y_center) on the current graphic screen.  The circle will

be filled (i.e., a disc) if SOLID is non-zero.

CIRCLE 160,100,50

The  above example will draw a circle in the center of a 320x200 pixel

screen (160,100) having a radius of 50 pixels. The aspect ratio of the

screen  may  cause it to appear as an ellipse,  however.  See also the

ELLIPSE command.

CLOSE -- Close a logical I/O channel

CLOSE logical_channel_number

This command closes the input/output channel associated with the given

logical_channel_number, established by an OPEN statement.  In the case

of  buffered  output (such as the serial bus or RS232) any data in the

device's  buffer  will  be  transmitted  before the channel is closed.

Refer to specific I/O operations for details.

The  logical_channel_number  is  required;  to close all channels on a

given device,  use the DCLOSE  command.  Note that RUN,  NEW,  and CLR

commands  will  initialize  the  logical  channel  tables but will not

actually close any channels.

CLR -- Clear program variables

CLR

This  statement initializes BASIC's variable list, setting all numeric

variables  to  zero and string variables to null.  It also initializes

the  DATA  pointer,  BASIC  runtime  stack pointer  (i.e.,  clears all

GOSUBs, DO/LOOPs, FOR/NEXT loops, etc.), and clears any user functions

(DEF  FNx).  Any  OPEN  channels  are  forgotten  (but  a CLOSE is not

performed; don't use if there are any open disk output files). A CLeaR

is automatically performed by a RUN or a NEW command.

CMD -- Set default output channel

CMD logical_channel_number [,string]

CMD  changes the default output device,  normally the screen,  to that

specified.  The  logical_channel_number  can  be any previously OPENed

write channel, such as one to a disk file, printer, or RS232.

When  redirected  via  CMD,  all output which normally would go to the

screen (such as PRINT commands,  LIST output,  DIRECTORY lists,  etc.)

is sent to another device or file.

The  redirection  is  terminated  by  CLOSE-ing  the  CMD  channel  or

executing  a PRINT# to the CMD channel.  Some output devices require a

PRINT#  to  be  performed  before  the CMD channel is closed,  such as

printers,   to   cause  the  device's  buffer  to  be  flushed  (i.e.,

displayed).

Any  system  error  will  redirect  output back to the system default,

normally the screen, but will not flush nor close the output channel.

If  the  optional  string is given, it is output immediately after the

CMD  device  is  established.  This feature is normally used to set up

printers  (eg., set printer modes via escape codes) or to identify the

output (eg., title printouts).

OPEN 4,4        OPENS device #4, which is the printer.

CMD 4           All normal output now goes to the printer.

LIST            The LISTing goes to the printer.

PRINT#4         Set output back to the screen.

CLOSE 4         Close the printer channel.

COLLECT -- Check (validate) disk, delete bad files and free lost sectors

COLLECT [Ddrive] [<ON|,>Udevice]

Refer  to the DOS 'V'alidate command.  This command will cause the DOS

to recalculate the Block Availability Map (BAM) of the diskette in the

indicated  drive,  allocating  only those sectors being used by valid,

properly  closed  files.  All  other  sectors are marked as "free" and

improper files are automatically deleted.

Note:  COLLECT should be used with extreme care,  and MUST NOT be used

on  diskettes with special boot sectors or direct access (eg., random)

files. In any case, be sure the diskette has been BACKUP-ed first.

COLLISION -- Setup subroutine to handle special events

COLLISION type [,linenumber]

[*** THIS MIGHT CHANGE ***]

COLLISION  is  used to handle "interrupt" situations in BASIC, such as

sprites  bumping into things or lightpen triggers.  When the specified

situation occurs, BASIC will finish processing the currently executing

instruction and perform an automatic GOSUB to the linenumber given.

When  the subroutine terminates (it must end with a RETURN) BASIC will

resume  processing  where  it  left off.  Interrupt handling continues

until  a  COLLISION  of  the  same  type but without any linenumber is

specified.  More  than  one  type interrupt may be enabled at the same

time,  but  only  one  interrupt  can  be handled at a time  (i.e., no

recursion and no nesting of interrupts). The type interrupt can be:

1 = Sprite to sprite collision

2 = Sprite to display data collision

3 = Light pen

Note that what caused an interrupt may continue causing interrupts for

some  time  unless  the  situation  is  altered  or  the  interrupt is

disabled.  This is especially true for BASIC, which is slow to respond

to  interrupts.  Use  the  BUMP  and  RSPPOS functions to evaluate the

results  of  sprite collisions,  and the LPEN function to evaluate the

position of a light pen.

10 COLLISION 1,90

20 SPRITE 1,1 : MOVSPR 1,100,100 : MOVSPR 1,0#5

30 SPRITE 2,1 : MOVSPR 2,100,150 : MOVSPR 2,180#5

40 DO : PRINT : LOOP

50 END

90 PRINT"BUMP! ";:RETURN

In  this  example,  sprite-to-sprite collisions are enabled (line 10),

and  two  sprites  are turned on,  positioned, and made to move (lines

20  &  30).  One  sprite  moves  up and the other moves down while the

program  does nothing other than print blank lines to the screen (line

40).  When the sprite collide, the subroutine at line 90 is called, it

prints "BUMP!", and the computer goes back to printing blank lines.

COLOR -- Enable or disable screen color (character attribute) control

COLOR <ON|OFF>

COLOR  turns  on  or  turns off the screen editor's attribute handler.

When  colors  are  turned off, whatever character attributes are being

currently  displayed (text color, underline, flash, etc.) are "stuck".

The  main  purpose  for  doing  this  is  to  speed up screen handling

(writing to the screen or scrolling the screen) about two times, since

the  screen  editor  no  longer has to manipulate the attributes. Note

that only FOREGROUND colors (and special VIC attributes) are affected.

To change screen colors, use the following commands:

FOREGROUND color#       Set Foreground color (text)

HIGHLIGHT color#        Set Highlight color (text)

BACKGROUND color#       Set VIC background color

BORDER color#           Set VIC border color

CONCAT -- Concatenate (merge) two sequential disk files

CONCAT "file1"[,Ddrive1] TO "file2"[,Ddrive2] [<ON|,>Udevice]

CONCAT  merges two SEQuential files, appending the contents of "file1"

to "file2".  Upon completion, "file2" contains the data of both files,

and  "file1" is unchanged.  Both files must exist on drives of the the

same unit, and pattern matching is not allowed.

Some  disk  drives handle CONCAT differently;  refer to the DOS manual

for specific details.

CONT -- Continue program execution

CONT

CONTinue is used to re-start a BASIC program that was halted by a STOP

or END statement,  or interrupted by the <STOP> key.  The program will

resume  at  the statement following the STOP or END instruction, or at

the  statement  after  the one that was interrupted by the <STOP> key.

CONT  is  typically used during program debugging. You can look at and

alter variables while the program is halted.

Programs  halted as a result of an untrapped error condition cannot be

CONTinued.  Programs  that  have  been  edited  in  any  way cannot be

restarted.  Any  error  condition  that  occurs  since the program was

halted  will prevent it from being restarted.  Programs that cannot be

restarted via CONT can be restarted with a GOTO,  as long as you don't

need  to  resume execution in the middle of a line of commands and you

recall where the halt occurred.

Note that the <STOP> key can interrupt some commands in mid-execution,

such as file I/O, drawing commands, etc.  In such cases,  programs may

not run correctly after a CONTinue.

COPY -- Copy disk files

COPY ["file1"][,Dd1] TO ["file2"][,Dd2] [<ON|,>Udevice]

COPYs  a  disk file to another disk file.  On single drive units,  the

filenames  must  be different.  On  dual  drive units,  copying can be

done between two drives on the same unit, and the filenames can be the

same  or different.  Pattern matching can be used.  Copying files from

one  unit  to  a  different  unit  cannot  be done: use a copy utility

program  in  such  cases.  Only legal type files can be copied; direct

access data, boot sectors, and partitions cannot be copied.

Refer to the DOS manual for your disk drive specific details.

COPY "file1" TO (F2$)         Copies "file1" to another file

whose name is in F2$ on the

same drive. Names must differ.

COPY "file1",D0 TO D1,U9      Copies "file1" from unit 9

drive-0 to unit 9 drive-1.

COPY D0 TO D1                 Copies all files from drive-0

to drive-1 on the same unit.

COPY "???.src",D0 TO "*",D1   Copies all files on drive-0

matching the pattern to a file

of the same name on drive-1.

COS -- Cosine function

COS (expression)

This  function  returns the cosine of X,  where X is an angle measured

in radians. The result is in the range -1 to 1.

X = COS (pi)                Result is X=-1

To  get  the cosine  of  an  angle  measured in degrees,  multiply the

numeric expression by pi/180.

CUT -- Cut a graphic area into a temporary structure

CUT x,y,dx,dy

[*** NOT YET IMPLEMENTED ***]

DATA -- Define program constant data to be accessed by READ command

DATA [list of constants]

DATA  statements  store  lists  of  data  that will be accessed during

program  execution  by a READ statement. The DATA statement can appear

anywhere  in  the  program,  and  it is never executed.  BASIC keeps a

pointer  to  the  earliest  un-READ  DATA  statement, and data is read

sequentially  from  first  item  in a DATA statement to the last item,

from  the  earliest  DATA  statement  in  the program to the last DATA

statement in the program.

The  list  of  constants  can  contain  both numeric data  (integer or

floating point) and string data,  but cannot contain expressions which

must be evaluated  (such as 1+2, DEC("1234"), or CHR$(13)).  Items are

separated by commas. String data need not be enclosed in quotes unless

it  contains  certain  characters,  such  as  spaces,  commas, colons,

graphic  characters,  or  control codes.  If two commands have nothing

between them, the data will be READ as 0 if numeric or a null string.

The  RESTORE  command allows you to position BASIC's data pointer to a

specific  line  number.  If  the  program tries to read more DATA than

exists  in  the  program,  an 'OUT  OF DATA' error results.  If a READ

statement's  variable  type does not agree with the DATA being read, a

'TYPE MISMATCH' error results.

DATA 100, 200, FRED, "HELLO, MOM", , 3.14, ABC123, -1.7E-9

DCLEAR -- Clear all open channels on disk drive

DCLEAR [Ddrive] [<ON|,>Udevice]

DCLEAR  sends  the indicated disk drive an 'I'nitialize command.  This

clears all open channels, closes all open files, and causes the DOS to

re-read  the  diskette's  Block Allocation Map (BAM). Note that DCLEAR

DOES  NOT  close open channels on the computer's side  (see the DCLOSE

command).  There  are  some  other side affects caused by this command

with different types of drives  --  refer the DOS manual for your disk

drive for specific details.

DCLOSE -- Close a disk file, or close all channels on a device

DCLOSE [#logical_file_number] [<ON|,>Udevice]

DCLOSE  is  intended  to  close  a file opened with the DOPEN command.

Specific  files  can be closed by specifying a logical_file_number, or

all  files  on  a  particular  drive can be closed by not specifying a

particular logical_file_number.

It is possible to close channels on non-disk devices with this command

by specifying only the device number.

DCLOSE#1        Closes the file associated with logical

logical file number 1.

DCLOSE          Closes all files currently open on the

default system drive.

DCLOSE U(U2)    Closes all channels open to device U2.

DEC -- Decimal value function

DEC (hex_string)

This  function  return  the  decimal  value of a string representing a

hexadecimal number in the range  "0000"  to  "FFFF".  The result is in

the range  0-65535.  If the string contains a non-hexadecimal digit or

is  more  than  four  (4)  characters  in length an 'ILLEGAL QUANTITY'

error is reported.

VIC = DEC("D000")       Result is VIC=53248,

the address of the VIC chip

DEF FN -- Define function

DEF FN name(numeric_variable) = numeric_expression

Define a user-written numeric function.  The DEF FNx statement must be

executed  before  the  function can be used.  Once a function has been

defined,  it can be used like any other numeric variable. The function

name   is  the  letters  FN  followed  by  any  legal  floating  point

(non-integer)  variable  name.  A  function  can  be defined only in a

program.

The  numeric_variable is a "dummy" variable. It names the variable the

numeric_expression  which  will be replaced when the function is used.

It's not required to be used in the numeric_expression,  and its value

won't be changed by the function call.

The  numeric_expression  performs the calculations of the function. It

is any legal numeric expression that fits on one line.  Variables used

in the expression have their value at the time the function is used.

Functions  can  be used only by the program which defines them. If one

program  chains  to  another  program,  the  first program's functions

cannot be used (usually a 'SYNTAX ERROR' results).  Similarly,  if the

program  is  moved  in  any  way  after  the  function is defined, the

function cannot be used.

10 DEF FN R(MAX) = INT(RND(0)*MAX)+1

20 INPUT "MAXIMUM"; MAX

30 PRINT FN R(MAX)

In  this example,  we've defined a function which will return a pseudo

random  number  between  1  and whatever MAX is.  Instead of using the

expression INT(RND(0)*MAX)+1 every time a random number is needed,  we

can now use FN R(MAX).  When we use FN R(x),  the value of 'x' will be

substituted everywhere MAX is used in the function definition.

10 DEF FN I(X) = X+1

20 DEF FN L(Z) = LEN(A$)

30 DEF FN AVG(N) = (TOT*CNT+N)/(CNT+1)

DELETE -- Delete lines of BASIC program, or

Delete disk files

DELETE [startline] [-[endline]]

DELETE "filespec" [,Ddrive] [<ON|,>Udevice] [,R]

There  are  two forms of DELETE. The first form is used in direct mode

to remove lines from a BASIC program:

DELETE 75      Deletes line 75

DELETE 10-50   Deletes line 10 through 50 inclusive.

DELETE -50     Deletes all lines from the beginning of

the program up to and including line 50.

DELETE 75-     Deletes all lines from 75 to the end of

the program.

The  second  form  is  used in program or direct mode to delete a disk

file. See the SCRATCH command.

DELETE "myfile"   Deletes the file MYFILE on the system drive.

DIM -- Declare array dimensions

DIM variable(subscripts) [,variable(subscripts)]...

Before arrays of variables can be used, the program must first execute

a  DIM  statement  to establish DIMensions of that array (unless there

are 11 or fewer elements in the array).  The statement DIM is followed

by the name of the array,  which may be any legal variable name. Then,

enclosed  in  parentheses,  put  the  number  (or numeric variable) of

elements  in each dimension.  An array with more than one dimension is

called  a  matrix.  Any number of dimensions may be used,  but keep in

mind  that the whole list of variables being created takes up space in

memory,  and it is easy to run out of memory if too many are used.  To

figure  the  number  of variables created with each DIM,  multiply the

total  number of elements in each dimension of the array.  Note:  each

array  starts with element 0, and integer arrays take up 2/5ths of the

space of floating point arrays.

More  than  one  array  can  be  dimensioned  in  a  DIM  statement by

separating  the  arrays  by  commas.  If  the  program  executes a DIM

statement for any array more than once, the message 'REDIM'D ARRAY' is

reported.  It  is  good  programming  practice to place DIM statements

near the beginning of the program.

10 DIM A$(40),B7(15),CC%(4,4,4)

|      |         |

41 elements   16 elements  125 elements

DIRECTORY -- List the files of a diskette

DIR

DIRECTORY ["filespec"] [,R] [,Ddrive] [<ON|,>Udevice]

A  directory  is  a  list  of  the  names  of  the files that are on a

diskette.  The directory listing consists of the name of the diskette,

the  names,  sizes,  and filetypes of all the files on a diskette, and

the  remaining  free  space  on  the diskette. The filespec is used to

specify  a pattern  match string to view selected files.  Not all disk

drives support the same options or filespecs; refer to your DOS manual

for details. The C64DX allows you to print DIR listings without having

to 'load' the directory; see example below.

The commands DIR, DIRECTORY, and CATALOG have the exact same function.

They can be used in direct or program mode.

DIRECTORY               List all files on the diskette

in the default system drive

DIR "*.src", U9         Lists the all the files ending with

".src" on unit 9.

DIR "*,=p",R            List all the deleted but recoverable

PRG-type files on the system drive.

OPEN 4,4:CMD 4:DIR:CLOSE 4   Print DIR listing to printer unit 4.

The following program can be used to load the directory into variables

for use within a program. In this case, the filename is simply printed

to the screen:

10 OPEN 1,8,0,"$0:*,P,R"                open dir as a file

20 : IF DS THEN PRINT DS$: GOTO 100     abort if error

30 GET#1,X$,X$                          trash load address

40 DO                                   read each line

50 : GET#1,X$,X$: IF ST THEN EXIT       trash links, check EOF

60 : GET#1,BL$,BH$                      get file size

70 : LINE INPUT#1, F$                   get filename & type

80 : PRINT LEFT$(F$,18)                 print filename

90 LOOP                                 loop until EOF

100 CLOSE 1                              close dir

DISK -- Send a disk command

DISK "command_string" [<ON|,>Udevice]

The  DISK  command is used to send special commands to the DOS via the

disk  drive's  command  channel.  The DISK command is analogous to the

following BASIC code:

OPEN 1,n,15: PRINT#1,"command_string": CLOSE 1

Not  all  disk drives understand the same commands.  Refer to your DOS

manual  for commands and command syntax for your drive.  Note that the

drive number, if any, must be included in the command_string.

DISK "U0>10"          Renumber system drive to 10.

DISR "U0>V"+chr$[0)   Turn off write verify

DISK "S0:file",U(n)   Scratch "file" on unit n

DLOAD -- Load a BASIC program file from disk

DLOAD "filename" [,Ddrive] [<ON|,>Udevice]

This  command  copies a BASIC program from disk into the BASIC program

area of the computer. It can then be edited, DSAVEd, or RUN.

Used  in  program mode,  it overlays the current program in memory and

begin  execution  automatically  at the first line of the new program.

Variable definitions will be left intact,  but any open data files and

the disk command channel will be automatically closed.  This is called

CHAINING.

See also RUN. Use BLOAD to load binary or machine language data.

DLOAD "myprogram"      Searches the default system disk drive

for the BASIC program "myprogram",

loads it, and relinks it.

DLOAD (F$),U9          LOADs a program whose name is in F$

from disk unit 9.

DMA -- Perform a DMA operation

DMA command [,length,source(l/h/b),dest(l/h/b),subcmd,mod(l/h) [,...]]

[*** THIS COMMAND IS SUBJECT TO CHANGE ***]

The DMA command defines and executes a Direct Memory Access operation.

The parameters are used to construct a DMA list,  which is then passed

to   the   DMA   processor  for  execution.  Refer  to  the  DMA  chip

specification for details.  Chained DMA commands are not allowed,  but

multiple  DMA  commands  can  be given and the DMA handler will set up

and execute each one, one at a time. Refer to the system memory map to

find out where things are.

Because  this  command  directly  accesses system memory, extreme care

should  be  taken in its use.  Changing the wrong memory locations can

crash the computer (press the reset button to reboot).

DMA 3, 2000, ASC("+"), 0, DEC("800"), 0        Fill screen with '+'

DMA 0, 2000, DEC("800"), 0, DEC("8000"), 1     Copy screen to $18000

DMODE -- Set graphic display mode

DMODE jam, comp, inverse, stencil, style, thickness

[*** THIS COMMAND IS SUBJECT TO CHANGE ***]

jam                     0-1

complement              0-1

inverse                 0-1

stencil                 0-1

style                   0-3

thickness               1-8

DO/LOOP/WHILE/UNTIL/EXIT -- Program loop definition and control

DO   [UNTIL boolean_expression | WHILE boolean_expression]

.

.         statements [EXIT]

.

LOOP [UNTIL boolean_expression | WHILE boolean_expression]

Performs  the  statements  between  the  DO  statement  and  the  LOOP

statement.  If  no  UNTIL  or WHILE modifies either the DO or the LOOP

statement,   execution   of   the   intervening  statements  continues

indefinitely.  If an EXIT statement is encountered in the body of a DO

loop,  execution  is  transferred to the first statement following the

nearest  LOOP  statement.  Do loops may be nested, following the rules

defined for FOR-NEXT loops. If the UNTIL parameter is used the program

continues  looping  until  the boolean argument is satisfied  (becomes

true).  The  WHILE  parameter  is  basically the opposite of the UNTIL

parameter:  the  program  continues  looping  as  long  as the boolean

argument is TRUE. An example of a boolean argument is A=1, or G>65.

DO UNTIL X=0 or X=1                        This loop will continue

: statements                       until X=0 or X=1. If

LOOP                                       X=0 or 1 at beginning

the loop won't execute.

10 A$="": DO GETKEY A$: LOOP UNTIL A$="Q"  This will loop until

the user types 'Q'

10 DOPEN#1,"FILE"                              This program will

20 C=0                                         count the number of

30 DO: LINEINPUT#1,A$: C=C+1: LOOP UNTIL ST    lines in FILE

40 DCLOSE#1

50 PRINT"FILE CONTAINS";C;" LINES."

DOPEN -- Open a disk file

DOPEN#lf,"filename[,<S|P>]"[,L[reclen]] [,W] [,Ddrive] [<ON|,>Udevice]

This  command  OPENs  a file on disk for reading or writing. If is the

logical  file  number,  which  you will use in PRINT#,  INPUT#,  GET#,

RECORD#,  and  DCLOSE# commands to reference the channel to your file.

The  filename is required.  The defaults are to OPEN a SEQuential file

for  Reading,  in which case the file must exist or a 'FILE NOT FOUND'

error results.  To  create  an  file and write to it,  use the 'W'rite

option.  'FILE  EXISTS'  error  is  report  if  an output file already

exists. To read or write a RELative file, use the 'L'ength option. The

'reclen' record length is required only when creating a relative file.

For  more information regarding Relative files, see the RECORD command

and refer to your DOS manual. See also APPEND.

See the OPEN command for a discussion about channel and device

numbers.

DOPEN#1,"readfile"     Opens sequential READFILE for reading.

DOPEN#1,"writefile",W  Creates & opens seq WRITEFILE for writing.

DOPEN#1,"file,P",U(u)  Opens a PRoGram type file for reading on unit U

DOPEN#1,(rf$),L        Open existing relative file whose name's in RF$

DOPEN#a,"rel",L80      Create a relative file with record length of 80

DPAT -- Set graphic draw pattern

DPAT type [, # bytes, byte1, byte2, byte3, byte4]

[*** THIS COMMAND IS SUBJECT TO CHANGE ***]

type                    0-63

# bytes                 1-4

byte1                   0-255

byte2                   0-255

byte3                   0-255

byte4                   0-255

DSAVE -- Save a BASIC program into a disk file

DSAVE "[@]filename" [,Ddrive] [<ON|,>Udevice]

This  command  copies  a  BASIC program in the computer's BASIC memory

area  into a PRoGram-type disk file.  If the file already exists,  the

program is NOT stored and the error message 'FILE EXISTS' is reported.

If  the  filename  is preceded with an '@', then if the file exists it

will  be  replaced  by the program in memory. Because of some problems

with the  'save-with-replace' option on older disk drives,  using this

option  is not recommended if you do not know what disk drive is being

used.  Use the DVERIFY to compare the program in memory with a program

on disk.

To save a binary program, use the BSAVE command.

DSAVE "myprogram"       Creates the PRG-type file MYPROGRAM

on the default system disk and copies

the BASIC program in memory into it.

DSAVE "@myprogram"      Replaces the PRG-type file MYPROGRAM

with a new version of MYPROGRAM. If

MYPROGRAM doesn't exist, it's created.

DSAVE (F$),U9           Saves a program whose name is in F$

on disk unit 9.

DVERIFY -- Compare a program in memory with one on disk

DVERIFY "filename" [,Ddrive] [<ON|,>Udevice]

This  command  is  just like a DLOAD, but instead of LOADing the BASIC

program  file  into  computer  memory  the  data is read from disk and

compared  to  computer  memory.  If  there's  any  difference at all a

'VERIFY ERROR' is reported.

Note:  If  the  BASIC  program  in  memory  is not located at the same

address  as  the  version  on  disk was SAVEd from, the files will not

match even if the program is otherwise identical.  The comparison ends

when the last byte is read from the disk file.

Use the BVERIFY command to compare memory with binary files.

DVERIFY "myprogram"

Good:   SEARCHING FOR 0:myprogram     Bad:   SEARCHING FOR 0:myprogram

VERIFYING                            VERIFYING

OK                                   ?VERIFY ERROR

ELLIPSE -- Draw an ellipse on a graphic screen

ELLIPSE x_center, y_center, x_radius, y_radius [,solid]

The ELLIPSE command will draw an ellipse with the given radii centered

at (x_center,y_center) on the current graphic screen. The ellipse will

be filled (i.e., a disc) if SOLID is non-zero.

ELLIPSE 160,100,65,50

The  above  example  will  draw  an ellipse in the center of a 320x200

pixel screen  (160,100)  having radii of  (65,50)  pixels.  The aspect

ratio of the screen may cause it to appear as an circle, however.  See

also the CIRCLE command.

ELSE -- See IF/THEN/ELSE

END -- Define the end of program execution

END

The  END  statement  terminates  program execution.  It does not close

channels  or  files,  and it does not clear any variables or reset any

pointers. An END statement does not need to be put at the last line of

a program.

The  CONTinue  command  can  be used to resume execution with the next

statement following the END statement. See also the STOP command.

ENVELOPE -- Define musical instrument envelopes

ENVELOPE n, [,[atk] [,[dec] [,[sus] [,[rel] [,[wf] [,pw] ]]]]]

n............... Envelope number        (0-9)

atk ............ Attack rate            (0-15)

dec ............ Decay rate             (0-15)

sus ............ Sustain rate           (0-15)

rel ............ Release rate           (0-15)

wf ............. Waveform:   0 = triangle

1 = sawtooth

2 = pulse (square)

3 = noise

4 = ring modulation

pw ............. Pulse width            (0-4095)

[*** THIS COMMAND IS SUBJECT TO CHANGE ***]

A parameter that is not specified will retain its current value. Pulse

width  applies  to  pulse  waves  (wf=2) only and is determined by the

formula (pwout = pw/40.95 %), so that pw = 2048 produces a square wave

and  values  of  0  or  4095  produce  constant  DC output.  The C64DX

initializes the ten (10) tune envelopes to:

n  A  D  S  R  wf  pw           instrument

------------------              -------------

ENVELOPE    0, 0, 9, 0, 0, 2, 1536           piano

ENVELOPE    1,12, 0,12, 0, 1                 accordion

ENVELOPE    2, 0, 0,15, 0, 0                 calliope

ENVELOPE    3, 0, 5, 5, 0, 3                 drum

ENVELOPE    4, 9, 4, 4, 0, 0                 flute

ENVELOPE    5, 0, 9, 2, 1, 1                 guitar

ENVELOPE    6, 0, 9, 0, 0, 2, 512            harpsichord

ENVELOPE    7, 0, 9, 9, 0, 2, 2048           organ

ENVELOPE    8, 8, 9, 4, 1, 2, 512            trumpet

ENVELOPE    9, 0, 9, 0, 0, 0                 xylophone

ERASE -- Delete disk files

ERASE "filespec" [,Ddrive] [<ON|,>Udevice] [,R]

This  command  is  identical  to  DELETE  and SCRATCH. See the SCRATCH

command for details.

ERASE "myfile" Deletes the file MYFILE on the system drive.

ERR$ -- Error message function

ERR$ (error_number)

This  function  returns  a  string  which  is  the BASIC error message

corresponding  to the given error_message.  If the given number is too

small  (less  than  1)  or  too  large  (greater  than 41) an 'ILLEGAL

QUANTITY' error is reported.

This  function is usually used to display a BASIC error condition in a

TRAP  routine,  using  the  BASIC  error  word ER as the error_number.

Note that when ER=-1, no BASIC error has occurred and ERR$(-1) results

in an 'ILLEGAL QUANTITY' error.

See the example at TRAP.

EXIT -- See DO/LOOP/WHILE/UNTIL/EXIT

EXP -- Function to return e^x

EXP (number)

This function returns the numeric value of e (2.71828183), the base of

natural  logarithms)  raised  to  the  power  of  given number. If the

number is greater than 88.0296919 an 'OVERFLOW' error is reported.

X = EXP(4)              Result is X=54.5981501

FAST -- Set system speed to 3.58MHz

FAST is the default state of the system.  FAST is used to restore this

state  following  direct  access of "slow" I/O devices such as the SID

sound chips.

FETCH -- (see the DMA command)

FILTER -- Define sound filter parameters

FILTER [freq] [,[lp] [,[bp] [,[hp] [,res] ]]]

freq ..... Filter cut-off frequency (0-2047)

lp ....... Low pass filter on (1) off (0)

bp ....... Band pass filter on (1), off(0)

hp ....... High pass filter on (1), off(0)

res ...... Resonance (0-15)

[*** THIS COMMAND IS SUBJECT TO CHANGE ***]

Unspecified  parameters  result in no change to the current value. The

filter  output  modes  are  additive.  For  example,  how low pass and

high  pass filters can be selected to produce a notch (or band reject)

filter  response.  For  the  filter to have an audible effect at least

one filter output mode must be selected and at least one voice must be

routed through the filter.

FIND -- Find text in a BASIC program.

FIND :string: [,line_range]

FIND "string" [,line_range]

This  is  a direct (edit) mode command. FIND looks for all occurrences

of  string  in  the  program and displays each line containing string,

with string highlighted.  Use the <C=> key to slow the display, or the

<NO-SCROLL> key to pause the display. Press <STOP> to cancel.

Any character can be used for the string delimiter, but there are side

effects.  Using  a  non-quote  delimiter  will  cause the string to be

tokenized,  and  FIND  will find only tokenized strings in the program

that  match.  Using  a quote character as the delimiter will cause the

string to be interpreted as plain PETSCII,  and any matches found will

therefore  be  plain  PETSCII.  Searching for some tokens such as DATA

statements  may  require  the  use  of colons as delimiters due to the

special affect these commands have upon the interpreter.

If  the  line  number range is not given  (see LIST for description of

range parameter), the entire program is searched.

FN xx -- User defined function

FN xx (expression)

The result of this numeric function is determined by the BASIC program

in a DEF FN statement. See the example at DEF FN.

FOR/TO/STEP/NEXT -- Program loop definition and control

FOR index = start TO end [STEP increment]

|

NEXT index [,index]

This command group performs a series of instructions a given number of

times.  The  loop  index  is  a  floating point (non-integer) variable

which  will  initially be set to the start value and be incremented by

the  STEP increment  when the NEXT statement is encountered.  The loop

continues until the index exceeds the end value at the NEXT statement.

The  start,  end,  and  increment  values  can be numeric variables or

expressions.  If the STEP increment is not specified, it is assumed to

be one (1).  The STEP increment can be any value,  positive, negative,

or non-integer.  If the STEP increment is negative, the loop continues

until the index is less than the end value at the NEXT statement.

Note that, regardless of the start, end, or increment values, the loop

will  always  execute at least once.  The index can be modified within

the loop, but it is bad practice to do so.  It is also bad practice to

GOTO  a  line  inside a loop structure,  or to similarly jump out of a

loop structure (which can cause an 'OUT OF MEMORY' error).

Loops may be nested.  If too many are nested, an 'OUT OF MEMORY' error

is reported (depends upon stack size, room for about 28 nested loops).

The  index  variable can be omitted from the NEXT statement,  in which

case  the NEXT will apply to the most recent FOR statement.  If a NEXT

statement is encountered and there is no preceding FOR statement, the

error 'NEXT WITHOUT FOR' is reported.

10 FOR L = 1 TO 10

20 PRINT L

30 NEXT L

40 PRINT "I'M DONE! L = "L

This  program  prints  the  numbers  from one to ten,  followed by the

message I'M DONE! L = 11.

10 FOR L = 1 TO 100

20 FOR A = 5 TO 11 STEP .5

30 NEXT A

40 NEXT L

This program illustrates a nested loop.

FOREGROUND -- Set the text color of the display

FOREGROUND color

Sets  the  text  color to the given color index.  Color must be in the

range  (0-15).  See  the Color Table.  COLOR must be ON (see the COLOR

command).

FRE -- Free byte function

FRE (x)

This  function  returns  the  number  of available ("free") bytes in a

specified area.

PRINT FRE(0)  Shows the amount of memory left in the program area,

C64DX bank 0

X = FRE(1)    X= the amount of available memory in variable area

C64DX bank 1. This causes a "garbage collect" to

occur, a process which compacts the string area.

X = FRE(2)    X= the number of expansion RAM banks present.

GCOPY -- Copy a graphic area

[*** NOT YET IMPLEMENTED ***]

GENLOCK -- Enable or disable video sync mode & colors

GENLOCK ON  [,color#]...

GENLOCK OFF [,color#,R,G,B]...

To  enable  video sync mode and specify which colors are affected, use

the  GENLOCK  ON  command,  and list the palette color indices (0-255)

which will display external video.

To  disable  video sync mode and restore the associated palette colors

use  the  GENLOCK  OFF  command,  and list the color index and its RGB

values to restore them (see the SET PALETTE command for details). Also

see the PALETTE RESTORE command.

GET -- Get input data from the keyboard

GET variable_list

The GET statement is a way to get data from the keyboard one character

at  a time.  When the GET is executed, the character that was typed is

received.  If no character was typed, then a null (empty) character is

returned,  and the program continues without waiting for a key.  There

is  no need to hit the <RETURN> key,  and in fact the <RETURN> key can

be received  with  a GET.  The word GET is followed by a variable name,

usually  a  string  variable. If a numeric were used and any key other

than  a number was hit,  the program would stop with an error message.

The  GET statement may also be put into a loop,  checking for an empty

result,  that  waits  for  a key to be struck to continue.  The GETKEY

statement could also be used in this case.  This statement can only be

executed within a program.

10 DO: GET A$: LOOP UNTIL A$ ="A"

This line waits for the A key to be pressed to continue.

GETKEY -- Get input character from keyboard (wait for key)

GETKEY variable_list

The GETKEY statement is very similar to the GET statement.  Unlike the

GET  statement,  GETKEY  waits for the user to type a character on the

keyboard.  This  lets it be used easily to wait for a single character

to be typed. This statement can only be executed within a program.

10 GETKEY A$

This  line  waits for a key to be struck. Typing any key will continue

the program.

GET# -- Get input data from a channel (file)

GET# logical_channel_number, variable_list

Used with a previously OPENed device or file to input one character at

a time. Otherwise, it works like the GET statement. This statement can

only executed within a program.

10 GET#1,A$

GO64 -- Exit C64DX mode and switch to C64 mode

GO64

This statement switches from C64DX mode to C64 mode. The question 'ARE

YOU SURE?' (in direct mode only) is posted for the user to respond to.

If Y and return is typed then the currently loaded program is lost and

control  is  given  to C64 mode.  This statement can be used in direct

mode or within a program.

GOSUB -- Call a BASIC subroutine

GOSUB line

This  statement  is  like the GOTO statement, except that the computer

remembers  from where it came.  When a line with a RETURN statement is

encountered,  the  program  jumps  back  to  the statement immediately

following  the  GOSUB.  The  target  of  a GOSUB statement is called a

subroutine.  A  subroutine  is  useful  if  there  is a section of the

program  that  can  be used by several different parts of the program.

Instead of duplicating the section over and over,  it can be set up as

a subroutine and called with a GOSUB statement from different parts of

the  program.  This  also make the main part of your program much more

readable. See also the RETURN statement.

Variables  are  shared with the main program and all subroutines.  You

can  pass information to,  and get information back from,  subroutines

by using variables as messengers.

GOSUB  statements  can  be  nested.  That is,  one subroutine can call

another  subroutine,  and  the  computer  automatically keeps track of

all  the calls.  It's important not to jump into or out of subroutines

since  this  can  confuse the computer.  If too many GOSUBs are nested

(usually  cause  by  jumping  out of them) an 'OUT OF MEMORY' error is

reported  because  the  computer  ran out of room to keep track of all

the calls.

10 DIR : GOSUB 100              show directory, check status

20 GOSUB 200                    print gap

30 LIST "PROGRAM": GOSUB 100    show listing, check status

40 GOSUB 200                    print gap

50  etc...

90 END

99 :

100 REM SUBROUTINE TO CHECK DISK STATUS

110 IF DS THEN GOSUB 200: PRINT "DISK ERROR: ";DS$

120 RETURN

199 :

200 REM SUBROUTINE TO PRINT A SPACER ON THE SCREEN

210 PRINT

220 FOR I=1 TO 39: PRINT"-";: NEXT

230 PRINT

240 RETURN

GOTO -- Transfer program execution to specified line number

GOTO  line_number

GO TO line_number

After a GOTO statement is executed,  the next line to be executed will

be the one with the line number following the word GOTO.  When used in

direct  mode,  GOTO  line  number  allows starting of execution of the

program at the given line number without clearing the variables.

10 PRINT"COMMODORE"

20 GOTO 10

The  GOTO  in  line 20 makes line 10 repeat continuously until STOP is

pressed.

GRAPHIC -- select graphic mode

GRAPHIC CLR

GRAPHIC command# [,args]

Basically this is a modified C64-type SYS command,  minus the address.

In  the  C64DX system,  this will represent the ML interface,  not the

BASIC 10.0 interface which is implemented in the development system.

[*** THIS COMMAND IS SUBJECT TO CHANGE ***]

GRAPHIC CLR  initializes  (warm-starts)  the BASIC graphic system.  It

clears any existing graphic modes, screens, etc.  and allows a program

to commence graphic operations from scratch.

HEADER -- Format a diskette

HEADER "diskname" [,Iid] [,Ddrive] [<ON|,>Udevice]

The  HEADER  command prepares a new diskette for use, sometimes called

FORMATting a diskette. There are two types of "newing" a diskette -- a

long form and a quick (or short) form. You must use the long form when

preparing a new diskette for its first use. Thereafter you can use the

quick form.

WARNING:  Formatting  a  diskette  (long  or  short)  will destroy all

existing  data  on  the  diskette!  In  direct  mode, you are asked to

confirm  what  you are doing with 'ARE YOU SURE?'.  Type 'Y' and press

return  to  proceed,  or  TYPE ANY OTHER CHARACTER AND PRESS RETURN TO

CANCEL the command. In program mode there is no confirmation prompt.

The long HEADER form requires a diskname and an ID.  The diskette will

be  completely  (re)sectored,  zeros  written to all blocks, and a new

system track (directory, BAM, etc.) will be created.

HEADER "newdisk",I01            prepares a new diskette

The short HEADER form is performed when the ID option is omitted.  The

diskette is assumed to have been previously formatted,  and only a new

system track  (directory,  BAM,  etc.)  is installed.  This is roughly

equivalent to deleteing all the files, but much quicker.

HEADER "makelikenew"            re-news an working diskette

The  diskname  is  limited  to  16 characters and the ID string to two

characters.  The  same rules apply for the diskname as for a filename.

Some  Disk  Systems  use  the  ID string to tell if you have swapped a

diskette in a drive,  so it's recommended that the ID string be unique

for each of your diskettes. Some more examples:

HEADER "QUICK"

HEADER "MYDISK", I23

HEADER "RECS", I"FB", U9

HEADER (FILES), I(ID$), U(UNIT)

HELP -- Show the BASIC line that cause the last error

HELP

The  HELP  command  is  used  after  an  error  has been reported in a

program.  When  HELP  is  typed,  the  line  where  the error occurred

listed,  with  the  portion  containing  the error highlighted.  Print

ERR$(ER)  for  the  error  message,  and  print EN or EL for the error

number and error line,  respectively.  HELP can be used in direct mode

or in program mode.  Note that,  in the case of many I/O errors, there

is no associated BASIC error. Check ST or DS$ errors in these cases.

HEX$ -- Hexadecimal value function

HEX$ (decimal_expression)

This  function  returns  a  4-character  string  that  represents  the

hexadecimal  value  of the numeric decimal expression.  The expression

must  be  in  the  range  (0-65535,  $0000-$FFFF  hex)  or an 'ILLEGAL

QUANTITY' error is reported.

PRINT HEX$(10)                The string "000A" is printed.

PRINT RIGHT$(HEX$(10),2)      The string   "0A" is printed.

HIGHLIGHT -- Set the text highlight color of the display

HIGHLIGHT color

Sets  the  highlight  color to the given color index.  The color value

must  be  in the range (0-15).  See the Color Table.  COLOR must be ON

(see the COLOR command).  The highlight color is used in HELP messages

and FIND/CHANGE strings.

IF/THEN/GOTO/ELSE -- Conditional program execution

IF expression <GOTO line | THEN then_clause> [:ELSE else_clause]

IF...THEN  lets the computer analyze a BASIC expression preceded by IF

and  take one of two possible courses of action.  If the expression is

true, the statement following THEN is executed. This expression can be

any  BASIC  statement.  If  the expression is false,  the program goes

directly to the next line, unless an ELSE clause is present.  The ELSE

clause, if present, must be in the same line as the IF-THEN part. When

an  ELSE clause is present,  it is executed when the THEN clause isn't

executed. In other words, the ELSE clause executes when the expression

is  FALSE.  See BEGIN/BEND to spread the IF statement out over several

lines.  An  ELSE statement is matched to the closest THEN statement in

the case of nested IF/THEN statements.

The expression being evaluated may be a variable or formula,  in which

case  it  is  considered true if nonzero,  and false if zero.  Usually

expressions involve relational operators =, <, >, <=, >=, <>.

50 IF X>0 THEN PRINT "X>0": ELSE PRINT "X<=0"

If  X  is  greater  than 0, the THEN clause is executed,  and the ELSE

clause  isn't.  If  X  is  less than or equal to 0, the ELSE clause is

executed and the THEN clause isn't.

INPUT -- Get input from the keyboard

[LINE] INPUT ["prompt"<,l;>] variable_list

The INPUT statement pauses the BASIC program, prints the prompt string

if present,  prints a question mark and a space, and waits for data to

be typed by the user, terminated by a return character.  If the prompt

string  ends with a comma instead of a semicolon,  a question mark and

space is not printed.

Input is gathered and assigned to variables in the variable_list.  The

type  of  variable  must  match  the  type  of  input typed or a 'TYPE

MISMATCH'  error  is  reported.  Separate data items typed by the user

must  be  separated  with commas.  String data with imbedded spaces or

commas must be surrounded with quotes.  If insufficent data to satisfy

the  variable-list  is  typed, two question marks are displayed by the

computer  to  prompt for additional data to be input.  If the computer

does  not  understand  the input (such as the user typing cursor up or

down keys)  the computer responds with the message  'REDO FROM START?'

and  waits  for acceptable data to be entered. Input is limited to 160

characters (two screen lines in 80-column mode),  which is the size of

the input buffer.

The INPUT statement can only be executed from within a program.

LINE  INPUT  allows  the  program to input a string which includes any

PETSCII  character  (including  colons, commas, imbedded spaces, etc.)

up  to  but not including a null or return character.  There should be

only  one string-type variable name in the variable_list in this case,

but  if there are more the computer prompts as usual with two question

marks for more data to assign to the additional variables.

10 INPUT "WHAT'S YOUR FIRST NAME AND AGE"; NA$,A

20 PRINT "YOUR NAME IS ";NAS;" AND YOU ARE";A;" YEARS OLD"

The above INPUT is the traditional BASIC form.

10 LINE INPUT "WHAT'S YOUR ADDRESS"; AD$

20 PRINT "YOUR ADDRESS IS: ";AD$

The  above  INPUT  allows  an  entire line of data to be assigned to a

string variable, including commas and other common punctuation marks.

10 INPUT "ENTER YOUR NAME HERE: ", NA$

The  above  INPUT  suppresses  the  traditional '? ' prompt by using a

comma instead of a semicolon after the prompt string.  To suppress the

'?' without a prompt string, make the prompt string null.

INPUT# -- Input data from an I/O channel (file)

[LINE] INPUT#logical_channel_number, variable_list

The  INPUT#  command  works  like  the INPUT command, except no prompt

string  is  allowed  and  input  is  gathered from a previously OPENed

channel or file. This command can only be used in a program.

The logical channel number is the number assigned to the device (file)

in an OPEN (or DOPEN) statement. Items in the variable list must agree

with the type of data input, or a 'FILE DATA ERROR' will resuit.

On  the  C64DX,  an End Of File (EOF) condition or bad I/O status will

terminate  input,  as  if  a return character was received.  It's good

practice  to  examine  the I/O status byte (and the DS disk status for

file I/O) after every I/O instruction to check for problems or errors.

10 DOPEN#1, "FILE"                             This program will

20 C=0                                         count the number of

30 DO: LINE INPUT#1,AS: C=C+1: LOOP UNTIL ST   lines in FILE

40 DCLOSE#1

50 PRINT"FILE CONTAINS";C;" LINES."

INSTR -- Get the location of one string inside another string

INSTR (string_1, string_2 [,starting_position])

This  function  searches  for  the  first  occurrence  of  string_2 in

string_1 and returns its location.  A value of zero (0) is returned if

no match is found,  if either string is null (empty),  or if  string_2

is longer than string_1.

If the starting position is given, the search begins at that location,

otherwise the search begins at the first character of string_1.

The strings can be literals, variables, or string expressions.

X = INSTR("123456","4")         Result is X=  0

X = INSTR("123456","X")         Result is X= -1

X = INSTR("123123","2")         Result is X= 123

X = INSTR("123123","2",3)       Result is X=-124

INT -- Greatest integer function

INT (expression)

This  function returns the greatest integer less than or equal to the

numeric expression.

JOY -- Joystick function

JOY (port)

This  function  returns  the  state  of  a  joystick controller in the

specified port.

When port=l returns position of joystick 1

When port=2 returns position of joystick 2

The value returned is encoded as follows:

Fire = 128 +                  1

8           2

7          0          3

6           4

5

A value of zero  (0) means that the joystick is not being manipulated.

A  value of  128  or more means that the fire button is being pressed.

The possible vales returned are:

0     No activity             128     fire

1     up                      129     fire + up

2     up + right              130     fire + up + right

3     right                   131     fire + right

4     right + down            132     fire + right + down

5     down                    133     fire + down

6     down + left             134     fire + down + left

7     left                    135     fire + left

8     left + up               136     fire + left + up

KEY -- Enable, disable, display, or define function keys

KEY ON

KEY OFF

KEY [key#, string]

There are 14 function keys available on the C64DX (seven unshifted and

seven  shifted).  The  user  can  assign  a string consisting of BASIC

commands, control codes, escape functions, or a combination of each to

function key. The data assigned to a key is typed out when that key is

pressed,  just  as  if  the  characters  were  typed one by one on the

keyboard.  The user can enable ("turn on") or disable ("turn off") the

function keys.  When they are disabled, pressing a function key return

that key's normal character code instead of the string assigned to it.

This includes the HELP and (shifted) RUN keys.  It is also possible to

redefine the HELP and (shifted) RUN keys,  as function keys 15 and 16,

respectively.  The  system  has  default  assignments for all function

keys.  KEY  with  no  parameters  displays  a  listing  of the current

assignments for all the function keys.

The maximum length for all the definitions together is 240 characters.

If an assignment would be too big to fit,  an 'OUT OF MEMORY' error is

reported and the assignment is not made.

KEY 2, "DIR U9"+CHR$(13)

This  causes  the  computer to display the directory from disk unit #9

when function key 2 is pressed.  This is equivalent to typing 'DIR U9'

and pressing the <RETURN> key directly.  The CHR$(13) is the character

for  <RETURN>.  Other  often  used  control  codes  are  CHR$(141) for

'shifted RETURN', CHR$(27) for 'ESCape', and CHR$(34) to incorporate a

double quote into a KEY string.

KEY 2, "DIR"+CHR$(34)+"*=P"+CHR$(34)+CHR$(13)

This  is  equivalent  to  typing DIR"*=P" and pressing <RETURN> at the

keyboard.   Note   the   way   quotes  can  be  incorporated  into  an

assignment. When function key 2 is pressed, a directory of all program

files on the default system disk will be displayed.

KEY OFF

This turns off function key strings. Pressing a function key now would

return the character codes associated with F-keys as on the VIC-20 and

C64 computers.  KEY ON would re-enable function key strings, unchanged

from  their  previous  assignments.  To  restore  the  system  default

assignments, reset the computer.

LEFT$ -- Get the leftmost characters of a string

LEFT$ (string,count)

This  function returns a string containing the leftmost 'count' number

of   characters   of  the  string  expression.  Count  is  an  numeric

expression  in the range (0-255).  If count is greater than the length

of the string,  the entire string will be returned.  If count is zero,

a null (empty) string will be returned.

A$ = LEFT$("123ABC",3)          Result is A$="123"

LEN -- Get the length of a string

LEN (string)

This function returns the number of characters in a string expression.

Nonprinting characters and blanks are counted.

A = LEN("ABC")                  Result is A=3

LET -- Assign a value to a variable

[LET] variable = expression

The  LET  command  is  optional,  since  the  equal  sign by itself is

understood  by  the computer to mean assignment.  Multiple assignments

on LET statements are not allowed.

10 LET A=1: LET B=A+1: LET C$=" THREE"

20 :   D=1:     E=D+1:     F$=" THREE"

30 PRINT A;B;C$

40 PRINT D;E;F$

Output:         1 2 THREE

1 2 THREE

LINE -- Draw a line on a graphic screen

LINE x0, y0, x1, y1

LINE  draws  a  line  on the currently defined graphic screen with the

currently  defined  draw  modes.  The  line  is  draw  from (x0,y0) to

(x1,y1).

LIST -- List a BASIC program from memory or disk

LIST [startline] [- [endline] ]

LIST "filename" [,Ddrive] [<,|ON>Udevice]

LIST  is  used to view part or all of a BASIC program in memory or all

of  a  BASIC  program  on disk  (without affecting the program that is

currently in memory).

The  display can be slowed down by holding down the <C=> key or it can

be paused  by pressing the <NO-SCROLL> key or <CONTROL><S>.  A listing

that is paused  can  be  restarted by pressing <NO-SCROLL> again or by

pressing <CONTROL><Q>. The display can be stopped by pressing <STOP>.

If the word LIST is followed by a line number, the computer shows only

that  line  number.  If  LIST is typed with two numbers separated by a

dash,  the  computer shows all lines from the first to the second line

number.  If  LIST  is  typed followed by a number and just a dash,  it

shows  all  lines from  that number to the end of the program.  And if

LIST is typed, a dash, and then a number, all lines from the beginning

of  the  program  to  that  line  number  are  LISTed.  By using these

variations, any portion of a program can be examined or easily brought

to the screen for modification.  LIST can be used in direct mode or in

a BASIC program.

LIST         Shows entire program.

LIST 100-    Shows from line 100 until the end of the program.

LIST 10      Shows only line 10.

LIST -100    Shows lines from the beginning until line 100.

LIST 10-200  Shows lines from 10 to 200, inclusive.

LOAD -- Load a program or data into memory from disk

LOAD "filename" [,device_number [,relocate_flag]]

This  command  loads  a file into the computer's memory.  The filename

must be given,  and pattern matching may be used.  In the case of dual

drive  systems,  the  drive number must be part of the filename.  If a

device number is given, the file is sought on that unit, which must be

a  disk  drive.  If  a device number is not given,  the default system

drive is used. See also DLOAD and RUN commands.

The  relocate_flag is used to LOAD binary files.  If the relocate_flag

is present and non-zero,  the file will be copied into memory starting

at the address stored on disk when the file was SAVEd.  See BLOAD.  Do

not  use  the  relocate_flag  to  load  BASIC  programs:  they will be

automatically  relocated  to  the  start of the BASIC program area and

relinked.

To  compare  a  program  in  memory  to a disk file, use the VERIFY or

DVERIFY command. To compare a binary file, use BVERIFY.

See the discussion at DLOAD regarding CHAINING programs.

LOAD "PROG"        Loads BASIC program PROG from the system drive.

LOAD FILE$,DRV     Loads a program whose name is in the variable

called F$ from the unit whose number is in DRV.

LOAD "0:PROG" 8    Loads BASIC program PROG from unit 8, drive 0.

LOAD "BIN",8,1     Loads a binary file into memory.

LOCATE -- [*** NOT YET IMPLEMENTED ***]

LOG -- Get the natural logarithm of a number

LOG (number)

This function returns the natural logarithm of a numeric expression. A

natural log is a log to the base e (2.71828183). See the EXP function.

To convert to log base 10, divide by LOG(10).

A = LOG(123)                   Result is A=4.81218436

A = LOG(123) / LOG(10)         Result is A=2.08990511

LOOP -- See DO/LOOP/WHILE/UNTIL/EXIT

LPEN -- Get the position of a lightpen

PEN (position)

This  function  returns  the  current  position  of  a lightpen on the

screen.  When  position=0,  the  X  position  is  returned,  and  when

position=1 the Y position is returned. Note that lightpen coordinates,

like sprite coordinates, are offset from the normal graphic coordinate

map.  This  means  you  have  to  calculate where the lightpen is with

respect  to the screen display.  The electronics of each lightpen also

introduces a skew which must be factored into your calculations.

The  X resolution is limited to every 2 pixels,  and will always be an

even  number  in the approximate range (60-320).  The Y position is in

the approximate range  (50-250).  If either the X or the Y position is

zero, the lightpen is off-screen.

Note  that  a  lightpen  COLLISION need not be enabled to use LPEN.  A

bright  background  color,  such  as  white,  is  usually  required to

stimulate the light pen. Lightpens only work in game port 1.

10 TRAP 40                        We're done if STOP key

15 BACKGROUND 1                   Make background color white

16 FOREGROUND 0                   Make text color black

20 COLLISION 3,100                Enable lightpen interrupt

30 DO:LOOP                        Hang here until done

40 END                            Done

100 COLLISION 3                    Got one, don't want more

110 PRINT LPEN(0),LPEN(1)          Display lightpen position

120 COLLISION 3,100                Re-enable interrupt

130 RETURN

MID$ -- Substring function

MID$ (string, position [,length])

This  function  can  appear  on  the  left  or  the  right  side of an

assignment statement:

Case 1:   string_var = MID$ (string_expression, position [,length])

This  form  returns a piece of another string.  The function returns a

string  of  the  specified  length  taken  from  the string_expression

beginning at the indicated position. The position must be in the range

(1-255),  one  (1)  being  the first character.  The length can be any

number  in  the range (0-255),  or it can be omitted.  If the position

specified   is   greater   than   the  number  of  characters  in  the

string_expression, a null (empty) string is returned. If the length is

greater  than  the number of characters from the given position to the

end of the string_expression,  or the length is omitted,  then all the

rightmost characters beginning at the position are returned.

A$ = MID$("TICTACTOE",4,3)        Result is A$="TAC"

A$ = MID$("TICTACTOE",4)          Result is A$="TACTOE"

A$ = MID$("TICTACTOE",10,1)       Result is AS="" (empty)

Case 2:   MID$ (string_var, position [,length]) = string_expression

This  form  replaces  a  portion of the string contained in string_var

with  data from another string_expression,  beginning at the specified

position  in  the string_var.  If the length is given only,  that many

characters  from the  string_expression  are taken,  otherwise all the

characters  in  the  string_expression  will replace characters in the

string_var beginning at the position specified. The there are too many

characters  to  fit in the string_var,  an 'ILLEGAL QUANTITY' error is

reported. If the length given is zero, no characters will be replaced.

A$="TICTACTOE": MID$(A$,4,3)="123456"   Result is A$="TIC123TOE"

A$="TICTACTOE": MID$(A$,4)  ="123456"   Result is A$="TIC123456"

A$="TICTACTOE": MID$(A$,5)  ="123456"   Result is 'ILLEGAL QUANTITY'

MONITOR -- Enter the built-in machine language monitor

SEE SECTION 3.2 ON THE C64DX MONITOR.

MOUSE -- Enable or disable the mouse driver

MOUSE ON [,port [,sprite [,position] ] ]

MOUSE OFF

port      = joyport 1, 2, or either (both)         (1-3)

sprite    = sprite pointer                         (0-7)

position  = initial pointer location               (x,y)

normal, relative, or angular coordinate

defaults to sprite 0, port 2

????    add min/max x/y positions

[*** THIS COMMAND IS SUBJECT TO CHANGE ***]

Mouse  ON  enables  the  built-in  mouse driver.  The user must load a

pointer  into  the proper sprite area ($600-$7FF).  The driver assumes

the  "hot  point"  is the top left corner of the sprite,  and does not

allow this point to leave the screen.

Mouse  OFF  will  turn  off  the  driver  and the currently associated

sprite.

Use the RMOUSE function to get the current pointer position and button

status. See the sample program at RMOUSE.

MOVSPR -- Position sprite or set sprite in motion

MOVSPR sprite <,x,y>

Use the  SPRITE command to turn on a sprite and MOVSPR to position it.

Sprites are numbered 0-7. The sprite's position can be specified using

one of the following coordinate types:

[+/-]x, [+/-]y = [relative] position

x#y   = angle and speed

x;y   = distance and angle

Angles  are  specified  as  0-360  degrees,  with 0 being straight up.

Speeds  are  specified as a number of pixels per frame, 0-255. Sprites

are  moved  through  each  pixel  so  that  collisions  are accurately

detected.

NEW -- Delete program in memory and clear all variables

NEW [RESTORE]

This  command  erases  the  entire  program  in  memory and clears all

variables  and  open  channels  (but  it  does NOT properly close open

disk write files -- used DCLOSE or DCLEAR beforehand). NEW also resets

the runtime stack pointer  (clears GOSUB & FOR/NEXT stacks),  the DATA

pointer, and the PRINTUSING characters.

The  BASIC program in memory is lost unless it was previously SAVEd to

disk.  If  you  have  not  entered  or loaded any BASIC programs since

typing  NEW,  the  RESTORE  option  will  recover the BASIC program in

memory.  But if the BASIC environment has been changed in any way, the

program  may not be restored correctly.  If BASIC can tell something's

wrong, it will report 'PROGRAM MANGLED'.

NEW  can  be  used  in  direct (edit) mode or in a program.  When it's

encountered in a program, the program terminates.

NEXT -- See FOR/NEXT/STEP and RESUME

NOT -- Get the complement of a number

NOT (expression)

The  NOT  function  returns  the complement of an integer in the range

(-32768 to 32767). The function operates on the binary value of signed

16-bit  integers.  An  expression  outside of this range will cause an

'ILLEGAL QUAUTITY' error.

X = NOT(5)               Result is X=-6

X = NOT(-6)              Result is X=5

NOT  is often used in logical comparisons (such as an IF statement) to

invert  the  result,  since -1 (true) is the result of NOT(0) (false),

and 0 (false) is the result of NOT(-1) (true).

X = NOT("ABC"="ABC") AND ("DEF"="DEF")      Result is X= 0 (false)

X = NOT("ABC"="ABC") AND ("DEF"="XYZ")      Result is X=-1 (true)

OFF -- Subcommand used with various BASIC commands.

ON -- Computed GOTO/GOSUB

ON expression <GOTO|GOSUB> line_number_list

This  is  a  variation  of  the  IF  <expression>  GOTO statement that

branches  to  one  of  several line numbers based upon the value of an

expression.  The  integer value of the evaluated expression determines

which line number in the line_number_list gets control.

If the expression evaluates to one,  the first line number in the list

gets control,  if it's two the second line number gets control, and so

on.  Fractional  parts  of  the value are truncated  (for example, 2.9

becomes  2).  If the value is zero or greater than the number of items

in  the  list the computer takes none of the branches and continues on

with  the  next  statement.  If  the  value  is negative,  an 'ILLEGAL

QUANTITY ERROR' is reported.

The ON/GOSUB statement must call the first line number of a subroutine

and  the subroutine must end with a RETURN statement.  After executing

the  subroutine,  control  is  returned to the statement following the

ON/GOSUB statement.

10 INPUT"ENTER A NUMBER 1-3: ",X

20 ON X GOTO 100, 200, 300

30 PRINT"TOO LOW OR TOO HIGH": RUN

100 PRINT"ONE":   RUN

200 PRINT"TWO":   RUN

300 PRINT"THREE": RUN

OPEN -- Open a channel to a device or disk file

OPEN logical_chnl_num, device_number [,secondary_adr

[,<filespec|command>]]

Before a program can access a device or a file, an I/O channel must be

opened  to  it to communicate through.  When something is opened,  you

associate a logical channel number with it, and it is with this number

that  all  other  I/O  statements access the device or file.  The OPEN

command can be used in direct (edit) mode or in a program.

The  channel number, device number, and optional secondary address are

integers   from   0-255.   Refer  to  the  device's  manual  for  more

information about what (if any) secondary addresses it uses.

channel:    0-127    return = output return character only

128-255    return = output return + linefeed

device:         0    Keyboard

1    Default system drive

whatever its number is (see SET DEF)

2    RS232

3    Screen

4-7  Serial bus

(usually reserved for printers)

8-31 Serial bus

(usually reserved for disk drives)

The filespec is the file name in the case of disk files (refer to your

DOS  manual for details).  Typically,  the filename is a string having

the the following form:

[[@|S]drive:] filename [,type] [,mode]

An  example  would  be  0:MYFILE,SEQ,READ  to open the sequential file

MYFILE  for reading on drive 0.  Disk drives usually support some kind

of  filename pattern matching.  Most disk drives support the following

file types and modes (can be abbreviated to first character):

types:          'S'equential

'P'rogram

'R'elative

'U'ser

modes:          'R'ead

'W'rite

'L'ength (for relative type files)

Some channels or devices accept a command string instead of a filename

when they are opened.  An example would be the disk command channel or

the RS232 open/setup command. Refer to the device's documentation.

OPEN 1,8,15,"I"        Open CBM disk command channel & send

it the 'I'nitialize command.

OPEN 4,4,7             Open CBM printer channel in upper/lower

case mode.

OPEN 128,2,2,CHR$(14)  Open a 9600 8N1 RS232 channel and

translate CR into CRLF on output.

See   also  DOPEN,  DCLOSE,  CLOSE,  CMD,  GET#,  INPUT#,  and  PRINT#

statements and I/O status variables ST, DS, and DS$.

OR -- Boolean operator

expression OR expression

The OR operator returns a numeric value equal to the logical OR of two

numeric  expressions,  operating  on the binary value of signed 16-bit

integers  in  the range (-32768 to 32767).  Numbers outside this range

result in an 'ILLEGAL QUANTITY' error.

X = 4 OR 8              Result is X=12

In  the  case  of  logical  comparisons,  the  numeric value of a true

situation  is  -1  (equivalent  to 65535 or $FFFF hex) and the numeric

value of a false situation is zero.

X = ("ABC"="ABC") OR ("DEF"="DEF")      Result is X=-l (true)

X = ("ABC"="ABC") OR ("DEF"="XYZ")      Result is X=-1 (true)

X = ("ABC"="XYZ") OR ("DEF"="XYZ")      Result is X= 0 (false)

PAINT -- Fill a graphics area with color

PAINT x,y, mode [,color]

x,y     coordinate to begin fill at

mode    0: fill area to edge = color

1: fill area to edge=same as color at x,y

PAINT  fills an enclosed graphic area starting at the given coordinate

with  the  color  of  the  currently  defined PEN.  The mode parameter

identifies the region to be filled.

[*** THIS COMMAND IS NOT YET IMPLEMENTED ***]

PALETTE -- Define a color

PALETTE [screen#|COLOR], color#, red, green, blue

PALETTE RESTORE

screen# 0-1

color#  0-255

red     0-15

green   0-15

blue    0-15

The  PALETTE  command  can  be  used  to  define a color for a logical

graphic  screen,  set an absolute color,  or restore the C64DX VIC-III

default colors. PALETTE can be used in direct mode or in a program.

The  VIC-III  pre-defines  the  first  16 colors to the usual C64-type

colors,  but  you  can  change  them with the PALETTE COLOR command or

restore them all with the PALETTE RESTORE command.

See the sample program after the SCREEN command.

PASTE -- Put a CUT graphic area on the screen

PASTE x,y

[*** NOT YET IMPLEMENTED ***]

PEEK -- Function returning the contents of a memory location

PEEK (address)

This  function returns the contents of a memory location.  The address

must  be  an  integer in the range of 0-65535 ($0-$FFFF) and the value

returned will be an integer in the range of 0-255 ($0-$FF).

Use  the  BANK command to specify which 64K memory bank the address is

in.  Note  that  a  BANK  number greater than 127 (i.e., a bank number

with  the  most  significant  bit set)  must be used to address an I/O

location,  such  as the VIC chip or color memory.  Refer to the system

memory map for details. PEEK uses the DMA device to access memory.

Use the POKE command to change the contents of a memory location.

BANK 0: X = PEEK (208)     Reads the keyboard buffer index. If

it's empty, X will be zero, otherwise X

will be the number of characters in it.

PEN -- Specify a pen color for drawing on graphic screen

PEN pen, color

pen     0-2

color   0-255

Before  you  can  draw anything on a graphic screen,  you have to tell

BASIC  what  color  your  PENs are.  You should first define what your

colors are using the PALETTE command,  then use PEN to associate those

colors  with  a  PEN.  Whatever  graphic  commands you use after a PEN

command will use the PEN you specified.

PEN 0,1         Put color 1 "ink" into draw pen 0

See the sample program after the SCREEN command.

PIC -- Graphic picture subcommand

PLAY -- Play a musical string

PLAY "[Vn,On,Tn,Un,Xn,elements]"

[*** WILL CHANGE TO ADD 2ND SID SUPPORT ***]

The PLAY command lets you select a  voice, octave, instrument, volume,

filter,  and  musical  notes.  All  these parameters are packed into a

string (spaces are allowed for readability).

On = Octave (n=0-6)

Tn = Tune envelope # (n=0-9)

0= piano        (defaults)

1= accordion

2= calliope

3= drum

4= flute

5= guitar

6= harpsichord

7= organ

8= trumpet

9= xylophone

Un = Volume (n=0-9)

Vn = Voice (n=1-3)

Xn = filter on (n=1), off (n=0)

Elements:

A,B,C,D,E,F,G   ... Notes, may be preceded by:

# ................. Sharp

S ................. Flat

. ................. Dotted

W ................. Whole note

H ................. Half note

Q ................. Quarter note

I ................. Eighth note

S ................. Sixteenth note

R ................. Rest

M ................. Wait for all voices playing to end

(a measure)

Once the music string starts PLAYing,  the computer will continue with

the  next  statement.  The  music will continue to play automatically.

Using  the 'M'easure command will cause the computer to wait until the

music has up to that point has been played out.

Use  the  TEMPO command to alter the tempo (speed) of PLAY.  Note that

the VOLume command can change a PLAY string's volume setting.

POINTER -- Get the address of a variable descriptor

POINTER (variable_name)

This  function  returns the address of an entry in the variable table.

If  the  value  returned is zero, the variable is currently undefined.

The  variable  table  is normally in the second RAM bank (BANK 1). See

the section on variable storage for details.

Note that,  while the location of a string descriptor will not change,

the  location  of  the  actual  string in memory changes all the time.

Also,  when  working  with an array name you must specify a particular

element,  to  which  POINTER  will  return a pointer to that element's

descriptor and not to the array descriptor.

10 A$="FRED"                     Define A$

20 DESC=POINTER (A$)             Lookup A$ in variable table

30 BANK1: PRINT PEEK(DESC)       Displays the length of A$

PORE -- Write a byte to memory location

POKE address, byte [,byte ...]

POKE  is  used  to  write  one  or  more bytes into one or more memory

locations.  The  address  must  be  an integer in the range of 0-65535

($0-SFFFF) and the value to be written must be an integer in the range

of 0-255 ($0-$FF). If more than one byte is given,  it will be written

into successive memory locations.

Use  the  BANK command to specify which 64K memory bank the address is

in.  Note  that  a  BANK number greater than 127  (i.e., a bank number

with  the  most  significant  bit  set) must be used to address an I/O

location,  such  as the VIC chip or color memory.  Refer to the system

memory map for details. Also note that, unlike previous CBM computers,

POKEs  to a ROM location will not "bleed through" into a corresponding

RAM location. POKE uses the DMA device to access memory.

Use the PEEK function to read a byte from a memory location.

Because  this  command  directly accesses system memory,  extreme care

should  be  taken  in its use.  Altering the wrong memory location can

crash the computer (press the reset button to reboot).

BANK 0: POKE 208,0                Resets location 208 ($000D0),

clearing the keyboard buffer.

BANK 128: POKE DEC("D023"),1,2,3  Sets the VIC extended background

colors to 1, 2, and 3 respectively

POLYGON -- Draw a regular n-sided figure on a graphic screen

POLYGON x,y, xradius,yradius, [solid], angle,drawsides,sides,subtend

x,y         = center of polygon

x,yradius   = radii of polygon

solid       = solid flag            (0-1)

angle       = starting angle        (0-360)

drawsides   = # of sides to draw    (3-127)

sides       = # sides of polygon    (drawsides<=sides)

subtend     = subtend flag          (0-1)

POS -- Get the column number of the cursor

POS(0)

This  function returns the current text column the cursor is in,  with

respect  to  the currently defined window (see RWINDOW).  It's usually

used  to  format  text printed to the screen.  The argument (0) is not

used  for  anything.  POS  will not work as expected if text output is

redirected to a disk file or the printer.

10 MAXCOL = RWINDOW(l)

20 FOR ADR=DEC("600") TO DEC("7FF")

30 PRINT HEX$(PEEK(ADR));" ";

40 IF POS(0) > (MAXC0L-5) THEN PRINT

50 NEXT

This  example  illustrates  one  way  to  format output to the screen,

keeping  the  last  item on a line from being split between two lines,

regardless of the window size  (as long as the window size is at least

4 characters wide). It dumps the data for the first sprite in hex.

POT -- Paddle function

POT (paddle)

This  function  returns  the  state  of  a game paddle (POTentiometer)

controller in one of the two game ports.

paddle=1 ..... Position of paddle #1 (port 1, paddle "A")

paddle=2 ..... Position of paddle #2 (port 1, paddle "B")

paddle=3 ..... Position of paddle #3 (port 2, paddle "A")

paddle=4 ..... Position of paddle #4 (port 2, paddle "B")

The value returned by POT ranges from 0 to 255. Any value greater than

255  means  that  the  fire  button  is also pressed. Paddles are read

"backwards"  from normal things like volume knobs or faucets.  A value

of  255 means the paddle has been turned counterclockwise as far as it

will  go  ("off"),  and  a value of 0 means the paddle has been turned

clockwise as far as it will go "on").

Note  that  some paddles are "noisy" and their output must be averaged

or "damped" to prevent whatever they are controlling from jittering.

10 SPRITE 1,1                      Turn on a sprite

20 DO                              Begin a loop

30 X=POT(3)                        Read paddle "A" in port 2

40 MOVSPR 1,300-(X AND 254),200    Move the sprite

50 LOOP UNTIL X>255                Loop until button pressed

60 SPRITE 1,0                      Turn off sprite

This   sample  program  turns  on  a  sprite  and  lets  you  move  it

horizontally  with a paddle. If you press the paddle's fire button, it

turns off the sprite and the program ends. The calculations in line 40

do  several things all at once -- they mask the fire button and "damp"

the  output to reduce jitter by masking the least significant bit (the

X  AND  254  part) and invert the output so that turning the paddle to

the right makes the sprite go right (subtracting result from 300).

PRINT -- Display data on text screen

PRINT [expression_list] [<,|;>]

PRINT  will  evaluate  each  item  in the expression_list and pass the

results  to  the  system screen editor to display on the screen.  If a

screen  window is defined,  the output will be confined to the window.

PRINT  can  be  used to send control codes and escape sequences to the

screen  editor  to  do  such  things as set windows, change TAB stops,

change text colors or set reverse field,  or choose cursor styles. See

the section on Editor modes for details.

PRINT can be followed by any of the following:

Numeric or string expressions      12, "HELLO", 1+1, "S"+STR$(I)

Variable names                     A, B, A$, X$

Functions                          ABS(33), HEX$(160)

Punctuation marks                  ;,

Nothing

Numeric  values  are always followed by a space.  Positive numbers are

preceded by a space, and negative numbers are preceded by a minus sign

('-').  Scientific notation is used when a number is less than 0.01 or

greater than or equal to 999999999.2 .

A  semicolon (';') or space between list items causes the next item to

be  printed  immediately  following  the previous item.  A comma (',')

causes  the next item to be printed at the next comma stop (similar to

TAB stops,  but every 10 spaces).  These rules apply to the next print

statement,  if  the  expression_list ends with either a semicolon or a

comma,  otherwise  a  return  is  printed.  Note  that  floating point

variable  names  should  not  be separated from the next variable name

with  a space,  and constants should not be preceeded or followed by a

space.

For formatted PRINT output, see the PRINT USING command.

PRINT "HELLO"                      HELLO

A$="THERE": PRINT "HELLO ";A$      HELLO THERE

A=4:B=2: PRINT A+B                 6

J=41: PRINT J;: PRINT J-1          41  40

C=A+B:D=B-A: PRINT A;B;C;D         4  2  6 -2

C=A+B:D=B-A: PRINT A,B,C,D         4         2         6        -2

A=1:B=2:AB=3: PRINT A B            3

PRINT 1 2 3, 1 2 3 +1              123       124

PRINT 0.009, 0.01                  9E-03     .01

PRINT 999999999; 999999999.2       999999999  1E+09

The  CMD  command  can be used to redirect PRINT output to a device or

file. Also see the POS, SPC, TAB functions, CHAR and PRINT USING.

PRINT# -- Send data to an I/O channel (file)

PRINT#logical_channel_number [,expression_list] [<,|;>]

This command is used to send (transmit) data to a device or file.  The

logical_channel  number  is  the  number assigned to the device (file)

in an OPEN (or DOPEN) statement.  The output is otherwise identical to

that   of  a  PRINT  statement,  including  the  comma  and  semicolon

conventions.  Note that certain screen-oriented functions, such as TAB

and SPC do not have the same effect as they do with screen I/O.

It's  good  practice  to  examine the I/O status byte (and the DS disk

status for file I/O) after every I/O instruction to check for problems

or errors.

For formatted output, use the PRINT# USING command.

10 OPEN 1,8,15                  Initialize disk drive

20 PRINT#1,"I"                  (same as DCLEAR)

30 CLOSE 1

10 DOPEN#1,"NEWFILE",W          Create a SEQ file

20 FOR I=1 TO 10

30 PRINT#1, I, STR$(I)          Write numbers 1-10 to it

40 NEXT

50 DCLOSE#1

10 OPEN 2,2,2,CHR$(12)          Open 1200 baud RS232 channel

20 PRINT#2, "ATDT,5551212"      Send modem a Hayes dial command

PRINT USING -- Output formatted data to the screen, device, or file

PRINT [#logical_channel_number,] USING format; expression_list [<,|:>]

Read  about  the  PRINT  and  PRINT#  commands  first  for information

regarding  the  syntax of the expression list and,  for device output,

establishing the logical_channel_number.

The items in the expression list must be separated by commas (',').

The  format  is  defined in a string literal or string variable and is

described   below.   See  the  PUDEF  command  for  specifing  special

formatting characters. The various formatting characters are:

CHARACTER            SYMBOL         NUMERIC   STRING

----------------     ------         -------   ------

Pound sign             #               X        X

Plus sign              +               X

Minus sign             -               X

Decimal Point          .               X

Comma                  ,               X

Dollar Sign            $               X

Four Carets            ^^^^            X

Equal Sign             =                        X

Greater Than Sign      >                        X

The  pound  sign  ('#')  reserves  room  for a single character in the

output  field.  If  the  data  item  contains more characters than the

number  of  pound signs in the format field,  the entire field will be

filled with asterisks ('*').

10 PRINT USING "####";X

For these values of X, this format displays:

A = 12.34      12

A = 567.89     568

A = 123456     ****

For  a  STRING item, the string data is truncated at the bounds of the

field.  Only  as  many characters are printed as there are pound signs

in the format item. Truncation occurs on the right.

The  plus  ('+') and minus ('-') signs can be used in either the first

or  last  position  of  a format field but not both.  The plus sign is

printed  if  the number is positive.  The minus sign is printed if the

number is negative.

If a minus sign is used and the number is positive, a blank is printed

in the character position indicated by the minus sign.

If  neither  a  plus sign nor a minus sign is used in the format field

for  a  numeric  data  item,  a minus sign is printed before the first

digit  or  dollar  symbol  if  the  number  is negative and no sign is

printed if the number is positive.  This means that one more character

is printed if the number is positive.  If there are too many digits to

fit into the field specified by the pound sign and +/- signs,  then an

overflow occurs and the field is filled with asterisks ('*').

A  decimal  point  ('.') symbol designates the position of the decimal

point in the number. There can be only one decimal point in any format

field.  If  a decimal point is not specified in the format field,  the

number  is  rounded  to  the  nearest  integer and printed without any

decimal places.

When a decimal point is specified,  the number of digits preceding the

decimal  point  (including the minus sign,  if the number is negative)

must not exceed the number of pound signs before the decimal point. If

there  are  too many digits an overflow occurs and the field is filled

with asterisks ('*').

A comma (',') allows placing of commas in numeric fields. The position

of  the comma in the format list indicates where the commas appears in

a  printed  number.  Only  commas within a number are printed.  Unused

commas  to the left of the first digit appear as the filler character.

At least one pound sign must precede the first comma in a field.

If commas are specified in a field and the number is negative,  then a

minus  sign  is  printed  as the first character even if the character

position is specified as a comma.

FIELD    EXPRESSION    RESULT   COMMENT

------   -----------   ------   -----------------------------

##.#       -.1         -0.1    Leading zero added

##.#         1         1.0     Trailing zero added

####      -100.5       -101    Rounded to no decimal places

###.        10         10.     Decimal point added

#$##         1         $1      Leading dollar sign

####      -1000        ****    Overflow because 4 digits and

minus sign don't fit in field

A dollar sign ('$') symbol shows that a dollar sign will be printed in

the  number.  If the dollar sign is to float  (always be placed before

the  number),  specify at least one pound sign before the dollar sign.

If a dollar sign is specified without a leading pound sign, the dollar

sign  is printed in the position shown in the format field.  If commas

and/or  a  plus  or  minus  sign is specified in a format field with a

dollar  sign,  the  program  prints  a comma or sign before the dollar

sign.  The four up arrows or carets symbol is used to specify that the

the number is to be printed in E format (scientific notation). A pound

sign  must  be  used  in addition to the four up arrows to specify the

field  width.  The  arrows can appear either before or after the pound

sign in the format field.  Four carats must be specified when a number

is  to  be  printed in E format.  If more than one but fewer than four

carats are specified, a syntax error results. If more than four carats

are  specified  only  the  first  four  are  used.  The fifth carat is

interpreted  as  a  no  text  symbol.  An  equal sign ('=') is used to

center a string in a field. The field width is specified by the number

of characters  (pound sign and =)  in the format field.  If the string

contains fewer characters than the field width, the string is centered

in  the field.  If the string contains more characters that can be fit

into  the  field,  then the rightmost characters are truncated and the

string  fills  the entire field.  A greater than sign ('>') is used to

right justify a string in a field.

5 X=32: Y=100.23: A$="TEST"

10 PRINT USING "$##.## ";13.25,X,Y

20 PRINT USING "###>#";"CBM",A$

When this is RUN, the following output appears on the screen:

$13.25 $32.00 $*****

CBM TEST

$*****  is printed instead of Y because Y has 5 digits,  which exceeds

the  format specification.  The second line asks for the strings to be

right justified, which they are.

PUDEF -- Redefine PRINT USING symbols

PUDEF definition_string

PUDEF  allows  redefinition  of  up  to  4  symbols in the PRINT USING

statement.  Blanks,  commas,  decimal  points, and dollar signs can be

changed  into some other character by placing the new character in the

correct position in the PUDEF definition_string.

Position 1 is the filler character.  The default is a space character.

Place another character here to be used instead of spaces. Similarly,

Position 2 is the comma character. Default is a comma.

Position 3 is the decimal point.

Position 4 is the dollar sign.

10 PUDEF "*"         PRINTs * in the place of blanks.

20 PUDEF " @"        PRINTs @ in place of commas.

QUIT -- [*** UNIMPLEMENTED ***]

RCLR -- Get the current screen color

RCLR(source)

[*** CURRENTLY UNIMPLEMENTED ***]

This function returns the color assigned to source as an number in the

range of 0-15. The color sources are:

0 = background

1 = foreground

2 = multicolor 1

3 = multicolor 2

4 = border

5 = highlight color

RDOT -- Get the current position or color of the pixel cursor

RDOT(source)

[*** CURRENTLY UNIMPLEMENTED ***]

This function returns information about the current pixel location.

0 = current X position

1 = current Y position

READ -- Read data from DATA statements

READ variable_list

READ  statements are used along with DATA statements.  READ statements

read  data  from  DATA  statements into variables,  just like an INPUT

statement reads data typed by the user. READ statements can be used in

direct or program mode, but DATA statements must be in a program.

The  variable  types  in the variable_list must match the type of DATA

being  read,  or  a  'TYPE MISMATCH'  error is reported.  If there are

insufficient data in the program's  DATA  statements  to  satisfy  all

of  the  variables  in  the READ statement,  an 'OUT OF DATA' error is

reported.

The computer maintains a pointer to the next DATA item to be read by a

READ  statement.  Initially  this  pointer  points to the beginning of

the  program.  As  each  variable  in a READ statement is filled,  the

computer  moves  the  DATA pointer to the next DATA item.  If all of a

READ  statement's variables are filled before all of the data has been

read from a DATA statement, the next READ statement will begin reading

data at the point where the previous READ stopped.

The  DATA  pointer  can  be changed by the RESTORE command.  It can be

reset  back to the beginning of the program,  or pointed to a specific

line number. See RESTORE.

10 DATA 100, 200, FRED, "HELLO, MOM", , 3.14, ABC123, -1.7E-9

20 READ X,Y

30 READ NAME$, MSG$, NULL$

40 READ PI, JUNK$, S

50 RESTORE

RECORD -- Specify a relative disk file record number

RECORD #logical_channel_number, record [,byte]

This command allows you to access any part of any record in a RELative

type disk file.  If the byte parameter is omitted,  the access pointer

is pointed at the first byte of the specified record number.

Before  you can use RECORD,  you must OPEN a file.  See OPEN and DOPEN

for instructions.  Also refer to your DOS manual for an explanation of

RELative type files.

10 INPUT"ENTER RELATIVE FILENAME: ",F$  get name of existing file

20 DOPEN#1, (F$),L: PRINT DS$           open it & display disk status

30 R=1: INPUT"ENTER RECORD NUMBER: ",R  get a record number

40 B=1: INPUT"ENTER BYTE (RETURN): ",B  get byte number, if any

50 RECORD#1, R,B                        position file pointer

60 INPUT#1, REC$                        read the record

70 PRINT REC$                           display the record

80 PRINT "CONTINUE? (Y/N)"

90 GETKEY A$: IF A$="Y" THEN 30

100 DCLOSE#1                             close the file

REM -- Place an explanatory remark or comment in a program

REM plain text message

The  REMark  command  is  just  a  way  to leave a note to whomever is

reading  a  LISTing of the program.  It might explain a section of the

program, give information about the author, etc.

REM  statements  in no way effect the operation of the program, except

to  add  length to it (and therefore slow it down a little).  No other

executable statement can follow a REMark on the same line.

10 REM  THIS PROGRAM WAS WRITTEN ON 2/14/91 BY F.BOWEN

20 REM  SAMPLE PROGRAM

30 :

40 DIR                     :REM  DISPLAY THE DISK DIRECTORY

50 LIST "SAMPLE PROGRAM"   :REM  DISPLAY THIS PROGRAM

60 END

RENAME -- Rename a disk file

RENAME "oldname" TO "newname" [,Ddrive] [<ON|,>Udevice]

The  RENAME  command changes the name of a file in the disk directory.

Pattern  matching  is  not  allowed,  and  "newname"  must  be a valid

filename  that  does  not  already  exist on the disk.  The file being

renamed does not need to be open.

RENAME "TEST" TO "FINALTEST"

RENAME (OLD$) TO (OLD$+".OLD") ON U(DEV)

RENUMBER -- Renumber the lines of a BASIC program

RENUMBER [new_starting_line [,[increment] [,old_starting_line]]]

Renumber  is used to resequence the line numbers of a BASIC program in

memory.  All  or  part  of  a program can be renumbered.  The RENUMBER

command  first  scans  the  program  to make sure all the line numbers

referenced in commands  (such as GOTO, GOSUB, TRAP, etc.) exist,  that

new line numbers are in the legal range, and that changing the program

would  not  overflow the available memory.  An 'UNRESOLVED REFERENCE',

'LINE  NUMBER  TOO  LARGE',  or  'OUT  OF MEMORY' error is reported if

there's  a  problem,  and  RENUMBER  is automatically canceled without

having changed anything.

If  the program passes all the checks,  RENUMBER changes the specified

line  numbers and updates all references to the old numbers throughout

the program and relinks the program.

The  new_starting_line  is the number of the first line in the program

after  renumbering.  It  defaults to 10.  The increment is the spacing

between line numbers  (eg., 10, 20, 30 would mean an increment of 10).

It  also  defaults to 10.  The old_starting_line is the line number in

the program where you want renumbering to begin.

RENUMBER can be used in direct (edit) mode only. Note that line number

zero (0) is a valid line number.

RENUMBER                Renumbers the entire program. After

renumbering, the first line will be 10

the second 20, etc. through the end

of the program.

RENUMBER ,1             Renumbers the entire program as above,

but in increments of one. The first

line will be 10, the second 11, etc.

RENUMBER 100, 5, 80     Starting at line 80, renumbers the

program. Line 80 becomes line 100,

and lines after that are numbered in

increments of 5, through the end of

the program.

RENUMBER ,,65           Starting at line 65, renumbers lines

in increments of 10, starting at line

10 through the rest of the program.

RESTORE -- Position READ pointer at specific DATA statement

RESTORE [line]

The computer maintains a pointer to the next DATA item to be read by a

READ  statement.  Initially  this  pointer  points to the beginning of

the program. The DATA pointer can be changed by the RESTORE command.

Using  RESTORE  without  specifying  a line number will reset the DATA

pointer  back  to  the  beginning of the program.  If a line number is

specified, the DATA pointer is pointed to that line. The line does not

have  to  contain  a  DATA  statement.  When the computer executes the

next  READ statement,  it will look for the next DATA item starting at

the line the DATA pointer is at.

See the READ command an example.

RESUME - Resume program execution after error TRAP

RESUME [line|NEXT]

Used to return to execution after TRAPping an error.  If a line number

is given, the computer performs a 'GOTO line' and resumes execution at

that  line.  RESUME NEXT  resumes execution at the statement following

the  one  that  cause  the  error.  RESUME without any parameters will

resume execution at the statement that cause the error.

If  the  computer  encounters  a  RESUME  statement  outside of a TRAP

routine  or  if  a  TRAP  was  not in effect a 'CAN'T RESUME' error is

reported. RESUME can only be used in program mode.

10 TRAP 90

20 FOR I=-5 TO 5

30 PRINT 5/I

40 NEXT

50 END

60 :

90 PRINT ERR$(ER): RESUME NEXT

RETURN -- Return from subroutine or event handler

RETURN

This statement is associated with the GOSUB (GO SUBroutine) statement.

When  a  subroutine  is  called  by  a GOSUB  statement,  the computer

remembers  where  it's  at  before  it calls the subroutine.  When the

computer  encounters  a  RETURN statement,  it returns to the place it

last encountered a GOSUB and continues with the next statement.

If there wasn't a previous GOSUB,  then a 'RETURN WITHOUT GOSUB' error

is reported.

RETURN  is  also  used  by  event  handlers,  set  up by the COLLISION

command. See COLLISION.

RGR -- Get the current graphic mode

RGR(0)

[*** CURRENTLY UNIMPLEMENTED ***]

This function returns current graphic mode. A result of zero means the

display is text, a non-zero result means it's graphic.

RIGHT$ -- Get the rightmost characters of a string

RIGHT$ (string,count)

This function returns a string containing the rightmost 'count' number

of characters of the string expression. Count is an numeric expression

in  the  range (0-255).  If  count  is  greater than the length of the

string,  the entire string will be returned.  If count is zero, a null

(empty) string will be returned.

A$ = RIGHT$("123ABC",3)         Result is A$="ABC"

RMOUSE -- Get the mouse position and button status

RMOUSE [Xposition [,Yposition [,button]]]

X,Yposition = current position of mouse pointer sprite

Button      = current status of mouse buttons

0   = no button

1   = right button

128 = left button

129 = both buttons

RMOUSE is a command which retrieves a mouse's current position and the

state  of its buttons,  and places this information into the specified

numeric  variables.  If a mouse is not installed, "-1" is returned for

all variables.  If both ports are enabled,  buttons from each port are

merged. Use the MOUSE command to turn a mouse on or off.

10 MOUSE ON, 2, 1           Turn mouse on, port 2, sprite 1

20 DO                       Begin loop

30 RMOUSE X, Y, B           Get mouse position & buttons

40 PRINTUSING"### ";X,Y,B   Show  "       "         "

50 LOOP UNTIL B=129         Loop until user presses both buttons

60 MOUSE OFF                Turn mouse off

RND -- Get a pseudo-random number

RND (type)

The  RND function returns a pseudo RaNDom number between 0 and 1.  The

random sequence returned is determined by the type parameter:

type = 0    Returns a random number based upon the system clock.

type < 0    Negative numbers "seed" the random number generator,

defining a new but reproducible random sequence.

type > 0    Positive numbers draw the next random number from the

sequence defined by the last "seed" value.

This  lets  a  programmer  use a reproducible sequence while debugging

(fixing)  a  program,  so that random errors can be easily reproduced.

Once the program has been fixed, it can be "seeded" such that a random

sequence is used every time the program is run.

10 DO

20 INPUT "SEED"; S

30 IF S=0 THEN END

40 FOR I=1 TO S

50 PRINT INT(RND(1)*6)+1, INT(RND(1)*6)+1

60 NEXT

70 LOOP

The  above  program will demonstrate the results of seeding the random

number  generator.  It  lets  you  specify a positive or negative seed

value,  and  then  prints  the  first S random pairs of that sequence.

Enter a zero to end the program.  The calculations in line 50 make the

random numbers be integers from 1 to 6, like dice.  Type in a negative

dice from that sequence.  Every time you enter "-1",  for example, you

will roll the same numbers:

first roll        2 and 6

second            6 and 1

third             1 and 1

fourth            1 and 4

fifth             5 and 5

Games  and  statistical programs should use RND(0) for true randomness

or seed the generator with a random number, such as RND(-TI).

The general form for getting random integers using RND is:

INT( RND(0) * MAX ) + 1

where MAX is the highest number you can get. This gives you numbers as

low  as  1 and as high as MAX.  For dice,  MAX is 6 (or 12 if you want

to simulate rolling two dice at once). For cards, MAX is 52.

INT( RND(0) * 16)

This  form  will return integers from zero to 15,  which is useful for

generating random colour values, for example.

RREG -- Get register data after a SYS call

RREG [a_reg] [,[x_reg] [,[y_reg] [,[z_reg] [,status] ]]]

Following  a SYS call,  the RREG command retrieves the contents of the

microprocessor's  registers  and  puts them into the specified numeric

variables. See the sample program at SYS.

RSPCOLOR -- Get multicolor sprite colors

RSPCOLOR (multicolor#)

Returns the current colors for multicolor sprites.  Color values range

from 0-15. Use RSPRITE function to get the foreground sprite color.

multicolor# = 1 gets multicolor #1

multicolor# = 2 gets multicolor #2

See SPRITE and SPRCOLOR.

RSPPOS -- Get the location and speed of a sprite

RSPPOS (sprite,parameter)

The  RSPPOS  function  returns the current X or Y position of a sprite

and  its speed,  set by the MOVSPR command.  A sprite does not have to

be  on  to use RSPPOS.  The sprite number must be in the range of 0-7,

and the parameter is:

0 to get current X position

1 to get current Y position

2 to get current speed (0-255)

RSPRITE -- Get information about a sprite

RSPRITE (sprite,parameter)

The RSPRITE function returns the current state of a sprite, set by the

SPRITE command. The sprite number must be in the range of 0-7, and the

parameter is:

0   to see if it's turned on           (1)=yes  (0)=no

1   to get sprite foreground color     (0-15)

2   to get priority over background    (1)=yes  (0)=no

3   to get X-expansion factor          (1)=yes  (0)=no

4   to get Y-expansion factor          (1)=yes  (0)=no

5   to get multicolor factor           (1)=yes  (0)=no

RUN -- execute BASIC program

RUN [line #]

RUN "filename" [,Ddrive] [<On|,>Udevice]

RUN  executes  the  BASIC  program  that  is currently in memory.  The

program has to be  LOADed  (DLOAD)  or manually typed in before it can

be executed.  If a line number is specified,  execution begins at that

line.  If a filename is specified, the program is automatically loaded

from disk into memory and executed. RUN can be used in a program.

RUN  clears  all variables and open channels (but it does NOT properly

close open disk write files -- used DCLOSE or DCLEAR beforehand).  RUN

also resets the runtime stack pointer (clears GOSUB & FOR/NEXT stacks)

the DATA pointer,  and the PRINT USING characters.  To start a program

without initializing everything, use GOTO.

RUN              Starts the program at the first line.

RUN 100          Starts the program at line 100.

RUN "TEST"       Loads the program TEST from the, default system

disk and starts the program at the first line.

RWINDOW -- Get information about the current text window

RWINDOW (parameter)

This  is a function that returns information about the current console

text display. The parameter is specified as:

0      to get the maximum line # in the current window

1      to get the maximum column # in the current window

2      to get the screen size, either 40 or 80 columns

SAVE -- Save a BASIC program in memory to disk

SAVE "[[@]drive:]filename" [,device_number]

This  command  copies  a  BASIC program in the computer's BASIC memory

area  into  a PRoGram-type disk file.  If the file already exists, the

program is NOT stored and the error message 'FILE EXISTS' is reported.

If the filename is preceded with an '@0:',  then if the file exists it

will  be  replaced by the program in memory.  Because of some problems

with  the 'save-with-replace' option on older disk drives,  using this

option  is not recommended if you do not know what disk drive is being

used  (DELETE  the  file  before  SAVEing).  Pattern  matching  is not

allowed.  In the case of dual drive systems,  the drive number must be

part of the filename.

Use  the  VERIFY  or  DVERIFY command to compare the program in memory

with  a  program  on  disk.  To  save a binary program,  use the BSAVE

command.

SAVE "myprogram"       Creates the PRG-type file MYPROGRAM

on the default system disk and copies

the BASIC program in memory into it.

SAVE "@0:myprogram"    Replaces the PRG-type file MYPROGRAM

with a new version of MYPROGRAM. If

MYPROGRAM doesn't exist, it's created.

SAVE F$,9              Saves a program whose name is in F$

on disk unit 9.

SCALE -- Set the logical dimension of the graphic screen

[*** NOT YET IMPLEMENTED ***]

SCNCLR -- Clear a text or graphic screen

SCNCLR [color]

This  command  will  clear the current text window if [color] omitted,

otherwise it will clear the current graphic  screen  using  the  given

color value. See also SCREEN CLR.

SCNCLR         Clears the text screen. If a window is defined

it clears only the window area.

SCNCLR 0       Clears the current graphic screen with color 0.

SCRATCH -- Delete files from disk directory

Recover accidentally deleted files

SCRATCH "filespec" [,Ddrive] [<ON|,>Udevice] [,R]

SCRATCH,  ERASE,  or  DELETE  are different names of the same command.

They are used to delete a file from a disk directory, or optionally to

recover  if  possible an accidentally deleted file.  The diskette must

not be 'write protected', or a 'WRITE PROTECT ON' error is reported.

WARNING:  Deleting a file will destroy all existing data in that file.

Be  extremely  careful  if  you are using pattern matching,  which can

delete any or all files. In direct mode, you are asked to confirm what

you  are  doing  with 'ARE YOU SURE?'.  Type 'Y' and press <RETURN> to

proceed,  or type any OTHER CHARACTER and press <RETURN> to cancel the

command. In program mode there is no confirmation prompt.

Upon  completion,  in  direct mode only, the computer will display the

number of files deleted.

Refer  to  your  disk manual for other details.  Different disk drives

implement   slightly  different  pattern  matching  rules  or  support

features such a specially protected files.

If the 'R'ecover option is present and the DOS supports it,  a deleted

file can be recovered if nothing else has been written to the diskette

since  the  file was accidentally deleted.  You will still be asked to

confirm  the operation,  and upon completion the computer will display

the number of files restored.

SCRATCH "oldfile"       Deletes the file OLDFILE from the disk

in the default system drive.

SCRATCH "file.*"        Deletes all files beginning with FILE.

SCRATCH (F$), U(DD)     Deletes the file whose name is in F$

from the disk in device DD.

SCRATCH "SAVEME", R     Attempt to recover the program SAVEME.

SCREEN -- Graphic command

The  SCREEN  command is used to initiate a graphic command.  It always

precedes  another  command word which identifies the graphic operation

to be performed:

SCREEN CLR      - Set graphic screen color

SCREEN CLR color#

Clears  (erases)  the  currently opened graphic screen using the given

color value. Use SCNCLR to clear a text screen. See also SCNCLR.

SCREEN DEF      - Define a graphic screen

SCREEN DEF screen#, width, height, depth

screen#      0-1

width        0=320, 1=640, 2=1280

height       0=200, 1=400

depth        1-8 bitplanes (2-256 colors)

Defines a logical screen (numbered 0 or 1), specifies its size and how

many  colors  (bitplanes)  it  has.  It  does  not allow access to the

screen and it does not display the screen.  The screen must be defined

before it is opened for viewing and/or drawing to.

SCREEN SET      - Set draw and view screens

SCREEN SET  DrawScreen#, ViewScreen#

draw screen #   0-1

view screen #   0-1

This  command specifies which logical screen is to be viewed and which

logical  screen  is  to be accessed by the various draw commands.  The

screen  must  be defined and opened first.  Both the draw and the view

screen can be,  and usually are,  the same logical screen.  For double

buffering, they are different.

SCREEN OPEN     - Open a screen for access

SCREEN OPEN  screen# [,error_variable]

screen#         0-1

error_variable  [*** NOT YET IMPLEMENTED ***]

This  command  actually sets up the screen and allocates the necessary

memory for it.  If it's the view screen it will be displayed.  If it's

the draw screen, it can now be drawn to. If there is not enough memory

for the screen,  'NO GRAPHICS AREA'  is reported and the screen is not

opened.

SCREEN CLOSE    - Close a screen

SCREEN CLOSE  screen#

screen#       0-1

This command closes a logical screen,  ending access to it by the draw

commands if it's the draw screen and restoring the text screen if it's

the view screen.  SCREEN CLOSE deallocates any memory reserved for the

screen.

SAMPLE GRAPHIC PROGRAM:

1 TRAP 170                        in case of error want text screen

10 GRAPHIC CLR                     initialize graphics

20 SCREEN DEF 1,0,0,2              define a 320x200x2 graphic screen

30 SCREEN OPEN 1                   open it

40 PALETTE 1,0,  0, 0, 0           define screen 1 color 0 = black

50 PALETTE 1,1, 15, 0, 0           define screen 1 color 1 = red

55 PALETTE 1,2,  0, 0,15           define screen 1 color 2 = blue

60 PALETTE 1,3,  0,15, 0           define screen 1 color 3 = green

70 SCREEN SET 1,1                  make it the view screen

80 SCNCLR 0                        clear screen with palette color 0

90 BORDER 0                        set border color to color 0

100 PEN 0,1                         make draw pen = color 1 (red)

110 LINE 100,100, 150,150           draw a diagonal red line

120 PEN 0,2                         make draw pen = color 2 (blue)

130 BOX 50,50, 50,80, 80,50, 80,80  draw a blue box

140 PEN 0,3                         make draw pen = color 3 (green)

150 CHAR 25,50, 1,1,2, "WORDS"      draw green text

160 SLEEP 5                         pause for 5 seconds

170 SCREEN CLOSE 1                  close graphic, get text screen

180 PALETTE RESTORE                 restore normal system colors

190 BORDER 6                        restore normal border color

200 END

SET -- Set various system parameters

The SET command is used to set a system parameter.  It always precedes

another command word which identifies the parameter to be changed:

SET DEF         - Set default system disk drive

SET DEF device

The BASIC  DOS commands default to disk unit 8.  Use SET DEF to change

which device these commands default to. This command does not renumber

a disk device, use SET DISK for that.  Commands which specify a device

will  still  access  the device they specified.  A program can be made

more "user friendly" by either not specifying a drive  (thus using the

user's  preferred  drive)  or by specifying device 1.  Device number 1

means "use the system default drive, whatever its number is."

10 DIR                 gets directory of device 8

20 DIR U1              gets directory of device 8

30 DIR U10             gets directory of device 10

40 SET DEF 10          change the default drive to unit 10

50 DIR                 gets directory of device 10

60 DIR U1              gets directory of device 10

70 DIR U8              gets directory of device 8

SET DISK        - Change a disk device number

SET DISK oldnumber TO newnumber

Use this command to renumber (change) a disk drive's unit number.  Not

all  drives  can  be renumbered -- refer to your disk drive manual for

details.  This  command  sends  to  the  disk's  command  channel  the

conventional  CBM  serial disk drive "M-W" command.  See also the DISK

command, which lets you send any command to a disk drive.

SET DISK 8 TO 10        Change unit 8's number to 10

Because  the  built-in C64DX drives always take precedence over serial

bus drives, this is one way to get the built-in drive "out of the way"

so that you can access a serial bus drive #8.

SGN -- Get the sign of a number

SGN (expression)

The SiGN function returns the sign of a numeric expression as follows:

If the expression is < 0 (negative) .... returns -1

If the expression is = 0 (zero) ........ returns  0

If the expression is > 0 (positive) .... returns  1

SIN -- Sine function

SIN (expression)

This  function returns the sine of X,  where X is an angle measured in

radians. The result is in the range -1 to 1.

X = SIN (pi/4)          Result is X=0.707106781

To get the sine of an angle measured in degrees,  multiply the numeric

expression by pi/180.

SLEEP -- Pause program execution of a specified period of time

SLEEP seconds

Temporarily suspends execution of your program for 1 to 65535 seconds.

SLOW -- Set system speed to 1.02MHz

SLOW  is  used  primarily  to directly access "slow mode only" devices

such as the SID sound chips. FAST is the default system speed.

SOUND -- Produce sound effects

SOUND v, f, d [,[dir] [,[m] [,[s] [,[w] [,p] ]]]]

v   = voice          (1-6)

f   = frequency      (0-65535)

d   = duration       (0-32767)

dir = step direction (0(up), 1(down), or 2(oscillate))   default=0

m   = min frequency  (0-65535)                           default=0

s   = sweep          (0-65535)                           default=0

w   = waveform       (0=triangle,1=saw,2=square,3=noise) default=2

p   = pulse width    (0-4095)             50% duty cycle=default=2048

The  sound  command is a fast and easy way to create sound effects and

musical  tones.  The first three parameters are required to select the

voice, frequency, and duration of the tone.  The duration is specified

in "jiffies" (60 jiffies = 1 second).

Optionally,  you  can  specify a waveform and,  for square waves,  the

pulse  width.  The SOUND command can sweep a voice through a series of

equally-spaced  frequencies.  The direction of the sweep,  minimum and

maximum  frequencies  can  be  programmed.  If time expires before the

sweep is done, the sound stops. If the minimum or maximum frequency is

reached before time expires, the sound repeats.

For programming details, refer to the SID hardware documentation.  Use

the  VOLume  command to change the volume of the sound.  Note that the

TEMPO command affects PLAY strings only, not SOUND effects.

FREQout = ( f * 0.0596 ) Hz

PWout = ( p / 40.95 ) %

Each  voice can be programmed separately and played simultaneously for

a  wide  variety  of sound effects.  Once a sound effect is initiated,

BASIC  execution  continues  with  the  next statement while the sound

plays  out,  allowing you to combine and control graphics,  animation,

and sound from a BASIC program. The examples below include information

about  how  to  generate  precise tones for exact times,  but for most

casual users trial and error are perfectly acceptable!  (Note that the

values used are for 60Hz (NTSC) systems):

Using voice 1, emit a square-wave, 440Hz tone for 1 second.  Note that

440Hz = 7382 * 0.0596 using the above formula.

SOUND 1, 7382, 60

Using  voice  2,  sweep  from  100Hz  (m=1638)  to  440Hz  (f=7382) in

increments  of  1Hz  (s=17).  The  time  required  to  do  this can be

calculated as t=(f-m)/s, so t=336 jiffies.

SOUND 2, 7382, 336, 0, 1678, 17

Using  voice  3,  make a neat sound using an oscillating sweep (dir=2)

and a sawtooth waveform (w=1) for 3 seconds (t=180).

SOUND 3, 5000, 180, 2, 3000, 500, 1

SPC -- Space PRINT output

SPC (number)

The  SPaCe  function  is  used to format PRINTed data to the screen, a

printer,  or a file.  It specifies the number of spaces to be skipped,

from 0 to 255. A semicolon (';') is always assumed to follow SPC, even

if it appears at the end of a print line.

The  SPC  function  works a little differently on screen,  printer and

disk output.  On the screen,  SPC skips over characters already on the

screen,  which  is  not  the  case  with  printer and disk output.  On

printers,  if  the  last  character on a line is skipped,  the printer

will automatically perform a carriage return and linefeed.

PRINT "123";SPC(3);"456"       Displays '123   456'

PRINT "X";SPC(5) :PRINT"X"     Displays 'X     X'

See also the TAB function. A better way to format PRINT output is with

PRINT USING.

SPRCOLOR -- Set multicolor sprite colors

SPRCOLOR [sprite_mc1] [,sprite_mc2]

Use the SPRITE command to set up a multicolor sprite, and use SPRCOLOR

to set the additional colors. Note that these colors are common to all

multicolor sprites.  The color values must be in the range (0-15). Use

the RSPCOL0R function to get the current multicolor sprite colors, and

RSPRITE to get the current sprite foreground color.

SPRDEF -- Define a sprite pattern

[*** NOT EXPECTED TO BE IMPLEMENTED ***]

SPRITE -- Turn a sprite on or off, and set its characteristics

SPRITE number [,[on] [,[color] [,[priority] [,[x_exp] [,[y_exp] [,mode] ]]]]]

The  SPRITE  command  allows  you  set all of the characteristics of a

sprite.  Use  the  MOVSPR  command to position it or set it in motion.

Use the SPRCOLOR to set the multicolor sprite colors, if you are using

multicolor sprites.

All the parameters except the sprite number are optional. If you don't

specify a parameter then it won't be changed.

number  = sprite number                 (0-7)

on      = enable (1) or disable(0)

color   = sprite foreground color       (0-15)

priority= sprite to display data priority:

0 means sprite goes over screen data

1 means sprite goes under screen data

x,y_exp = sprite expansion on (1) or off (0)

mode    = sprite mode:

0 high resolution

1 multicolor

The  SPRITE  command does not define a sprite.  The sprite definitions

must  be loaded into the sprite area first ($600-$7FF).  Use the BLOAD

and  BSAVE commands.  [*** THIS MAY CHANGE ***]  A sprite is 24 pixels

wide and 21 pixels high.  Each sprite definition requires 63 ($40 hex)

bytes:

$600    Sprite 0 definition

$640   Sprite 1 definition

$680    Sprite 2 definition

$6C0    Sprite 3 definition

$700    Sprite 4 definition

$740    Sprite 5 definition

$780    Sprite 6 definition

$7C0    Sprite 7 definition

Use  the  RSPRITE function to read a sprite's characteristics,  or the

RSPPOS  function  to  read a sprite's position.  The RSPCOLOR function

is used to get the current multicolor sprite colors.

10 BLOAD"sprite 1 data",           Load sprite 1's definition

P(DEC("640"))

20 SPRITE 1, 1, 2                  Turn it on, make it red

30 MOVSPR 1, 24, 50                Put it at top-leftmost corner

40 SPRSAV 1, 2                     Copy sprite 1 definition to 2

50 SPRITE 2, 1, 7                  Turn on sprite 2 make it yellow

60 MOVSPR 2, 320, 229              Put it at bottom-rightmost corner

70 BSAVE"sprite 2 data",           Save sprite 2

P(DEC("680")) TO P(DEC("6C0"))

80 SPRITE 1, 0                     Turn off sprite 1

90 SPRITE 2, 0                     Turn off sprite 2

SPRSAV -- Copy a sprite definition

SPRSAV source, destination

Use  this command to copy a sprite's data (shape) to another sprite or

into a string variable,  or copy a shape from a string variable into a

sprite.  You  can  have  many different sprite shapes in memory at one

time,  all  stored  in  strings.  This  makes  it  possible to animate

sprites  from  BASIC by quickly "flipping through" shapes,  using each

shape like a frame from a movie film.

SPRSAV 0, A$     copy the data (shape) of sprite 0 into A$

SPRSAV A$, 2     copy the data (shape) in A$ into sprite 2

SPRSAV 1, 2      copy the data (shape) in sprite 1 to sprite 2

STASH -- (see the DMA command)

SQR -- Square root function

SQR (number)

This function returns the SQuare Root of the given numeric expression.

The  numeric  expression must not be negative or an 'ILLEGAL QUANTITY'

error is reported.

A = SQR(10)     Result is A = 3.16227766

STEP -- See FOR/NEXT/STEP

STOP -- Halt program execution

When  STOP  is  executed,  the  computer immediately stops running the

program  and reports 'BREAK IN LINE xx'.  No variables are cleared and

files are not closed.

This command is usually used while debugging (fixing) a BASIC program,

since it lets you stop at a specific place,  examine variables, change

variables,  and  restart the program where it was halted (see CONTinue

command)  or some other line (see GOTO).  In many cases,  you can even

change  the  program  and  use GOTO to resume execution with variables

and open channels intact.

SWAP -- (see the DMA command)

STR$ -- Get the string representation of a number

STR$ (number)

The  STRing  function  returns a string identical to PRINT's output of

the  given  numeric  expression.  See  PRINT for details regarding the

format of numeric output. STR$ is the opposite of VAL.

A$ = STR$(123)         Result is A$ = " 123"

A$ = STR$(-123)        Result is A$ = "-123"

A$ = STR$(.009)        Result is A$ = " 9E-03"

SYS -- Call a ROM routine or user machine language routine

SYS address [,[a] [,[x] [,[y] [,[z] [,s] ]]]]

This  statement  performs  a call to a machine language routine at the

specified  address  (range 0-65535, $3000-$FFFF)  in a memory bank set

up previously by the BANK command.

The microprocessor's registers are loaded with the values specified in

the  parameters  following  the  address  (if  given)  and a JSR (Jump

SubRoutine)  instruction  is  performed.  When the called routine ends

with an RTS  (ReTurn from Subroutine),  the microprocessor's registers

are   saved  and  control  is  returned  to  the  BASIC  program.  The

microprocessor's registers can be examined with the RREG command.

Because  this command instructs the computer's microprocessor (CPU) to

perform  something,  extreme  care should be taken in its use.  It can

easily  crash the computer if you do something wrong  (press the reset

button to reboot). Also see the BOOT SYS command.

BANK 128: SYS DEC("FF5C")       Call the Kernel's PHOENIX routine.

BANK 128: SYS DEC("FF81")       Reset the Screen Editor

10 BANK 128

20 BLOAD"user routine",P(DEC("1800"))   Load a user routine

30 SYS DEC("1800"), areg, xreg          Call it with args in A and X

40 RREG areg, xreg, , , sreg            Get args back in A, X, and S

50 carry = (sreg AND 1)                 Get carry flag from S

60 PRINT "ACCUMULATOR = ";HEX$(areg)    Display registers

70 PRINT "X REGISTER  = ";HEX$(xreg)

80 PRINT "CARRY FLAG  = ";carry

See  the  USR  function  for  another  way  to  call  machine language

routines.

TAB -- Space PRINT output

TAB (number)

The  TAB  function  is  used  to  format PRINTed data to the screen, a

printer,  or  a  file.  It's  primarily for screen text output, moving

the  cursor  to the specified column (plus one) as long as the current

print position is not already beyond that point  (for example,  if the

current  print  position  is  the  first  column,  TAB(1)  would print

subsequent text beginning in column 2).  If the current print position

is already beyond the column specified by the TAB function, nothing is

done.  For  disk  and  printer output,  TAB works exactly like the SPC

function (see SPC).

A semicolon (';') is always assumed to follow TAB,  even if it appears

at the end of a print line.

PRINT "TEXT";TAB(10);"HERE"        Result is 'TEXT      HERE'

PRINT "TEXT";SPC(10);"HERE"        Result is 'TEXT          HERE'

The above examples illustrate the difference between TAB and SPC.  See

also  the  SPC  function.  A better way to format PRINT output is with

PRIUT  USING.  Don't  confuse the TAB function with the TAB character,

CHR$(9),  which  is  used  to  format  data using the programmable TAB

stops.

TAN -- Tangent function

TAN (expression)

This function returns the tangent of the numeric expression,  measured

in  radians.  If  the  result  overflows,  TAN(pi/2)  for  example, an

'OVERFLOW' error is reported.

X = TAN(1)             Result is X=1.55740772

To  get  the  tangent  of  an angle measured in degrees,  multiply the

numeric expression by pi/180.

TEMPO -- Set the tempo (speed) of a PLAY string

TEMPO rate

Use  this command to adjust the tempo (speed) of music playback by the

PLAY  command.  The rate determines the duration of a whole note.  The

default is 12, making a whole in 4/4 time last 2 seconds.  The formula

is:

duration = 24/rate

The higher the rate, the faster the note. The range is (1-255).

THEN -- See IF/THEN/ELSE

TO -- See FOR/NEXT/STEP. Also used as a subcommand.

TRAP -- Define an BASIC error handler

TRAP [line_number]

When  turned on,  TRAP intercepts all BASIC execution error conditions

except 'UNDEF'D STATEMENT ERROR'. Even the STOP key can be TRAPped.

When an error occurs,  BASIC saves the error's location,  line number,

and  error  number.  If TRAP is not set,  BASIC returns to direct mode

and displays the error message and line number.  If TRAP is set, BASIC

performs a GOTO to the line number specified in the TRAP statement and

continues executing.

Your  BASIC  error  handling  routine  can  examine  the error number,

message,  and  the  line number where the error occurred and determine

the proper course of action. The system error words are:

ER      Error Number

EL      Error Line (line where the error occurred)

ERR$()  Error Message

If  ER  is  -1,  then  a BASIC error did not occur.  The error routine

should  check  the  disk status words,  in case they were the cause of

the error:

DS      Disk Error Number

DS$     Disk Error Message

Refer to the list of BASIC and Disk error messages in the appendix.

Note that an error in your TRAP routine cannot be trapped.  The RESUME

statement can be used to resume execution -- see RESUME.

TRAP with no line number specified turns off error TRAPping.

10 TRAP 90                       enable trapping

20 FOR I=-5 TO 5

30 PRINT 5/I                     error when I=0

40 NEXT

50 TRAP                          turn trapping off

60 END

70 :

90 PRINT ERR$(ER): RESUME NEXT   error routine

TROFF -- Turn off trace mode

TRON  -- Turn on  trace mode

TROFF

TRON

Trace  mode  is  used  while debugging (fixing) a BASIC program.  TRON

enables tracing,  and TROFF disables tracing.  When the program is run

and  trace  mode  is  on, the line number of the command that is being

executed  is  displayed on the screen.  If there are three commands on

the  line,  the  line number will be displayed three times,  once each

time  one  of the commands is executed.  Trace mode lets you know what

the computer is doing.

Trace  mode  works even when a graphic screen is being displayed,  but

the  line number is still displayed on the text screen so you won't be

able to see it until the graphic screen is turned off. If your program

is  doing  alot  of  PRINT  statements,  the display can seen a little

confusing.

Trace  mode can be set in direct mode to trace the entire program,  or

it  can be turned on and off from within your program to let you trace

only selected portions of the program.

Trace  mode  has  no effect on commands entered in direct (edit) mode.

The NEW command disables trace mode, but RUN and CLR do not.

10 FOR I=-5 TO 5

15 TRON

20 PRINT 5/I

25 TROFF

30 NEXT

TYPE -- Display the contents of a sequential disk file

TYPE "filename" [,Ddrive] [<,|ON>Udevice]

Use  this  command to print the contents of a PETSCII data file on the

screen. The file must contain lines no longer than 255 characters long

and terminated by a return character (CHR$(13)). Lines too long result

in a 'STRING TOO LONG' error.

TYPE "readme"    display the contents of the README file on the screen

The  command sequence below will print the contents of the README file

on a CBM serial bus printer in upper/lower case mode.

OPEN 4,4,7: CMD4: TYPE"readme": CLOSE4

UNTIL -- See DO/LOOP/WHILE/UNTIL/EXIT

USR - Call a user defined machine language function

USR (expression)

When  this  function  is used, the program jumps to a machine language

subroutine  whose  starting  address  must be POKEd into system memory

(BANK 128) at address 760 (low byte) and 761 (high byte), or $2F8 hex.

The  floating  point  value of the numeric expression is passed to the

routine  in  the  Floating point ACCumulator (FACC),  and the value to

be returned is taken from the FACC when the routine ends.

If  the  USR  vector  is  not set up prior to making the USR call,  an

'UNDEF'D FUNCTION'  error is reported.  The routine must be located in

the system bank. The BANK command does not affect USR.

Using  this  method  of  calling a machine language routine requires a

fair  amount  of  set  up and a good knowledge of the lower level math

routines built into BASIC. See the SYS command, which is more commonly

used to call a machine language routine.

The  following  program  illustrates  the  basic  steps  required  for

installing a USR routine and calling it:

10 BANK 128                               System bank for poke & load

20 UV = DEC("1800")                       Where my routine is

30 BLOAD "my user routines",P(UV)         Load my routine

40 POKE DEC("2F8"), UV AND 255, UV / 256  Set up USR address

50 x = USR(123): PRINT X                  Call my routine with the

the value 123, get back and

print whatever my routine

leaves in FACC

The following program actually works.  It points the USR vector to the

BASIC  math  jump  table  entry for the routine which inverts the sign

of the number in the FACC. Type in positive & negative numbers:

10 BANK 128                               System bank for poke

20 POKE DEC("2F8"), DEC("33"), DEC("7F")  Set up USR address

30 DO: INPUT"SIGNED NUMBER"; N            Get number input

40 : PRINT USR(N)                         Display USR output

50 : LOOP UNTIL N=0                       End if user types zero

USING -- See PRINT USING

VAL -- Get the numerical value of a string

VAL (string)

The  VALue  function  converts a string into a number.  The conversion

starts  with  the first character and ends at the end of the string or

the first character that is not allowed in normal number input. Spaces

are  ignored.  If  the  first  character  of the string is not a legal

character, a zero is returned.

The  VAL  function  works the same way the INPUT and READ commands do.

VAL is the opposite of STR$.

X = VAL(" 123")         Result is X =  123

X = VAL("-123")         Result is X = -123

X = VAL(" 9E-02")       Result is X = .09

VERIFY -- Compare a program or data in memory with a disk file

VERIFY "filename" [,device_number [,relocate_flag]]

This  command  is just like a LOAD command,  except instead of putting

the  data  read  from a file into memory,  the computer compares it to

what  is already in memory. If there's any difference at all a 'VERIFY

ERROR' is reported.

The filename  must be given, and pattern matching may be used.  In the

case  of  dual  drive  systems,  the  drive number must be part of the

filename.  If  a  device  number is given,  the file is sought on that

unit,  which  must  be a disk drive.  If a device number is not given,

the default system drive is used. See also DVERIFY.

Note:  If  the  BASIC  program  in  memory  is not located at the same

address  as  the  version  on disk was SAVEd from,  the files will not

match even if the program is otherwise identical.

The relocate_flag is used to VERIFY binary files. If the relocate_flag

is present and non-zero,  the file will be compared to memory starting

at the address stored on disk when the file was SAVEd. The memory bank

used is the bank given in the last BANK statement.  The ending address

is  determined  by  the length of the disk file.  The comparison halts

on  the  first  mismatch  or  at  the end of the file.  The area to be

compared  must  be  confined  to the indicated memory bank. Do not use

the relocate_flag to verify BASIC programs. See also BVERIFY.

VERIFY "myprogram"

Good:   SEARCHING FOR 0:myprogram     Bad:   SEARCHING FOR 0:myprogram

VERIFYING                            VERIFYING

OK                                   ?VERIFY ERROR

VERIFY "PROG"        Compares BASIC program in memory to file PROG

on the default system disk.

VERIFY FILE$,DRV     Compares program in memory to a program whose

name  is  in  the  variable FILE$ on the unit

whose number is in DRV.

VERIFY "0:PROG",8    Compares memory to BASIC program PROG on unit

8, drive 0.

BANK 128             Compares  a  binary  file  into  memory.  The

VERIFY "BIN",8,1     address  used  comes from the disk file,  but

you must specify the memory bank.

VIEWPORT --     [*** CURRENTLY UNIMPLEMENTED ***]

VOL -- Set audio volume level

VOL volume

[*** THIS COMMAND WILL CHANGE ***]

This statement sets the volume level for SOUND and PLAY statements.

VOLUME  can  be  set  from  0 to 15, where 15 is the maximum volume. A

volume  of  0  turns  sound  output off.  VOLume affects all 3 voices.

Note that PLAY strings can change the volume, too.

WAIT -- Pause BASIC program until a memory state satisfied

WAIT address, and_mask [,xor_mask]

The WAIT statement causes program execution to be suspended until data

at a specified memory location matches a given bit pattern.  It's used

to pause your program until an event occurs.

The  event  could be an I/O state (such as a fire button or peripheral

port  change),  a hardware state (such as the raster position or RS232

status),  or  memory  change  caused  by an interrupt event (such as a

keyboard scan).

The WAIT statement tells the computer to read (PEEK) a memory location

(0-65535)  and  AND  the  value  it  got  with  the number in and_mask

(0-255).  If the result is zero, repeat the operation until the result

is not zero.  This is like the following BASIC instructions,  but much

faster:

DO: result = PEEK(address): LOOP UNTIL (result AND and_mask) <> 0

This  works  if  the  state  you are WAITing for is non-zero (a one or

"high"  state).  If you want to wait for a zero state (a "low" state),

you need to use the xor_mask option to "flip" the bits of the result.

Note  that  it's  possible  to "hang" your program indefinitely if the

state you are waiting for never happens or you specify the wrong data.

Press the STOP and RESTORE keys at the same time to get control back.

Be  sure  to use the BANK command before you tell the computer to WAIT

to specify  which 64K memory bank the address is in.  Note that a BANK

number greater than 127 (i.e., a bank number with the most significant

bit  set)  must  be  used to address an I/O location,  such as the VIC

chip. Refer to the system memory map for details.

10 BANK 128                     Wait for the VIC raster to be

20 WAIT DEC("D011"), 128        offscreen (want RC8 = 1)

10 BANK 128                     Wait for the VIC raster to be

20 WAIT DEC("D011"), 128, 128   onscreen (want RC8 = 0)

10 BANK 128

20 WAIT DEC("D3"), 1            Wait for user to press <SHIFT> key

30 WAIT DEC("D3"), 2            Wait for user to press <C=> key

40 WAIT DEC("D3"), 4            Wait for user to press <CTRL> key

50 WAIT DEC("D3"), 8            Wait for user to press <ALT> key

WHILE -- See DO/LOOP/WHILE/UNTIL/EXIT

WIDTH --         [*** CURRENTLY UNIMPLEMENTED ***]

WINDOW -- Set a text window

WINDOW  left_column, top_row, right_column, bottom_row [,clear]

This  command defines a logical text screen window.  All text I/O will

be  confined  to this window.  The row parameters must be in the range

(0-24),  and  the  column  parameters  must be in the range (0-79) for

80-column screens or (0-39) for 40-column screens.  The parameters are

always  referenced  to the physical screen  (i.e., you cannot define a

window  within a window).  If the clear flag is given,  the new window

area will be cleared after it's set up.

Use the RWINDOW function to get the current window size.

You  are  responsible  for  saving  and  restoring  screen data in all

windows because the WINDOW command simply sets the window margins. The

WINDOW  command  does  not  draw  a border around a window.  All color

commands  and  screen  modes (such as scroll disable, TAB stops, etc.)

are global.

Two  consecutive  "home"  characters  will reset the window definition

back to the physical screen.

WINDOW 0,0,39,24          Define  a  window  in  80-column  mode

that is the left half of the screen.

WINDOW 40,0,79,24         Define  a  window  in  80-column  mode

that is the right half of the screen.

WINDOW 0,0,79,12          Define  a  window  in  80-column  mode

that is the top half of the screen.

WINDOW 0,13,79,24         Define  a  window  in  80-column  mode

that is the bottom half of the screen.

WINDOW 20,6,59,12,1       Define  a  window in 80-column mode in

the center of the screen and clear it.

The  window  is 12 characters high and

40 characters wide.

PRINT CHR$(19)CHR$(19);   Reset  the  window back to full screen

in either 40 or 80-column mode and put

the cursor in top left corner.

XOR -- Exclusive-Or function

XOR (number,number)

The  XOR  function returns a numeric value equal to the logical XOR of

two numeric expressions, operating on the binary value of the unsigned

16-bit integers in the range (0 to 65535).  Numbers outside this range

result in an 'ILLEGAL QUANTITY' error.

X = XOR(4,12)           Result is X= 8

X = XOR(2,12)           Result is X=14

3.1.4. VARIABLES

The C64DX uses three types of variables in BASIC:

floating point          X

integer                 X%

string                  X$

Normal  NUMERIC VARIABLES,  also called floating point variables,

can have any from up to nine digits of accuracy. When a number becomes

larger  than  nine  digits  can show,  as in +10 or -10,  the computer

displays it in scientific notation form, with the number normalized to

1  digit  and  eight decimal places,  followed by the letter E and the

power  of  ten  by  which the number is multiplied.  For example,  the

number 12345678901 is displayed as 1.23456789E+10.

INTEGER VARIABLES  can  be used when the number is a signed whole

number from +32767 to -32768.  Integer data is a number like 5, 10, or

-100.  Integers  take  up  less  space  than floating point variables,

particularly when used in an array.

STRING VARIABLES  are  those  used for character data,  which may

contain  numbers,  letters,  and any other character that the computer

can make. An example of string data is "Commodore C64DX".

VARIABLE NAMES may consist of a single letter,  a letter followed

by  a  number,  or  two  letters.  Variable names may be longer than 2

characters,  but  only  the  first two are significant.  An integer is

specified  by  using  the  percent  (%)  sign after the variable name.

String variables have a dollar sign ($) after their names.

EXAMPLES:

Numeric Variable Names: A,  A5 , BZ

Integer Variable Names: A%, A5%, BZ%

String Variable Names : A$, A5$, BZ$

ARRAYS are lists of variables with the same name,  using an extra

number  (or  numbers)  to specify an element of the array.  Arrays are

defined using the DIM statement,  and may be floating point,  integer,

or  string  variable arrays.  The array variable name is followed by a

set  of  parentheses  ()  enclosing  the number of the variable in the

list.

EXAMPLE:

A(7), BZ%(11), A$(87)

Arrays can have more than one dimension.  A two dimensional array

may  be  viewed  as having  rows  and  columns,  with the first number

identifying  the  row and the second number identifying the column (as

if specifying a certain grid on the map).

EXAMPLE:

A(7,2), BZ%(2,3,4), Z$(3,2)

RESERVED  VARIABLE  NAMES  are names that are reserved for use by

the computer,  and may not be used for another purpose.  These are the

variables DS, DS$, ER, ERR$, EL, ST, TI, and TI$.  KEYWORDS such as TO

and IF or any other names that contain KEYWORDS,  such as RUN, NEW, or

LOAD cannot be used.

ST  is  a  status  variable  for input and output  (except normal

screen/keyboard operations). The value of ST depends on the results of

the last I/O operation.  In general,  if the value of ST is 0 then the

operation was successful.

TI and TI$ are variables that relate to the real-time clock built

into  the C64DX.  The system clock is reset to zero when the system is

powered up or reset, and can be changed by the user or a program.

TI$="hh:mm:ss.t"   Allows  optional colons to delimit parameters and

allows input to be abbrieviated  (eg., TI$="h:mm"

or   even   TI$=""),   defaulting   to  "00"  for

unspecified parameters. 24-hour clock (00:00:00.0

to 23:59:59.9).

TI                 24-hour TOD converted into tenths of seconds.

The  value  of the clock is lost when the computer is turned off.

It starts at zero when the computer is turned on, and is reset to zero

when the value of the clock exceeds 23:59:59.9.

The variable DS reads the disk drive command channel, and returns

the  current  status  of the drive.  To get this information in words,

PRINT  DS$.  These  status  variables are used after a disk operation,

like DLOAD or DSAVE, to find out why the error light on the disk drive

is blinking.

ER,  EL,  and ERR$ are variables used in error trapping routines.

They  are  usually  only useful within a program.  ER returns the last

error encountered since the program was RUN.  EL is the line where the

error  occurred.  ERR$  is a function that allows the program to print

one of the BASIC error messages.  PRINT ERR$(ER) prints out the proper

error message.

3.1.5. OPERATORS

The  BASIC OPERATORS  include ARITHMETIC, RELATIONAL, and LOGICAL

OPERATORS. The ARITHMETIC operators include the following signs:

+ addition

- subtraction

* multiplication

/ division

^ raising to a power (exponentiation)

On a line containing more than one operator, there is a set order

in  which  operations  always  occur.  If  several  operators are used

together,   the   computer   assigns   priorities  as  follows.  First

exponentiation,  then  multiplication and division, and last, addition

and  subtraction.  If  two  operators  have  the  same  priority, then

calculations  are  performed  in  order  from left to right.  If these

operations are to occur in a different order, BASIC 10.0 allows giving

a  calculation  a  higher  priority  by placing parentheses around it.

Operations enclosed in parentheses will be calculated before any other

operation.  Make  sure that the equations have the same number of left

and  right  parentheses,  or a SYNTAX ERROR message is posted when the

program is run.

There are also operators for equalities and inequalities,  called

RELATIONAL  operators.  Arithmetic operators always take priority over

relational operators.

=            is equal to

<            is less than

>            is greater than

<= or =<     is less than or equal to

>= or =>     is greater than or equal to

<> or ><     is not equal to

Finally,  there  are three LOGICAL operators, with lower priority

than both arithmetic and relational operators:

AND

OR

NOT

These  are most often used to join multiple formulas in IF...THEN

statements.  When  they  are used with arithmetic operators,  they are

evaluated last  (i.e.,  after + and -).  If the relationship stated in

the expression is the true the result is assigned an integer of -1 and

if false an integer of 0 is assigned. There is also an XOR function.

EXAMPLES:

IF A=B AND C=D THEN 100         requires both A=B & C=D to be true

IF A=B OR C=D THEN 100          allows either A=B or C=D to be true

A=5:B=4:PRINT A=B               displays  0

A=5:B=4:PRINT A>3               displays -1

PRINT 123 AND 15:PRINT 5 OR 7   displays 11 and 7

3.1.6. ERROR MESSAGES

3.1.6.1. BASIC ERROR MESSAGES

The  following  error  messages  are  displayed  by BASIC.  Error

messages  can  also  be displayed with the use of the ERR$() function.

The  error  number refers only to the number assigned to the error for

use with this function.  In direct mode,  DOS error messages (DS$) are

automatically  displayed. They are described in the section after this

one.

ERROR#     ERROR NAME                       DESCRIPTION

----------------------------------------------------------------------

1  TOO MANY FILES         There is a limit of 10 files OPEN at one

time.

2  FILE OPEN              An attempt was made to open a file using

the number of an already open file.

3  FILE NOT OPEN          The file number specified in an I/O

statement must be opened before use.

4  FILE NOT FOUND         No file with that name exists on the

specified drive.

5  DEVICE NOT PRESENT     The required I/O device not available.

6  NOT INPUT FILE         An attempt made to read data from a file

that was opened for writing.

7  NOT OUTPUT FILE        An attempt was made to write data to a

file that was opened for reading.

8  MISSING FILE NAME      Filename was missing in command.

9  ILLEGAL DEVICE NUMBER  An attempt was made to use a device

improperly (SAVE to the screen, etc) or

an illegal device number was specified.

10  NEXT WITHOUT FOR       Either loops are nested incorrectly, or

there is a variable name in a NEXT

statement that doesn't correspond with

one in FOR.

11  SYNTAX ERROR           A statement is unrecognizable by  BASIC.

This could be  because of missing or

extra parenthesis, parameters, delimiters,

or a misspelled keyword.

12  RETURN WITHOUT GOSUB   A RETURN statement was encountered when no

GOSUB statement was active.

13  OUT OF DATA            A READ statement was encountered with no

DATA left unREAD.

14  ILLEGAL QUANTITY       A number used as an argument is  outside

the allowable range (too big or too small)

15  OVERFLOW               The result of a computation is larger than

the largest number allowed (1.701411834E+38)

16  OUT OF MEMORY          There is not enough memory for the program,

or variables, or there are too many DO, FOR

or GOSUB statements in effect.

17  UNDEF'D STATEMENT      A line number referenced doesn't exist.

18  BAD SUBSCRIPT          The program tried to reference an element

of an array out of the range specified by

a DIM statement, a missing DIM statement

or a mistyped function name.

19  REDIM'D ARRAY          An array can only be DIMensioned once.

20  DIVISION BY ZERO       Division by zero is illegal.

21  ILLEGAL DIRECT         Command is only allowed to be used in a

program.

22  TYPE MISMATCH          A numeric variable was used in place of

a string variable or vice versa.

23  STRING TOO LONG        An attempt was made to assign more than

255 characters to a string, or enter more

than 160 characters from the keyboard, or

to input more than 255 characters from a

file.

24  FILE DATA              The wrong type of data was read from a

file.

25  FORMULA TOO COMPLEX    An expression is too complicated for BASIC

to process all at one time. Break it into

smaller pieces or use fewer parentheses.

26  CAN'T CONTINUE         The CONT command does not work if the

program was not RUN, there was an error

or a line has been edited.

27  UNDEFINED FUNCTION     An attempt was made to use a user defined

function that was never defined.

28  VERIFY                 The program on disk does not match the

program in memory.

29  LOAD                   There was a problem loading.

30  BREAK                  The program was halted by the STOP key or

a STOP statement.

31  CAN'T RESUME           A RESUME statement was encountered without

a TRAP in effect, or an error occurred in

the trap handler itself.

32  LOOP NOT FOUND         The program encountered a DO statement and

cannot find the corresponding LOOP.

33  LOOP WITHOUT DO        A LOOP was encountered without a DO

statement active.

34  DIRECT MODE ONLY       A command was used in a program that can

only be used in direct mode.

35  NO GRAPHICS AREA       A graphics command was used before a

graphics screen was defined and opened.

36  BAD DISK               A BOOT SYS command failed because the disk

could not be read.

37  BEND NOT FOUND         A BEND statement not found for BEGIN.

38  LINE NUMBER TOO LARGE  A line number cannot exceed 64000.

39  UNRESOLVED REFERENCE   Renumber failed because a referenced line

number does not exist.

40  UNIMPLEMENTED COMMAND  The given command is not currently

implemented in this computer.

41  FILE READ              There was a problem reading data from a

disk file. Similar to LOAD ERROR.

3.1.6.2. DOS ERROR MESSAGES

The  following error messages are returned through the DS and DS$

variables.  If  a disk command is type in direct mode,  these messages

will  be displayed automatically.  NOTE: DOS message numbers less than

20 are advisory and are not necessarily errors.  DOS messages may vary

slightly depending upon the drive model.  Refer to your DOS manual for

details.

ERROR #                           DESCRIPTION

------- --------------------------------------------------------------

00:   OK (no error)

01:   FILES SCRATCHED (not an error)

The following number (track) tells how many files were deleted

by the scratch command.

02:   PARTITION SELECTED (not an error)

The requested disk partition (subdirectory) has been selected.

03:   FILES LOCKED

The requested file(s) have been locked.

04:   FILES UNLOCKED

The requested file(s) have been unlocked.

05:   FILES RESTORED

The requested file(s) have been recovered (undeleted).

20:   READ ERROR (block header not found)

The  disk  controller  is  unable  to locate the header of the

requested  data  block. Caused by an illegal sector number, or

the header has been destroyed.

21:   READ ERROR (no sync character)

The  disk  controller  is  unable to detect a sync mark on the

desired track.  Caused by misalignment of the read/write head,

no  diskette  is present,  or unformatted or improperly seated

diskette. Can also indicate a hardware failure.

22:   READ ERROR (data block not present)

The  disk  controller  has  been requested to read or verify a

data block that was not properly written. This error occurs in

conjunction  with  the BLOCK commands and indicates an illegal

track and/or sector request.

23:   READ ERROR (checksum error in data block)

This  error message indicates that there is an error in one or

more  of  the data bytes.  The data has been read into the DOS

memory,  but  the  checksum  over  the data is in error.  This

message may also indicate grounding problems.

24:   READ ERROR (byte decoding error)

The  data  or  header has been read into the DOS memory, but a

hardware  error has been created due to an invalid bit pattern

in  the  data  byte.  This message may also indicate grounding

problems.

25:   WRITE ERROR (write-verify error)

This message is generated if the controller detects a mismatch

between the written data and the data in the DOS memory.

26:   WRITE PROTECT ON

This  message  is  generated  when  the  controller  has  been

requested to write a data block while the write protect switch

is depressed.

27:   READ ERROR

This  message  is  generated  when  a checksum error is in the

header.

28:   WRITE ERROR

This error message is generated when a data block is too long.

29:   DISK ID MISMATCH

This  message  is  generated  when  the  controller  has  been

requested to access a diskette which has not been initialized.

The message can also occur if a diskette has a bad header.

30:   SYNTAX ERROR (general syntax)

The  DOS  cannot  interpret  the  command  sent to the command

channel.  Typically,  this  is  caused by an illegal number of

file names,  or patterns are illegally used.  For example, two

file names appear on the left side of the COPY command.

31:   SYNTAX ERROR (invalid command)

The DOS does not recognize the command. The command must start

in the first position.

32:   SYNTAX ERROR (invalid command)

The command sent is longer than 58 characters.

33:   SYNTAX ERRROR (invalid file name)

Pattern  matching  is  invalidly  used  in  the  OPEN  or SAVE

command.

34:   SYNTAX ERROR (no file given)

The  file name was left out of the command or the DOS does not

recognize it as such.

39:   SYNTAX ERROR (invalid command)

This  error  may  result  if  the  command sent to the command

channel (secondary address 15) is unrecognized by the DOS.

40:   UNIMPLEMENTED COMMAND

Command is not implemented at this time.

41:   FILE READ

The file cannot be read.

50:   RECORD NOT PRESENT

Result  of disk reading past the last record through INPUT# or

GET# commands.  This message will also occur after positioning

to  a  record  beyond end_of_file  in a relative file.  If the

intent is to expand the file by adding the new record  (with a

PRINT# command),  the error message may be ignored.  INPUT and

GET  should  not  be  attempted  after  this error is detected

without first repositioning.

51:   OVERFLOW IN RECORD

PRINT#  statement  exceeds  record  boundary.  Information  is

truncated. Since the carriage return which is sent as a record

terminator  is  counted in the record size,  this message will

occur  if  the  total characters in the record  (including the

final carriage return) exceeds the defined size.

52:   FILE TOO LARGE

Record  position  within  a  relative file indicates that disk

overflow will result.

53:   BIG RELATIVE FILES DISABLED

60:   WRITE FILE OPEN

This  message is generated when a write file that has not been

closed is being opened for reading.

61:   FILE NOT OPEN

This  message  is generated when a file is being accessed that

has  not  been opened in the DOS.  Sometimes,  in this case, a

message is not generated: the request is simply ignored.

62:   FILE NOT FOUND

The requested file does not exist on the indicated drive.

63:   FILE EXISTS

The  file name of the file being created already exists on the

diskette.

64:   FILE TYPE MISMATCH

The  requested  access mode is not possible using the filetype

given.

65:   NO BLOCK

The  sector  you  tried  to  allocate with the B-A command was

already allocated.  The track and sector numbers hold the next

higher,  available track  and  sector.  If the track number is

zero, no higher sectors are free (try a lower track & sector).

66:   ILLEGAL TRACK AND SECTOR

The  DOS  has  attempted to access a track or block which does

not  exist  in  the  format  being  used.  This may indicate a

problem reading the pointer of the next block.

67:   ILLEGAL SYSTEM T OR S

This  special  error message indicates an illegal system track

or sector.

70:   NO CHANNEL

The requested channel is not available, or all channels are in

use.  A  maximum of five sequential files may be opened at one

time  to  the  DOS. Direct access channels may have six opened

files.

71:   DIRECTORY ERROR

The BAM is corrupted. Try initializing the disk.

72:   DISK FULL

Either the blocks on the diskette are used or the directory is

at  its  entry  limit.  DISK  FULL is sent when two blocks are

available  to  allow  the current file to be closed before its

data is lost.

73:   DOS MISMATCH (also the powerup message)

Initially  given  at  powerup  to  identify the drive. On some

drives  this  message  is  given  as  an error to indicate the

media was formatted by an incompatible DOS.

74:   DRIVE NOT READY

An  attempt  has  been  made  to  access the Floppy Disk Drive

without any diskette present.

75:   FORMAT ERROR

76:   CONTROLLER ERROR

The DOS has determined that the hardware is malfunctioning.

77:   SELECTED PARTITION ILLEGAL

An  attempt  was made to access a partition as a subdirectory,

but  it  has  no directory track or does not meet the criteria

of a directory partition.

78:   DIRECTORY FULL

There is no more room in the directory sector for another file

entry. Delete a file to make room, or change disks.

79:   FILE CORRUPTED

The DOS has determined that a file is bad, probably having bad

links.  Prepare  a  new  disk  and  copy the good files to it.

Could be the result of an unsuccessful file recovery.

3.2. MACHINE LANGUAGE MONITOR

3.2.1. INTRODUCTION

The  MONITOR is a built in machine language program that lets the

user  easily  write  machine  language  programs.  The  C64DX  MONITOR

includes a machine language monitor, an assembler, and a disassembler.

Machine  language  programs  written using the MONITOR can run by

themselves,  or be used as very fast 'subroutines' for BASIC programs.

Care  must  be  taken  to  position  the assembly language programs in

memory  so  that  the  BASIC  program  does not overwrite them and the

proper   memory   is   in  context  at  all  times  (including  during

interrupts).

3.2.2. MONITOR COMMANDS

A       ASSEMBLE        - Assemble a line of 4502 code

C       COMPARE         - Compare two sections of memory

D       DISASSEMBLE     - Disassemble a line of 4502 code

F       FILL            - Fill a section of memory with a value

G       GO              - Start execution at specified address

H       HUNT            - Find specified data in a section of memory

L       LOAD            - Load a file from disk

M       MEMORY          - Dump a section of memory

R       REGISTERS       - Display the contents of the 4502 registers

S       SAVE            - Save a section of memory to a disk file

T       TRANSFER        - Transfer memory to another location

V       VERIFY          - Compare a section of memory with a disk file

E       EXIT            - Exit Monitor mode

.       <period>        - Assembles a line of 4502 code

>       <greater-than>  - Modifies memory

;       <semicolon>     - Modifies register contents

@       <at sign>       - Display disk status

$       <hex>           - Display hex, decimal, octal, and binary value

+       <decimal>

&       <octal>

%       <binary>

The MONITOR accepts binary,  octal, decimal and hexadecimal values for

any  numeric field.  Numbers prefixed by one of the characters $ + & %

are interpreted as base  16,  10, 8, or 2 values respectively.  In the

absence of a prefix, the base defaults to hexadecimal always.

The  assembler  will use the base page form of an instruction wherever

possible  unless the address field is preceded by extra zeros to force

the absolute form (except binary notation).

The most significant byte of a 24-bit (3-byte) address field specifies

the  memory  BANK  to  implement  at  the  time  the  given command is

executed.  BANK bytes with the MSB set  (i.e., banks greater than $7F)

mean "use the current system configuration", which always includes the

I/O area. If a BANK is not specified, BANK 0 is assumed.

BANK 00         internal RAM bank 0 (System, BASIC program)

BANK 01         internal RAM hank 1 (DOS, BASIC vars, color bytes)

BANK 02         internal ROM bank 0 (DOS, C64 mode, CHRSETS)

BANK 03         internal ROM bank 1 (Monitor, C65 mode)

BANK 04-07      reserved for future expansion

BANK 08-7F      expansion RAM (graphic screens, RAM disk, etc.)

BANK 80-FF      MSB set means current config & I/O

The  monitor  supports  the editor autoscroll feature for memory dumps

(forwards and backwards) and disassemblies (forward disassembly only).

To send dump output to a printer, from BASIC open a CMD channel to the

printer and enter the monitor (OPEN 4,4: CMD4: MONITOR). Give the dump

command desired; output will be to the printer.

3.2.3. MONITOR COMMAND DESCRIPTIONS

COMMAND:       A

PURPOSE:       Enter a line of assembly code.

SYNTAX:        A <address> <mnemonic> <operand>

<address>      A number indicating the location in memory to place the

assembled binary code.

<mnemonic>     A 4502 assembly language mnemonic, eg., LDA

<operand>      The operand,  when required, can be of any of the legal

addressing modes.

A  <RETURN>  is used to indicate the end of the assembly line.  If are

any errors on the line,  a question mark is displayed to an error, and

the cursor  moves to the next line.  The screen can be used to correct

the error(s) on that line.

As each line is entered,  the machine code is written to the specified

address and the line is automatically disassembled.

Base  page  and  relative  addresses  are calculated for you,  and the

appropriate  word  or  byte  relative  mode selected automatically. To

force an absolute addressing mode, supply leading zeros if necessary.

.A 1800 LDX #$00

.A 1802

NOTE: A period (.) is equal to the ASSEMBLE command.

. 1900 LDA #$23

COMMAND:        C

PURPOSE:        Compare two areas of memory

SYNTAX:         C <address_1> <address_2> <address_3>

<address_1>     A number indicating the start of the area of memory to

compare against.

<address_2>     A number indicating the end of the area of  memory  to

compare against.

<address_3>     A number indicating the start of  the  other  area  of

memory to compare with.

The  following example compares $8000-$9FFF in bank 0 with $8000-$9FFF

in  bank  1.  Addresses of data that does not match are printed on the

screen.

C 8000 9FFF 18000

COMMAND:       D

PURPOSE:       Disassemble machine code

SYNTAX:        D [address_1 [address_2] ]

<address_1>    A number setting the address to start the disassembly.

<address_2>    An optional ending address of code to be disassembled.

The output of the disassembly is the same as that of an assembly, only

preceded by a comma instead of an A or period. The object code is also

displayed.  Relative addresses in the disassembly are displayed as the

16-bit destination.

A  disassembly  listing  can be modified using the screen editor.  Any

changes  to  the  mnemonic  or  operand  on  the screen,  then hit the

<RETURN>.   This   enters   the  line  and  calls  the  assembler  for

instructions. The object code cannot be modified this way.

A  disassembly  can  be  paged.  Typing a D<RETURN> causes the next of

disassembly  to  be displayed. The autoscroll feature works in forward

mode  only,  because backwards disassembly is not possible because all

256 opcodes are defined in the 4502 processor.

The following example disassembles from ROM bank 3:

D 3F000 3F005

. 03F000 A9 09      LDA #$09

. 03F002 A0 FF      LDY #$FF

. 03F004 18         CLC

. 03F005 86 C2      STX $C2

Note that banks wrap to the next higher bank number.

COMMAND:        F

PURPOSE:        Fill a range of locations with a specified byte.

SYNTAX:         F <address_1> <address_2> <byte>

<address_1>     The first location to fill with the <byte>.

<address_2>     The last location to fill with the <byte>.

<byte>          The byte to fill with

This  command  is useful for initializing data structures or any other

RAM area.

F 00600 007FF 00

Fills memory locations from $0600 to $07FF (RAM-0) with $00. Note that

banks  wrap to the next higher bank number.  The maximum area that can

be filled at one time is 64K, limited by the DMA device.

COMMAND:        G

PURPOSE:        Perform a JMP to a specified address

SYNTAX:         G <address>

<address>       The address where execution  is  to  start.  When  the

address is not  specified,  execution  begins  at  the

current  PC.  (The current PC can be viewed or changed

with the R command.)

The  GO  command loads the processor's registers (displayable by the R

command)  and  performs  a  JMP  to  the  specified  starting address.

Caution  is recommended in using the GO command.  To return to MONITOR

mode  after  performing  a GO command,  a BRK instruction must end the

called  routine.  Also,  the  BANK  specified  must  be able to handle

interrupts  (note  that  BANK  bytes  less than $80 do NOT include the

operating system or I/O space).

G FFC800

JuMPs to address $C800 in bank $FF (system configuration).

COMMAND:        H

PURPOSE:        Hunt through memory within a specified range  for  all

occurences of a set of bytes.

SYNTAX:         H <address_1> <address_2> <data>

<address_1>     Address to start at

<address_2>     Last address

<data>          Data  to  search  for.  May  be a number,  sequence of

numbers, or a PETSCII string.

H 02000 0FFFF 46 52 45 44

Hunts  for  the  series  of  bytes $46, $52, $45, $44 in memory bank 0

beginning  at  address  $2000  and  ending at $FFFF.  The addresses of

matches is displayed.

H 0200 0FFFF 'FRED

Hunts  for the PETSCII string following an apostrophe. Note that banks

wrap to the next higher bank number.

COMMAND:        L

PURPOSE:        Load a file from disk.

SYNTAX:         L <"filename"> [,device [,load_address] ]

<"filename">    Is a filename in quotes.

[device]        Is a number indicating the device to load from.

[load_address]  Optional load address.  If  not  given,  the  file  is

loaded  into  memory  at  the 16-bit address stored on

disk (always RAM bank 0).

The  LOAD command causes a file to be loaded into memory.  If the load

address (including BANK) is given, the data is placed there. Otherwise

the  file  is  loaded  into  RAM  bank  0  at  the 16-bit load address

specified  by  the  first  two  bytes read from the PRG (program) type

file. An error occurs if a load overflow the specified bank.

L "filename"

Loads  "filename"  from  default  system  drive into RAM bank 0 at the

address read from the file.

L "filename",+10,80000

Loads  "filename"  from  drive 10 (notice you must specify decimal for

the  drive  number,  or use hex equivalent) into expansion memory bank

8  at  address  $0000.  Note  that spaces between parameters after the

filename are not permitted.

COMMAND:        M

PURPOSE:        Dump a section of memory in hex and PETSCII.

SYNTAX:         M [address_1 [address_2] ]

[address_1]     Starting address of memory dump.  If omitted, one page

is displayed starting from the last address used.

[address_2]     Ending address of memory dump.  If  omitted,  one page

is displayed starting at address_1.

Memory  dump  width  is sized to 40 or 80 columns,  depending upon the

text screen width.  All data is displayed in hexadecimal and  followed

by a PETSCII interpretation of the data in reverse field (non-printing

characters appear as periods).

The autoscroll keys will scroll the dump forwards or backwards. Paging

is also possible by typing M<RETURN>.

The hex field of dump can be edited,  and memory will be updated after

a <RETURN> is typed on the edited line.

M 29000 2900C

>029000 3C 66 6E 6E 60 62 3C 00 :<FNN-B<.

>029008 46 41 49 54 20 4C 55 58 :FAIT LUX

COMMAND:        R

PURPOSE:        Display "shadow" 4502  registers.  The  PC  (address),

SR (status), A,X,Y,Z registers, and SP (stack pointer)

are displayed.

SYNTAX:         R

R

PC   SR AC XR YR SP

; BA1234 00 00 00 00 FB

The  address  field  contains  the  8-bit bank plus the 16-bit segment

address.  The  register  dump  can be edited by changing any field and

pressing return. The data is used by the G (JMP) and J (JSR) commands.

COMMAND:        S

PURPOSE:        Save a section of memory in a disk file.

SYNTAX:         S <"filename">,<device>,<address_1>,<address_2>

<"filename">    Is a filename in quotes.

<address_1>     Starting address of memory to be saved.

<address_2>     Ending address PLUS ONE of memory to be saved.

The  SAVE  command  creates  a PRG (program) type file and copies data

into it from the specified memory area. All parameters are required.

S "filename",8,A0000,AFFFF

Saves  expansion  bank  A  in "filename" on drive 8  (you must specify

decimal  for the drive number,  or use hex equivalent).  The last byte

at $FFFF will not be saved.  Note that spaces between parameters after

the  filename  are not permitted.  The 16-bit segment address is saved

as the first two bytes of the file, but the BANK address is not saved.

The BANK wraps automatically to the next higher bank number,  but note

that LOAD is restricted to one bank, 64K bytes maximum.

COMMAND:        T

PURPOSE:        Transfer (copy) memory from one memory area to another

SYNTAX:         T <address_1> <address_2> <address_3>

<address_1>     Starting address of data to be copied.

<address_2>     Ending address of data to be copied.

<address_3>     Starting address of new location to copy data to.

Data can be copied forwards or backwards to any location,  even within

the  source  range  (eg.,  shift data up or down one byte) without any

problem.  An  automatic  compare  is  performed  for  each  byte,  and

mismatches displayed on the screen.

Because  of  the  compare feature,  it's not recommended you use the T

command  to  copy  data  into  write-only  registers (the palette, for

example). It works, but all the compares will fail.

T 32000 3BFFF 82000

Copies BASIC ROM area to expansion RAM.

COMMAND:        V

PURPOSE:        Verify (compare) a disk file with the memory contents.

SYNTAX:         V <"filename"> [,device [,load_address] ]

<"filename">    Is a filename in quotes.

[device]        Is a number indicating the device the file is on.

[load_address]  Optional  load  address.  If  not  given,  the file is

compared  to  memory  at  the 16-bit address stored on

disk (always RAM bank 0).

The Verify command causes a file to be read and compared to memory. If

the  load  address  (including  BANK)  is  given,  the  data  read  is

compared  to  data  there.  Otherwise the data read is compared to RAM

bank  0  at  the  16-bit load address specified by the first two bytes

of  the PRG (program) type file.  If there is a mismatch,  the message

'VERIFYING ERROR'  is  displayed.  If  the  data  matches,  nothing is

displayed.  An  error  occurs  if  the  compare  address overflows the

specified bank.

V "filename"

Compares "filename" from the default system drive to RAM bank 0 at the

address read from the file.

V "filename",+10,80000

Compares "filename" from drive 10 (notice you must specify decimal for

the drive number, or use hex equivalent) to expansion memory bank 8 at

address   $0000.   Note  that  spaces  between  parameters  after  the

filename are not permitted.

COMMAND:        X

PURPOSE:        Exit to BASIC

SYNTAX:         X

COMMAND:        >               (greater than)

PURPOSE:        Pokes data (1 to 16 bytes) into memory

SYNTAX:         > <address>  [byte]...

<address>       Address to start "poking" or displaying

[byte]          Data to be "poked".  If not given,  nothing is changed

and   the   memory   at  that  location  is  "peeked".

Successive bytes are poked into successive locations.

COMMAND:        @               (at sign)

PURPOSE:        Disk operation: send command, display directory,status

SYNTAX:         @ [device] [,command]

[device]        Disk device number

[command]       Optional command (see DOS manual for specific commands)

This  command  can  be  used to read a drive's status message,  send a

drive a DOS command, or display a disk directory.

@               displays status of default system drive

@9              displays status of drive 9

@+10  or @A     displays status of drive 10

@,$             displays directory of default drive

@9,$            displays status of drive 9

@,S0:*=SEQ      displays all SEQ type files

@,S0:FILE       sends command to delete file "FILE"

3.3. EDITOR

3.3.1. EDITOR ESCAPE SEQUENCES

This  section  contains  a definition of the escape sequences that are

present in the C64DX and a brief description of what each does.

ESCape  sequences  are  given  by  hitting  the  <ESCAPE> key and then

another   key.  In  PRINT  strings,  escape  sequences  are  given  by

printing  the escape character CHR$(27) followed by another character.

In  either  case,  the  "other"  character  is  defined  as one of the

following:

KEY  FUNCTION

---  ----------------------------------------------------

@   Clear from cursor to end of screen

A   Enable auto-insert mode

B   Set bottom of screen window at cursor position

C   Disable auto-insert mode (set overwrite mode)

D   Delete current line

E   Set cursor to non-flashing mode

F   Set cursor to flashing mode

G   Enable bell (control-G)

H   Disable bell

I   Insert line

J   Move to start of current line

K   Move to end of current line

L   Enable scrolling

M   Disable scrolling

N   Normal screen fields        [not implemented on C64DX]

O   Cancel insert, quote, reverse, underline & flash modes

P   Erase from cursor to start of current line

Q   Erase from cursor to end of current line

R   Set screen to reverse video [not implemented on C64DX]

S   Set bold attribute (VIC-III colors 16-31)

T   Set top of screen window at cursor position

U   Unset bold attibute

V   Scroll up

W   Scroll down

X   Swap 40/80 column display output device

Y   Set default tab stops (8 spaces)

Z   Clear all tab stops

[   Set monochrome display (disable attributes)

/   Cancel insert, quote, rvs, ul & flash modes

]   Set color display (enable attributes)

3.3.2. EDITOR CONTROL CODES

This  section  contains  a  definition  of  the control codes that are

present in the C64DX and a brief description of what each does.

Control codes are given by pressing the <CTRL> key at the same time as

another key. In PRINT strings, control codes are given by printing the

control  character  with  the  CHRS()  function.  Control codes appear

within  quoted  strings  as reverse field characters. In any case, the

control characters are:

CHR$    KEYBOARD

VALUE   CONTROL    FUNCTION

-----   --------   ----------------------------------------------

2       B          Underline on

7       G          Bell tone

9       I          Forward TAB

10      J          Line feed

11      K          Disable case change <shift>C= key (was code 9)

12      L          Enable case change <shift>C= key  (was code 8)

14      N          Set display upper/lower case mode

15      O          Flash on

17      Q          Cursor down

18      R          Reverse on

19      S          Home cursor

20      T          Delete previous character

21      U          Backup word

23      W          Advance word

24      X          Tab set/clear

26      Z          Backup TAB

27      [          Escape character

29      ]          Cursor right

Shifted codes

---------------------------------------------------------------------

130                Underline off

142                Set uppercase/graphic mode

143                Flash off

145                Cursor up

146                Reverse mode off

147                Clear screen

148                Insert one character

157                Cursor left

Color codes

---------------------------------------------------------------------

5                white

28                red

30                green

31                blue

129                orange

144                black

149                brown

150                light red

151                light gray

152                medium gray

153                light green

154                light blue

155                dark gray

156                purple

158                yellow

159                cyan

Function keys

----------------------------------------------------------------

3               Stop

16               F9

21               F10

22               F11

23               F12

25               F13

26               F14

131               Run

132               Help

133               F1

134               F3

135               F5

136               F7

137               F2

138               F4

139               F6

140               F8

3.4. KEKNEL

3.4.1. C64DX KERNEL ENTRY POINTS

[*** THE FOLLOWING VECTORS AND JUMP TABLES ARE NOT FINAL ***]

Where the default indirect vectors point to:

FF09   nirq             ;IRQ handler

FF0B   monitor_brk      ;BRK handler (Monitor)

FF0D   nnmi             ;NMI handler

FF0F   nopen            ;open

FF11   nclose           ;close

FF13   nchkin           ;chkin

FF15   nckout           ;ckout

FF17   nclrch           ;clrch

FF19   nbasin           ;basin

FF1B   nbsout           ;bsout

FF1D   nstop            ;stop key scan

FF1F   ngetin           ;getin

FF21   nclall           ;clall

FF23   monitor_parser   ;monitor command parser

FF25   nload            ;load

FF27   nsave            ;save

FF29   talk             ;Low level serial bus routines

FF2B   listen

FF2D   talksa

FF2F   second

FF31   acptr

FF33   ciout

FF35   untalk

FF37   unlisten

FF39   DOS_talk         ;newDOS routines

FF3B   DOS_listen

FF3D   DOS_talksa

FF3F   DOS_second

FF41   DOS_acptr

FF43   DOS_ciout

FF45   DOS_untalk

FF47   DOS_unlisten

FF49   Get_DOS

FF4B   Leave_DOS

FF4D   jmp spin_spout   ;setup fast serial port for input or output

FF50   jmp close_all    ;close all logical files for a given device

FF53   jmp c64mode      ;reconfigure system as a c/64 (no return!)

FF56   jmp monitor_call ;map in Monitor & call it

FF59   jmp bootsys      ;boot alternate system from disk

FF5C   jmp phoenix      ;call cold start routines, disk boot loader

FF5F   jmp lkupla       ;search tables for given la

FF62   jmp lkupsa       ;search tables for given sa

FF65   jmp swapper      ;swap to alternate display device

FF68   jmp pfkey        ;program function key

FF6B   jmp setbnk       ;set bank for load/save/verify/open

FF6E   jmp jsr_far      ;JSR to any bank, RTS to calling bank

FF71   jmp jmp_far      ;JMP to any bank

FF74   jmp lda_far      ;LDA (X),Y from bank Z

FF77   jmp sta_far      :STA (X),Y to   bank Z

FF7A   jmp cmp_far      ;CMP (X),Y to   bank Z

FF7D   jmp primm        ;print immediate (always JSR to this routine!)

FF80   <FF>             ;release number of C65 Kernel ($FF=not released)

FF81   jmp cint         ;init screen editor & display chips

FF84   jmp ioinit       ;init I/O devices (ports, timers, etc.)

FF87   jmp ramtas       ;initialize RAM for system

FF8A   jmp restor       ;restore vectors to initial system

FF8D   jmp vector       ;change vectors for user

FF90   jmp setmsg       ;control OS messages

FF93   jmp (isecond)    ;send sa after listen

FF96   jmp (italksa)    ;send sa after talk

FF99   jmp memtop       ;set/read top of memory

FF9C   jmp membot       ;set/read bottom of memory

FF9F   jmp key          ;scan keyboard

FFA2   jmp settmo       ;old IEEE set timeout value

FFA5   jmp (iacptr)     ;read a byte from active serial bus talker

FFA8   jmp (iciout)     ;send a byte to active serial bus listener

FFAB   jmp (iuntalk)    ;command serial bus device to stop talking

FFAE   jmp (iunlisten)  ;command serial bus device to stop listening

FFB1   jmp (ilisten)    ;command serial bus device to listen

FFB4   jmp (italk)      ;command serial bus device to talk

FFB7   jmp readss       ;return I/O status byte

FFBA   jmp setlfs       ;set la, fa, sa

FFBD   jmp setnam       ;set length and fn adr

FFC0   jmp (iopen)      ;open logical file

FFC3   jmp (iclose)     ;close logical file

FFC6   jmp (ichkin)     ;open channel in

FFC9   jmp (ickout)     ;open channel out

FFCC   jmp (iclrch)     ;close I/O channel

FFCF   jmp (ibasin)     ;input from channel

FFD2   jmp (ibsout)     ;output to channel

FFD5   jmp load         ;load from file

FFD8   jmp save         ;save to file

FFDB   jmp Set Time     ;set internal clock

FFDE   jmp Read Time    :read internal clock

FFE1   jmp (istop)      ;scan stop key

FFE4   jmp (igetin)     ;get char from queue

FFE7   jmp (iclall)     ;clear all logical files (see close all)

FFEA   jmp ScanStopKey  ;(was increment clock) & scan stop key

FFED   jmp scrorg       ;return current screen window size

FFF0   jmp plot         ;read/set x,y coord

FFF3   jmp iobase       ;return I/O base

FFF6   c65mode          ;C64/C65 interface

FFF8   c65mode

FFFA   nmi              ;processor hardware vectors

FFFC   reset

FFFE   irq_kernel

3.4.2. C64DX EDITOR JUMP TABLE

[*** THE FOLLOWING VECTORS AND JUMP TABLES ARE NOT FINAL ***]

E000   cint            ;initialize editor & screen

E003   disply          ;display character in .a, color in .x

E006   lp2             ;get a key from IRQ buffer into .a

E009   loopS           ;get a chr from screen line into .a

E00C   print           ;print character in .a

E00F   scrorg          ;get size of window (rows,cols) in .x, .y

E012   keyboard_scan   ;scan keyboard subroutine

E015   repeat          ;repeat key logic & CKIT2 to store decoded key

E018   plot            ;read or set (.c) cursor position in .x, .y

E01B   mouse_cmd       ;install/remove mouse driver

E01E   escape          ;execute escape function using chr in .a

E021   keyset          ;redefine a programmable function key

E024   editor_irq      ;IRQ entry

E027   palette_init    ;initialize VIC palette

E02A   swap            ;40/80 mode change

E02D   window          ;set top left or bottom right (.c) of window

E030   cursor          ;turn on or off (.c) soft cursor

3.4.3. C64DX BASIC JUMP TABLE

[*** THE FOLLOWING VECTORS AND JUMP TABLES ARE NOT FINAL ***]

Format Conversions

7F00       ayint        ;convert floating point to integer

7F03       givayf       ;convert integer to floating point.

7F06       fout         ;convert floating point to ASCII string

7F09       val_1        ;convert ASCII string to floating point

7F0C       getadr       ;convert floating point to an address

7F0F       floatc       ;convert address to floating point

Math Functions

7F12       fsub         ;MEM - FACC

7F15       fsubt        ;ARG - FACC

7F18       fadd         ;MEM + FACC

7F1B       faddt        ;ARG - FACC

7F1E       fmult        ;MEM * FACC

7F21       fmultt       ;ARG * FACC

7F24       fdiv         ;MEM / FACC

7F27       fdivt        ;ARG / FACC

7F2A       log          ;compute natural log of FACC

7F2D       int          ;perform BASIC INT() on FACC

7F30       sqr          ;compute square root of FACC

7F33       negop        ;negate FACC

7F36       fpwr         ;raise ARG to the MEM power

7F39       fpwrt        ;raise ARG to the FACC power

7F3C       exp          ;compute EXP of FACC

7F3F       cos          ;compute COS of FACC

7F42       sin          ;compute SIN of FACC

7F45       tan          ;compute TAN of FACC

7F48       atn          ;compute ATN of FACC

7F4B       round        ;round FACC

7F4E       abs          ;absolute value of FACC

7F51       sign         ;test sign of FACC

7F54       fcomp        ;compare FACC with MEM

7F57       rnd_0        ;generate random floating point number

Movement

7F5A       conupk       ;move RAM MEM to ARG

7F5D       romupk       ;move ROM MEM to ARG

7F60       movfrm       :move RAM MEM to FACC

7F63       movfm        :move ROM MEM to FACC

7F66       movmf        :move FACC to MEM

7F69       movfa        ;move ARG to FACC

7F6C       movaf        ;move FACC to ARG

7F6F       run

7F72       runc

7F75       clear

7F78       new

7F7B       link_program

7F7E       crunch

7F81       FindLine

7F84       newstt

7F87       eval

7F8A       frmevl

7F8D       run_a_program

7F90       setexc

7F93       linget

7F96       garba2

7F99       execute_a_line

7F9C       chrget

7F9F       chrgot

7FA2       chkcom

7FAS       frmnum

7FA8       getadr

7FAB       getnum

7FAE       getbyt

7FB1       plsv

Graphic Jump Table

8000       init         ;Graphics BASIC init (same as command=0)

8002       parse        ;Graphics BASIC command parser

8003       start        ;0      commands

8006       screendef    ;1

8008       screenopen   ;2

800A       screenclose  ;3

800C       screenclear  ;4

800E       screen       ;5

8010       setpen       ;6

8012       setpalette   ;7

8014       setdmode     ;8

8016       setdpat      ;9

8018       line         ;10

801A       box          ;11

801C       circle       ;12

801E       polygon      ;13

8020       ellipse      ;14

8022       viewpclr     ;15

8024       copy         ;16

8026       cut          ;17

8028       paste        ;18

802A       load         ;19

802C       char         ;20

802E       viewportdef  ;21

3.4.4. C64DX SOFT VECTORS

[*** THE FOLLOWING VECTORS AND JUMP TABLES ARE NOT FINAL ***]

BASIC indirect vectors

02F7   jmp USR          ;USR vector (must be set by application)

02FC   esc_fn_vec       ;Escape Function vector

02FE   graphic_vector   ;Graphic Kernel vector

0300   ierror           ;indirect error (output error in .x)

0302   imain            ;indirect main (system direct loop)

0304   icrnch           ;indirect crunch (tokenization routine)

0306   iqplop           ;indirect list (char list)

0308   igone            ;indirect gone (char dispatch)

030A   ieval            ;indirect eval (symbol evaluation)

030C   iesclk           ;escape token crunch

030E   iescpr           ;escape token list

0310   iescex           ;escape token execute

Kernel indirect vectors

02FA   iAutoScroll      ;AutoScroll used by BASIC, Monitor, Editor

0312   itime            ;(unused)

0314   iirq             ;IRQ

0316   ibrk             ;BRK

0318   inmi             ;NMI

031A   iopen

031C   iclose

031E   ichkin

0320   ickout

0322   iclrch

0324   ibasin

0326   ibsout

0328   istop

032A   igetin

032C   iclall

032E   exmon            ;Monitor command indirect

0330   iload

0332   isave

Editor indirect vectors to routines & tables

0334   ctlvec           ;'contrl' characters

0336   shfvec           ;'shiftd' characters

0338   escvec           ;'escape' characters

033A   keyvec           ;post keyscan, pre-evaluation of keys

033C   keychk           ;post-evaluation, pre-buffering of keys

033E   decode           ;vectors to 6 keyboard matrix decode tables

33E                   - Mode 1 --> normal keys

340                   - Mode 2 --> <SHIFT> keys

342                   - Mode 3 --> <C=> keys

344                   - Mode 4 --> <CONTROL> keys

346                   - Mode 5 --> <CAPS LOCK> keys

348                   - Mode 6 --> <ALT> keys

3.4.5. KERNEL DOCUMENTATION

The  KERNEL is the ROM resident operating system of the Commodore 64DX

computer.  All input,  output,  and memory management is controlled by

the KERNEL. The KERNEL JUMP TABLE provides a standardized interface to

many   useful   routines  within  the  operating  system.  Application

programmers  are  encouraged  to  utilize  the JUMP TABLEs to simplify

their operations and guarantee their functionality should hardware or,

software modifications to the system become necessary.

C64 STANDARD KERNEL CALLS

The  following  system  calls  comprise the set of standard C64 system

calls for the C64 class of machines, including the PLUS-4.  Several of

the  calls,  however  function  somewhat  differently  or  may require

slightly  different setups. This was necessary to accommodate specific

features of the system,  notably the 40/80 column windowing Editor and

banked  memory  facilities.  As  with  all  Kernel  calls,  the system

configuration (BANK $FF) must be in context at the time of the call.

C64DX KERNEL JUMP TABLE DESCRIPTIONS

1. $FF81 CINT  ; initialize screen editor

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

.Y used

Memory:     init Editor RAM

init Editor I/O

Flags:      none

Example:

SEI

JSR $FF81   ; initialize screen editor

CLI

CINT  is the Editor's initialization routine.  Editor indirect vectors

installed,  programmable key definitions assigned, and the ASC/DIN key

scanned for NATIONAL keyboard/charset determination. CINT sets the VIC

bank,  VIC nybble bank,  enables the character ROM, resets SID volume,

and clears the screen.  The only thing it does not do that pertains to

the Editor which is needed for IRQs  (keyscan, VIC cursor blink, split

screen modes), key lines, screen background colors, etc. (see IOINIT).

Because  CINT updates Editor indirect vectors that are used during IRQ

processing, you should disable IRQs prior to calling it. CINT utilizes

the status byte INIT STATUS as follows:

$1104 bit 6 = 0  -->  Full initialization.

(Set INIT_STATUS bit 6)

= 1  -->  Partial initialization.

(not keymatrix pointers)

(not program key definitions)

2. $FF84 IOINIT  ; init I/O devices

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

.Y used

Memory:     initialize I/O

Flags:      none

Example:

SEI

JSR $FF84   ; initialize system I/O

CLI

IOINIT  is  perhaps  the  major  function  of  the  Reset  handler. It

initializes  both CIA's (timers, keyboard serial port, user port), the

4510  port, the VIC chip, the UART and the DOS. It distinguishes a PAL

system from an NTSC one and sets PALCNT if PAL. The system IRQ source,

the VIC raster, is started (pending IRQs are cleared). IOINIT utilizes

the status byte INIT STATUS as follows:

$1104 bit 7 = 0  -->  Full initialization.

(set INIT STATUS bit 7)

= 1  -->  Partial initialization.

You  should  be  sure IRQs are disabled before calling IOINIT to avoid

interrupts while the various I/O devices are being initialized.

3. $FF87 RAMTAS  ; init RAM and buffers

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

.Y used

Memory:     initializes RAM

Flags:      none

Example:

JSR $FF87   ; initialize system RAM

RAMTAS  clears  all  base page RAM, allocates the sets pointers to the

top  and  bottom  of  system RAM and points the SYSTEM_VECTOR to BASIC

cold  start.  Lastly  it  sets  a  flag,  DEJAVU, to indicate to other

routines   that   system   RAM  has  been  initialized  and  that  the

SYSTEM_VECTOR  is  valid.  It  should  be  noted that the C64DX RAMTAS

routine does NOT in any way test RAM.

4. $FF8A RESTOR  ; init Kernel indirects

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

.Y used

Memory:     kernel indirects restored

Flags:      none

Example:

SEI

JSR $FF8A   ; restore kernel indirects

CLI

RESTOR  restores the default values of all the Kernel indirect vectors

from the Kernel ROM list.  It does NOT affect any other vectors,  such

as  those  used  by  the  Editor  (see CINT) and BASIC.  Because it is

possible for an interrupt (IRQ or NMI) to occur during the updating of

the interrupt indirect vectors, you should disable interrupts prior to

calling RESTOR. See also the VECTOR call.

5. $FF8D VECTOR  ; init or copy indirects

Preparation:

Registers:  .X = adr (low) of user list

.Y = adr (high) of user list

Memory:     system map

Flags:      .C = 0 --> load Kernel vectors

.C = 1 --> copy Kernel vectors

Calls:      none

Results:

Registers:  .A used

.Y used

Memory:     as per call

Flags:      none

Example:

LDX #save_lo

LDY #save_hi

SEC

JSR $FF87  ; copy indirects to 'save'

VECTOR reads or writes the Kernel RAM indirect vectors. Calling VECTOR

with  the carry status set stores the current contents of the indirect

vectors  to  the RAM address passed in the .X and .Y registers (to the

current RAM bank).  Calling VECTOR with the carry status clear updates

the Kernel indirect vectors from the user list passed in the .X and .Y

registers (from the current RAM bank). Interrupts (IRQ and NMI) should

be disabled when updating the indirects. See also the RESTOR call.

6. $FF90 SETMSG  ; kernel messages on/off

Preparation:

Registers:  .A = message control

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     MSGFLG updated

Flags:      none

Example:

LDA #0

JSR $FF90   ; turn OFF all Kernel messages

SETMSG  updates  the  Kernel message flag byte MSGFLG which determines

whether system error and/or control messages will be displayed.  BASIC

normally  disables error messages always and disables control messages

in 'run' mode. Note that the Kernel error messages are not the verbose

ones  printed by BASIC,  but simply the 'I/O ERROR #' message that you

see  when  in  the  Monitor,  for example.  Examples of Kernel control

messages are 'LOADING' and 'FOUND'. The MSGFLG control bits are:

MSGFLG bit 7 = 1 --> enable CONTROL messages

bit 6 = 1 --> enable ERROR messages

7. $FF93 SECND  ; serial: send SA after LISTN

Preparation:

Registers:  .A = SA (secondary address)

Memory:     system map

Flags:      none

Calls:      LISTN

Results:

Registers:  .A used

Memory:     STATUS ($90)

Flags:      none

Example:

LDA #8

JSR $FFB1   ; LISTN device 8

LDA #15

JSR $FF93   ; pass it SA #15

SECND  is  a low-level serial routine used to send a secondary address

(SA) to a LISTeNing device  (see LISTN Kernel call).  An SA is usually

used  to  provide setup information to a device before the actual data

I/O  operation  begins.  Attention  is released after a call to SECND.

SECND is not used to send an SA to a TALKing device (see TKSA).  (Most

applications  should  use  the higher level I/O routines: see OPEN and

CKOUT).

8. $FF96 TKSA  ; serial: send SA after TALK

Preparation:

Registers:  .A = SA (secondary address)

Memory:     system map

Flags:      none

Calls:      TALK

Results:

Registers:  .A used

Memory:     STATUS ($90)

Flags:      none

Example:

LDA #8

JSR $FFB4   ; TALK device 8

LDA #15

JSR $FF93   ; pass it SA #15

TKSA  is  a  low-level serial routine used to send a secondary address

(SA)  to a device commanded to TALK  (see TALK Kernel call).  An SA is

usually  used  to  provide  setup  information  to a device before the

actual  data  I/O operation begins.  (Most applications should use the

higher level I/O routines: see OPEN and CHKIN).

9. $FF99 MEMTOP  ; set/read top of system RAM

Preparation:

Registers:  .X = lsb of MEMSIZ

.Y = msb of MEMSIZ

Memory:     system map

Flags:      .C = 0 --> set top of memory

.C = 1 --> read top of memory

Calls:      none

Results:

Registers:  .X = lsb of MEMSIZ

.Y = msb of MEMSIZ

Memory:     MEMSIZ

Flags:      none

Example:

SEC

JSR $FF99   ; get top of user RAM

DEY

CLC

JSR $FF99   ; lower it 1 block

MEMTOP  is  used  to read or set the top of system RAM,  pointed to by

MEMSIZ.  This  call  is  included  in the C64DX for completeness,  but

neither  the  Kernel nor BASIC utilize MEMTOP as it has little meaning

in  the  banked  memory  environment  of the computer (even the RS-232

buffers  are  permanently  allocated).  None-the-less,  set  the carry

status  to  load  MEMSIZ  into .X  and .Y,  and clear it to update the

pointer  from .X and .Y.  Note that MEMSIZ references only system RAM.

The Kernel initially sets MEMSIZ to $FF00.

10. $FF9C MEMBOT  ; set/read bottom of system RAM

Preparation:

Registers:  .X = lsb of MEMSTR

.Y = msb of MEMSTR

Memory:     system map

Flags:      .C = 0 --> set bot of memory

.C = 1 --> read bot of memory

Calls:      none

Results:

Registers:  .X = lsb of MEMSTR

.Y = msb of MEMSTR

Memory:     MEMSTR

Flags:      none

Example:

SEC

JSR $FF9C   ; get bottom of user RAM_0

INY

CLC

JSR $FF9C   ; raise it 1 block

MEMBOT  is used to read or set the bottom of system RAM, pointed to by

MEMSTR.  This  call  is  included  in the C64DX for completeness,  but

neither  the  Kernel nor BASIC utilize MEMBOT as it has little meaning

in the banked memory environment of the C64DX. None-the-less,  set the

carry status to load MEMSTR into .X and .Y, and clear it to update the

pointer  from .X and .Y.  Note that MEMSTR references only system RAM.

The Kernel initially sets MEMSTR to $2000 (BASIC text starts here).

11. $FF9F KEY  ; scan keyboard

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     keyboard buffer

keyboard flags

Flags:      none

Example:

JSR $FF9F   ; scan the keyboard

KEY  is  an  Editor  routine  which  scans  the  entire  keyboard.  It

distinguishes  between shifted and unshifted keys,  control keys,  and

programmable  keys,  setting  keyboard  status  bytes and managing the

keyboard buffer. After decoding the key, KEY will manage such features

as toggling cases,  pauses or delays, and key repeats.  It is normally

called  by  the operating system during the 60Hz IRQ processing.  Upon

conclusion,  KEY  leaves the keyboard hardware driving the key-line on

which the STOP key is located.

There  are two indirect RAM jumps encountered during a keyscan: KEYVEC

($33A)  and  KEYCHK ($33C).  KEYVEC (alias KEYLOG) is taken whenever a

key depression is discovered,  before the key in  .A has been decoded.

KEYCHK is taken after the key has been decoded, just before putting it

into  the  key  buffer.  KEYCHK carries the ASCII character in .A, the

keycode in .Y, and the shift-key status in .X.

The keyboard decode matrices are addressed via indirect RAM vectors as

well, located at DECODE.

12. $FFA2 SETTMO  ; (reserved)

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     TIMOUT

Flags:      none

Example:

LDA #value

JSR $FFA2   ; update TIMOUT byte

SETTMO  is  unused  in the C64DX and is included for compatibility and

completeness.  It  is  used  in  the  C64  by  the  IEEE communication

cartridge to disable I/O timeouts.

13. $FFA5 ACPTR  ; serial: byte input.

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      TALK

TKSA (if necessary)

Results:

Registers:  .A = data byte

Memory:     STATUS ($90)

Flags:      none

Example:

JSR $FFA5   ; input a byte from serial bus

STA data

ACPTR  is  a low-level serial I/O utility to accept a single byte from

the  current serial bus TALKer using full handshaking.  To prepare for

this  routine  a  device  must first have been established as a TALKer

(see TALK) and passed a secondary address if necessary (see TKSA). The

byte is returned in .A. (Most applications should use the higher level

I/O routines: see BASIN and GETIN).

14. $FFA8 CIOUT  ; serial: byte output

Preparation:

Registers:  .A = data byte

Memory:     system map

Flags:      none

Calls:      LISTN

SECND (if necessary)

Results:

Registers:  .A used

Memory:     STATUS ($90)

Flags:      none

Example:

LDA data

JSR $FFA8   ; send a byte via serial bus

CIOUT  is  a low-level serial I/O utility to transmit a single byte to

the current serial bus LISTNer using full handshaking.  To prepare for

this  routine  a  device must first have been established as a LISTNer

(see LISTN)  and passed a secondary address if necessary  (see SECND).

The  byte  is  passed  in  .A.  Serial  output data is buffered by one

character,  with the last character being transmitted with EOI after a

call  to  UNLSN.  (Most  applications  should use the higher level I/O

routines; see BSOUT).

15. $FFAB UNTLK  ; serial: send untalk

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

Memory:     STATUS ($90)

Flags:      none

Example:

JSR $FFAB   ; UNTALK serial device

UNTLK  is  a  low-level Kernel serial bus routine that sends an UNTALK

command to all serial bus devices.  It commands all TALKing devices to

stop sending data.  (Most applications should use the higher level I/O

routines; see CLRCH).

16. $FFAE UNLSN  ; serial: send unlisten

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

Memory:     STATUS ($90)

Flags:      none

Example:

JSR $FFAE   ; UNLISTEN serial device

UNLSN  is a low-level Kernel serial bus routine that sends an UNLISTEN

command  to all serial bus devices.  It commands all LISTENing devices

to  stop reading data.  (Most applications should use the higher level

I/O routines; see CLRCH).

17. $FFB1 LISTN  ; serial: send listen command

Preparation:

Registers:  .A = device (0-31)

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

Memory:     STATUS ($90)

Flags:      none

Example:

JSR $FFB1   ; command device to LISTEN

LISTN  is  a  low-level Kernel serial bus routine that sends an LISTEN

command  to  the  serial  bus device in .A.  It commands the device to

start reading data. (Most applications should use the higher level I/O

routines; see CKOUT).

18. $FFB4 TALK  ; serial: send talk command

Preparation:

Registers:  .A = device (0-31)

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

Memory:     STATUS ($90)

Flags:      none

Example:

JSR $FFB4   ; command device to TALK

TALK  is  a  low-level  Kernel  serial  bus routine that sends an TALK

command  to  the  serial  bus device in .A.  It commands the device to

start sending data. (Most applications should use the higher level I/O

routines; see CHKIN).

19. $FFB7 READSS  ; read I/O status byte

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A = STATUS ($90 or $A6)

Memory:     STATUS cleared if RS-232 ($A6)

Flags:      none

Example:

JSR $FFB7   ; STATUS for last I/O

READSS (alias READST) returns the status byte associated with the last

I/O  operation  (serial  or RS-232)  performed.  Serial bus and newDOS

devices update STATUS ($90) and RS-232 I/O updates RSSTAT ($A6).  Note

that,  to  simulate  an  6551,  RSSTAT is cleared after it is read via

READSS.  The  last  I/O  operation is determined by the contents of FA

($BA), thus applications which drive I/O devices using the lower-level

Kernel calls should not use READSS.

20. $FFBA SETLFS  ; set channel LA, FA, SA

Preparation:

Registers:  .A = LA (logical #)

.X = FA (device #)

.Y = SA (secondary adr)

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     LA, FA, SA updated

Example:

see OPEN

SETLFS  sets  the  logical  file number (LA, $B8),  device number (FA,

$BA),  and secondary address (SA, $B9) for the higher-level Kernel I/O

routines.  The  LA  must  be  unique among OPENed files and is used to

identify specific files for I/O operations. The device number range is

0  to 31 and is used to target I/O.  The SA is a command to be sent to

the indicated device, usually to place it in a particular mode. If the

SA  is  not needed,  the .Y register should pass $FF.  SETLFS is often

used along with SETNAM and SETBNK calls prior to OPENs. See the Kernel

OPEN, LOAD, and SAVE calls for examples.

21. $FFBD SETNAM  ; set filename pointers

Preparation:

Registers:  .A = string length

.X = string adr_low

.Y = string adr_high

Memory:     system map

Flags:      none

Calls:      SETBNK

Results:

Registers:  none

Memory:     FNLEN, FNADR updated

Flags:      none

Example:

see OPEN

SETNAM  sets up the filename or command string for higher-level Kernel

I/O  calls  such as  OPEN,  LOAD,  and  SAVE.  The string (filename or

command)  length is passed in .A and updates FNLEN ($B7).  The address

of  the  string is passed in .X (low) and Y (high).  See the companion

call,  SETBNK  which specifies which RAM bank the string is found.  If

there  is  no  string,  SETNAM should still be called and a null ($00)

length specified  (the address does not matter).  SETNAM is often used

along  with  SETBNK  and  SETLFS calls prior to OPENs.  See the Kernel

OPEN, LOAD, and SAVE calls for examples.

22. $FFC0 OPEN  ; open logical file

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      SETLFS, SETNAM, SETBNK

Results:

Registers:  .A = error code (if any)

.X used

.Y used

Memory:     setup for I/O

STATUS, RSSTAT updated

Flags:      .C = 1 --> error

Example:          OPEN 1,8,15,"I0"

LDA #length    ; fnlen

LDX #<filename ; fnadr (command)

LDY #>filename

JSR $FFBD      ; SETNAM

LDX #0         ; fnbank (RAM_0)

JSR $FF68      ; SETBNK

LDA #1         ; la

LDX #8         ; fa

LDY #15        ; sa

JSR $FFBA      ; SETLFS

JSR $FFC0      ; OPEN

BCS error

filename .BYTE 'I0'

length   = 2

OPEN  prepares  a logical file for I/O operations. It creates a unique

entry  in the Kernel logical file tables LAT ($362),  FAT ($36C),  and

SAT ($376)  using its index LDTND ($98)  and data supplied by the user

via   SETLFS.   There   can   be   up  to  ten  logical  files  OPENed

simultaneously.  OPEN  performs  device  specific  opening  tasks  for

serial,  RS-232,  keyboard & screen,  devices,  including clearing the

previous  status and transmitting any given filename or command string

supplied  by  the  user via SETNAM and SETBNK.  The I/O status will be

updated appropriately and can be read via READSS.

The  path  to  OPEN  is  through  an  indirect  RAM  vector  at  $31A.

Applications  may  therefore  provide  their  own  OPEN  procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

23. $FFC3 CLOSE  ; close logical file

Preparation:

Registers:  .A = LA (logical #)

Memory:     system map

Flags:      .C (see text below)

Calls:      none

Results:

Registers:  .A = error code (if any)

.X used

.Y used

Memory:     logical tables updated

STATUS, RSSTAT updated

Flags:      .C = 1 --> error

Example:

LDA #1      ; la

JSR $FFC3   ; CLOSE

BCS error

CLOSE removes the logical file (LA) passed in .A from the logical file

tables  and  performs device specific closing tasks. Keyboard, screen,

and  any  unOPENed  files  pass through. RS-232 devices are not closed

until  all buffered data has been transmitted. Serial files are closed

by  transmitting  a  'close'  command  (if an SA was given when it was

opened), sending any, buffered character, and UNLiSTeNing the bus.

There  is  a  special provision incorporated into the CLOSE routine of

systems  featuring BASIC DOS command.  If the following conditions are

all  TRUE,  a full CLOSE is NOT performed:  the table entry is removed

but  a  'close' command is NOT transmitted to the device.  This allows

the  disk command channel to be properly OPENed and CLOSEd without the

disk operating system closing ALL files on its end:

.C = 1 --> indicates special CLOSE

FA >=8 --> device is a disk

SA =15 --> command channel

The  path  to  CLOSE  is  through  an  indirect  RAM  vector  at $31C.

Applications  may  therefore  provide  their  own  CLOSE procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

24. $FFC6 CHKIN  ; set input channel

Preparation:

Registers:  .X = LA (logical #)

Memory:     system map

Flags:      none

Calls:      OPEN

Results:

Registers:  .A = error code (if any)

.X used

.Y used

Memory:     LA, FA, SA, DFLTN

STATUS, RSSTAT updated

Flags:      .C = 1 --> error

Example:

LDX #1      ; la

JSR $FFC6   ; CHKIN

BCS error

CHKIN  establishes  an input channel to the device associated with the

logical address (LA) passed in  .X, in preparation for a call to BASIN

or  GETIN.  The Kernel variable DFLTN ($99) is updated to indicate the

current input device and the variables LA, FA, and SA are updated with

the  file's  parameters from its entry in the logical file tables (put

there  by OPEN).  CHKIN performs certain device specific tasks: screen

and  keyboard  channels  pass  through, and serial channels are sent a

TALK  command  and  the  SA transmitted (if necessary).  Call CLRCH to

restore normal I/O channels.

CHKIN is required for all input except the keyboard. If keyboard input

is desired and no other input channel is established,  you do not need

to  call  CHKIN or OPEN.  The keyboard is the default input device for

BASIN and GETIN.

The  path  to  CHKIN  is  through  an  indirect  RAM  vector  at $31E.

Applications  may  therefore  provide  their  own  CHKIN procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

25. $FFC9 CKOUT  ; set output channel

Preparation:

Registers:  .X = LA (logical #)

Memory:     system map

Flags:      none

Calls:      OPEN

Results:

Registers:  .A = error code (if any)

.X used

.Y used

Memory:     LA, FA, SA, DFLTO

STATUS, RSSTAT updated

Flags:      .C = 1 --> error

Example:

LDX #1      ; la

JSR $FFC9   ; CKOUT

BCS error

CKOUT  establishes an output channel to the device associated with the

logical address (LA) passed in .X, in preparation for a call to BSOUT.

The  Kernel  variable  DFLTO  ($9A) is updated to indicate the current

output device and the variables  LA,  FA,  and SA are updated with the

file's parameters from its entry in the logical file tables (put there

by  OPEN).  CKOUT performs  certain  device  specific tasks:  keyboard

channels  are  illegal,  screen  channels  pass  through,  and  serial

channels  are  sent  a  LISTN  command  and  the  SA  transmitted  (if

necessary). Call CLRCH to restore normal I/O channels.

CKOUT  is  required for all output except the screen. If screen output

is desired and no other output channel is established, you do not need

to  call  CKOUT  or OPEN.  The screen is the default output device for

BSOUT.

The  path  to  CKOUT  is  through  an  indirect  RAM  vector  at $320.

Applications  may  therefore  provide  their  own  CKOUT procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

26. $FFCC CLRCH  ; restore default channels

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

Memory:     DFLTI, DFLTO updated

Flags:      none

Example:

JSR $FFCC   ; restore default I/O

CLRCH  (alias  CLRCHN)  is used to clear all open channels and restore

the  system  default  I/O  channels  after  other  channels  have been

established via CHKIN and/or CHKOUT. The keyboard is the default input

device  and  the  screen  is  the default output device.  If the input

channel  was to a serial device,  CLRCH first UNTLKs it. If the output

channel was to a serial device, it is UNLiSteNed first.

The  path  to  CLRCH  is  through  an  indirect  RAM  vector  at $322.

Applications  may  therefore  provide  their  own  CLRCH procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

27. $FFCF BASIN  ; input from channel

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      CHKIN (if necessary)

Results:

Registers:  .A = character (or error code)

Memory:     STATUS, RSSTAT updated

Flags:      .C = 1 if error

Example:

LDY #0      ; index

more JSR $FFCF   ; input a character

STA data,Y  ; buffer it

INY

CMP #$0D    ; carriage return?

BNE more

BASIN  (alias  CHRIN)  reads a character from the current input device

(DFLTN, $99)  and returns it in .A.  Input from devices other than the

keyboard  (the default input device)  must be OPENed and CHKINed.  The

character  is  read  from the input buffer associated with the current

input channel:

1. RS-232 data is returned a character at a time from the RS-232 input

buffer,  waiting  until  a  character is received if necessary.  If

RSSTAT  is  bad  from a prior operation,  input is skipped and null

input (carriage return) is substituted.

2. Serial  data  is  returned  a character at a time directly from the

serial  bus,  waiting  until  a character is sent if necessary.  If

STATUS ($90)  is  bad from a prior operation,  input is skipped and

null input (carriage return) is substituted.

3. Screen  data is read from screen RAM starting at the current cursor

position  and ending with a faked carriage return at the end of the

logical screen line.

4. Keyboard  data is input by turning on the cursor reading characters

from  the  keyboard  buffer  and echoing them on the screen until a

carriage return is encountered. Characters are then returned one at

a time from the screen until all characters input have been passed,

including  the carriage return.  Any calls after the eol will start

the process over again.

The  path  to  BASIN  is  through  an  indirect  RAM  vector  at $324.

Applications  may  therefore  provide  their  own  BASIN procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

28. $FFD2 BSOUT  ; output to channel

Preparation:

Registers:  .A = character

Memory:     system map

Flags:      none

Calls:      CKOUT (if necessary)

Results:

Registers:  .A = error code (if any)

Memory:     STATUS, RSSTAT updated

Flags:      .C = 1 if error

Example:

LDA #character

JSR $FFD2   ; output a character

BSOUT  (alias CHROUT) writes the character in .A to the current output

device  (DFLTO,  $9A).  Output  to  devices other than the screen (the

default  output device)  must be OPENed and CKOUTed.  The character is

written  to  the  output  buffer  associated  with  the current output

channel:

1. RS-232  data  is  put  a character at a time into the RS-232 output

buffer, waiting until there is room if necessary.

2. Serial  data  is  passed  to  CIOUT which buffers one character and

sends the previous character.

3. Screen data is put into screen RAM at the current cursor position.

4. Keyboard output is illegal.

The  path  to  BSOUT  is  through  an  indirect  RAM  vector  at $326.

Applications  may  therefore  provide  their  own  BSOUT procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

29. $FFD5 LOAD  ; load from file

Preparation:

Registers:  .A = 0 --> LOAD

.A > 0 --> VERIFY

.X = load adr_lo (if SA=0)

.Y = load adr_hi (if SA=0)

Memory:     system map

Flags:      none

Calls:      SETLFS, SETNAM, SETBNK

Results:

Registers:  .A = error code (if any)

.X = ending adr_lo

.Y = ending adr_hi

Memory:     per command

STATUS updated

Flags:      .C = 1 --> error

Example:          LOAD "program",8,1

LDA #length    ; fnlen

LDX #<filename ; fnadr

LDY #>filename

JSR $FFBD      ; SETNAM

LDA #0         ; load/verify bank (RAM_0)

LDX #0         ; fnbank (RAM_0)

JSR $FF68      ; SETBNK

LDA #0         ; la (not used)

LDX #8         ; fa

LDY #$FF       ; sa (SA>0 normal load)

JSR $FFBA      ; SETLFS

LDA #0         ; load, not verify

LDX #<load_adr ; (used only if SA=0)

LDY #>load_adr ; (used only if SA=0)

JSR $FFD5      ; LOAD

BCS error

STX end_lo

STY end_hi

filename .BYTE 'program'

length   = 7

This  routine LOADs data from an input device into memory. It can also

be  used  to  VERIFY that data in memory matches that in a file.  LOAD

performs device specific tasks for serial LOADs.  You cannot LOAD from

RS-232 devices,  the screen, or the keyboard.  While LOAD performs all

the  tasks of an OPEN, it does NOT create any logical files as an OPEN

does. Also note that LOAD cannot 'wrap' memory banks. As with any I/O,

the  I/O  status  is updated appropriately and can be read via READSS.

LOAD has two options that the user must select:

1. LOAD  vs.  VERIFY:  the  contents  of .A passed at the call to LOAD

determines which mode is in effect. If .A is zero, a LOAD operation

will  be  performed  and  memory  will  be  overwritten.  If  .A is

non-zero,  a  VERIFY  operation  will  be  performed and the result

passed via the error mechanism.

2. LOAD  ADDRESS:  the  secondary  address  (SA)  setup by the call to

SETLFS determines  where  the LOAD starting address comes from.  If

the SA is zero, the user wants the address in .X and .Y at the time

of  the call to be used.  If the SA is non-zero,  the LOAD starting

address  is read from the file header itself and the file loaded to

the same place from which it was SAVEd.

The  serial  LOAD  routine  automatically  attempts to access a newDOS

drive,  then  attempts to BURST load a file, and resorts to the normal

load mechanism (but still using the FAST serial routines) if the BURST

handshake is not returned.

The  path  to  LOAD  is  through  an  indirect  RAM  vector  at  $330.

Applications  may  therefore  provide  their  own  LOAD  procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

30. $FFD8 SAVE  ; save to file

Preparation:

Registers:  .A = pointer to start adr

.X = end_adr_lo

.Y = end_adr_hi

Memory:     system map

Flags:      none

Calls:      SETLFS, SETNAM, SETBNK

Results:

Registers:  .A = error code (if any)

.X = used

.Y = used

Memory:     STATUS updated

Flags:      .C = 1 --> error

Example:          SAVE "program",8

LDA #length     ; fnlen

LDX #<filename  ; fnadr

LDY #>filename

JSR $FFBD       ; SETNAM

LDA #0          ; save from bank (RAM_0)

LDX #0          ; fnbank (RAM_0)

JSR $FF68       ; SETBNK

LDA #0          ; la (not used)

LDX #8          ; fa

LDY #0          ; sa (cassette only)

JSR $FFBA       ; SETLFS

LDA #start      ; pointer to start address

LDX end         ; ending address lo

LDY end+1       ; ending address hi

JSR $FFD8       ; SAVE

BCS error

filename .BYTE 'program'

length   = 7

start    .WORD address1  ; page_0

end      .WORD address2

This routine SAVEs data from memory to an output device. SAVE performs

device  specific  tasks  for serial SAVEs. You cannot SAVE from RS-232

devices,  the  screen,  or  the keyboard.  While SAVE performs all the

tasks  of  an  OPEN,  it  does NOT create any logical files as an OPEN

does. The starting address of the area to be SAVEd must be placed in a

base-page  vector  and the address of this vector passed to SAVE in .A

at the time of the call. The address of the last byte to be SAVEd PLUS

ONE is passed in .X and .Y at the same time.

SAVE  first attempts to access a newDOS drive. There is no BURST save:

the  normal  FAST  serial  routines are used. As with any I/O, the I/O

status will be updated appropriately and can be read via READSS.

The  path  to  SAVE  is  through  an  indirect  RAM  vector  at  $332.

Applications  may  therefore  provide  their  own  SAVE  procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

31. $FFDB SETTIM  ; set internal clock

Preparation:

Registers:  .A = hours

.X = minutes

.Y = seconds

.Z = tenths

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     TOD at CIA $DC00 updated

Flags:      none

Example:

LDA #0      ; reset clock

TAX

TAY

TAZ

JSR $FFDB   ; SETTIM

SETTIM sets the system CIA 24-hour TOD clock, which counts tenths of a

second and automatically wraps at the 24-hour point.

32. $FFDE RDTIM  ; read internal clock

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A = hours

.X = minutes

.Y = seconds

.Z = tenths

Memory:     none

Flags:      none

Example:

JSR $FFDE   ; RDTIM

RDTIM reads the system CIA 24-hour TOD clock, which counts tenths of a

second. The timer is automatically wrapped at the 24-hour point.

33. $FFE1 STOP  ; scan stop key

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A = last keyboard row

.X = used (if STOP key)

Memory:     none

Flags:      status valid

Example:

JSR $FFE1   ; scan STOP key

BEQ stop    ; branch if down

STOP  checks a Kernel variable STKEY ($91),  which is updated by UDTIM

during  normal  IRQ  processing and contains the last scan of keyboard

column  C7.  The  STOP key is bit-7,  which will be zero if the key is

down.  If  it is,  default I/O channels are restored via CLRCH and the

keyboard queue is flushed by resetting NDX ($D0). The keys on keyboard

line C7 are:

bit:     7    6    5    4    3    2    1    0

key:    STOP  Q   C=  SPACE  2  CTRL  <--   1

The  path  to  STOP  is  through  an  indirect  RAM  vector  at  $328.

Applications  may  therefore  provide  their  own  STOP  procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

34. $FFE4 GETIN  ; read buffered data

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      CHKIN (if necessary)

Results:

Registers:  .A = character (or error code)

.X used

.Y used

Memory:     STATUS, RSSTAT updated

Flags:      .C = 1 if error

Example:

wait JSR $FFE4   ; get any key

BEQ wait

STA character

GETIN  reads  a  character  from  the current input device (DFLTN $99)

buffer  and  returns  it  in  .A.  Input  from  devices other than the

keyboard  (the  default input device) must be OPENed and CHKINed.  The

character  is  read  from the input buffer associated with the current

input channel:

1. Keyboard input: a character is removed from the keyboard buffer and

passed in .A. If the buffer is empty, a null ($00) is returned.

2. RS-232  input:  a character is removed from the RS-232 input buffer

and passed in .A.  If the buffer is empty, a null ($00) is returned

(use READSS to check validity).

3. Serial input:  GETIN automatically jumps to BASIN. See BASIN serial

I/O.

4. Screen input:  GETIN automatically jumps to BASIN. See BASIN serial

I/O.

The  path  to  GETIN  is  through  an  indirect  RAM  vector  at $32A.

Applications  may  therefore  provide  their  own  GETIN procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

35. $FFE7 CLALL  ; close all files and channels

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

Memory:     LDTND, DFLTN, DFLTO updated

Flags:      none

Example:

JSR $FFE7   ; close files

CLALL  deletes  all  logical file table entries by resetting the table

index,  LDTND  ($98).  It clears current serials channels (if any) and

restores the default I/O channels via CLRCH.

The  path  to  CLALL  is  through  an  indirect  RAM  vector  at $32C.

Applications  may  therefore  provide  their  own  CLALL procedures or

supplement  the  system's  by  re-directing  this  vector to their own

routine.

36. $FFEA ScanStopKey (was UDTIM, which has no purpose on C64DX)

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

Memory:     TIME, TIMER, STKEY updated

Flags:      none

Example:

JSR $FFEA   ; ScanStopKey

Scans  key  line C7, on which the STOP key lies, and stores the result

in STKEY ($91). The Kernel routine STOP utilizes this variable.

37. $FFED SCRORG  ; get current screen window size

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A = screen width

.X = window width

.Y = window height

Memory:     none

Flags:      none

Example:

JSR $FFED   ; SCRORG

SCRORG returns active window's size (maximum row & column #) & origin.

entry: nothing required.

exit:  .C = max. screen width (0=80, 1=40)       default = 0

.X = max. column number (# columns - 1)   default = 79

.Y = max. line number (# lines minus 1)   default = 24

.A = window address (home position), low  default = $0800

.Z = window address, high

38. $FFF0 PLOT  ; read/set cursor position

Preparation:

Registers:  .X = cursor line

.Y = cursor column

Memory:     system map

Flags:      .C = 0 --> set cursor position

.C = 1 --> get cursor position

Calls:      none

Results:

Registers:  .X = cursor line

.Y = cursor column

Memory:     TBLX, PNTR updated

Flags:      .C = 1 --> error

PLOT Reads or sets the cursor position within current window.

Entry: .C = 1  Returns  the  cursor   position   (.Y=column,  .X=line)

relative  to  the  current  window  origin  (NOT screen

origin).

.C = 0  Sets   the   cursor   position   (.Y=column,   .X=line)

relative  to  the  current  window  origin  (NOT screen

origin).

Exit:  When  reading  position,  .X=line,  .Y=column,  .C=1 if wrapped

line.

When setting new position, .X=line, .Y=column, and

.C = 0  Normal exit.  The cursor has been moved to the position

contained  in  .X  &  .Y relative to window origin (see

SCRORG).

.C = 1  Error  exit.  The  requested  position  was outside the

current  window  (see SCRORG).  The cursor has not been

moved.

When called with the carry status set, PLOT returns the current cursor

position  relative  to the current window origin  (NOT screen origin).

When  called  with  the  carry status clear,  PLOT attempt to move the

cursor to the indicated line and column relative to the current window

origin  (NOT screen origin).  PLOT will return a clear carry status if

the cursor was moved, and a set carry status if the requested position

was outside the current window (NO CHANGE has been made).

Editor variables that are useful:

SCBOT - $E4 --> window bottom

SCTOP - $E5 --> window top

SCLF  - $E6 --> window left side

SCRT  - $E7 --> window right side

TBLX  - $EC --> cursor line

PNTR  - $ED --> cursor column

LINES - $EE --> maximum screen height

COLUMNS $EF --> maximum screen width

39. $FFF3 IOBASE  ; read base address of I/O block

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .X = lsb of I/O block

.Y = msb of I/O block

Memory:     none

Flags:      none

Example:

JSR $FFF3   ; find the I/O block

IOBASE  is  unused  in the C64DX and is included for compatibility and

completeness. It returns the address of the I/O block in .X and .Y.

NEW C64DX KERNEL CALLS

The  following  system  calls  comprise  a  set  of  extensions to the

standard  CBM  jump table. They are specifically for the C64DX machine

and  as  such  should  not be considered as permanent additions to the

standard  jump  table. With the exception of C64MODE they are all true

subroutines and will terminate via RTSs. As with all Kernel calls, the

system  configuration (BANK $FF) must be in context at the time of the

call.

1. $FF4D SPIN_SPOUT  ;setup fast serial ports for I/O

Preparation:

Registers:  none

Memory:     system map

Flags:      .C = 0  -->  select SPINP

.C = 1  -->  select SPOUT

Calls:      none

Results:

Registers:  .A used

Memory:     CIA_1, FSDIR register

Flags:      none

Example:

CLC

JSR $FF4D   ;setup for fast serial input

The  fast serial protocol utilizes CIA_1 (6526 at $DC00) and a special

driver  circuit  controlled  in  part by the FSDIR register. SPINP and

SPOUT  are  routines  used  by  the  system to set up the CIA and fast

serial  driver  circuit  for input or output. SPINP sets up CRA (CIA_1

register  14)  and  clears the FSDIR bit for input. SPOUT sets up CRA,

ICR (CIA_1 register 13), timer A (CIA_l registers 4 & 5), and sets the

FSDIR  bit  for  output.  Note  the  state of the TOD_IN bit of CRA is

always  preserved.  These  routines  are required only by applications

driving the fast serial bus themselves from the lowest level.

2. $FF50 CLOSE_ALL  ;close all files on a device

Preparation:

Registers:  .A --> device # (FA: 0-31)

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

.Y used

Memory:     none

Flags:      none

Example:

LDA #$08

JSR $FF50   ; close all files on device 8

The  FAT  is  searched  for  the  given  FA.  A  proper CLOSE is

performed  for all matches. If one of the CLOSEd channels is the

current I/O channel then the default channel is restored.

This call is utilized, for example, by the BASIC command DCLOSE.

3. $FF53 C64MODE  ;reconfigure system as a C64

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     none

Flags:      none

Example:

JMP $FF53   ;switch to C64 mode

THERE IS NO RETURN FROM THIS ROUTINE. The system downloads code to RAM

which  reMAPs  the  system  to  put the C64 ROM in context, resets all

VIC-III modes, and jumps to the C64 start routine.

Return  to  C65  mode  is by resetting the machine, although a program

could  do  it  very  easily.  A  vector on the C64 side is provided to

restart C64DX mode.

4. $FF56 MonitorCall  ;enter Monitor mode

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     none

Flags:      none

Turns  off  BASIC receipt of IRQ, maps BASIC out, maps the Monitor in,

and calls it.

When  the  Monitor is exited, the system is restored, BASIC mapped in,

and  the  system_vector  taken  (usually  points  to  BASIC warm start

entry).

5. $FF59 BOOT_SYS  ;boot an alternate OS from disk

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  undefined

Memory:     undefined

Flags:      undefined

Boot  an  alternate  system.  Reads  the "home" sector of any diskette

(physical track 0  sector 1,  512 bytes) into memory at $00400,  turns

off  BASIC,  and JMPs to it. Nothing done if disk not present. JMP not

made if first byte is not $4C.

It forces the "system" memory map, not user environment.

No  support for C128-style BOOT sector. Not related to BASIC 10.0 BOOT

command, which RUNs a BASIC program called "AUTOBOOT.C65*" if found.

6. $FF5C PHOENIX  ;???? C64DX diagnostics ????

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  undefined

Memory:     undefined

Flags:      none

Example:

JSR $FF5C   ;PHOENIX

Not same thing as C128 Phoenix routine. In the C65 development system,

this  routine  is  called  after  BASIC inits and performs some system

diagnostics, displaying results on the screen.

7. $FF5F LKUPLA  ;search tables for given la

8. $FF62 LKUPSA  ;search tables for given sa

Preparation:

Registers:  .A = LA (logical file number)

if LKUPLA

.Y = SA (secondary address)

if LKUPSA

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A = LA (only if found)

.X = FA (only if found)

.Y = SA (only if found)

Memory:     none

Flags:      .C = 0 if found

.C = 1 if not found

Example:

LDY #$60     ;find an available SA

again INY

CPY #$6F

BCS too_many ;too many files open

JSR $FF62    ;LKUPSA

BCC again    ;get another if in use

LKUPLA  and  LKUPSA  are  Kernel  routines used primarily by BASIC DOS

commands  to  work  around  a  user's  open  disk channels. The Kernel

requires  unique  logical  device  numbers (LAs) and the disk requires

unique   secondary   addresses   (SAs),   therefore  BASIC  must  find

alternative  unused  values  whenever  it  needs  to  establish a disk

channel.

9. $FF65 SWAPPER  ;switch between 40 & 80 column modes

Preparation:

Registers:  none

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

.Y used

Memory:     screen cleared

Flags:      none

Example:

LDA $D7     ;check display mode

BMI if_80   ;branch if 80 column

JSR $FF5F   ;switch from 40 to 80

MODE,  location  $D7,  is  toggled  by SWAPPER to indicate the current

display  mode: $80 = 80-column, $00 = 40-column. Because they are both

VIC  screens,  changing  them requires clearing the screens since they

share the same memory location.

10. $FF68 PFKEY  ;program a function key

Preparation:

Registers:  .A = pointer to string adr

(lo/hi/bank)

.Y = string length

.X = key number (1-16)

Memory:     system map

Flags:      none

Calls:      none

Results:

Registers:  .A used

.X used

.Y used

Memory:     PKYBUF, PKYDEF tables updated

Flags:      .C = 0 if successful

.C = 1 if no room

Example:

LDA #$FA    ;pointer to string address

LDY #6      ;length

LDX #15     ;key # ('HELP' key)

JSR $FF68   ;install new key def'n

BCS no_room

>000FA 00 13 00           ;ptr to $1300 bank 0

>01300 53 54 52 49 4E 47  ;'string'

PFKEY  (alias  KEYSET)  is an Editor utility to replace a function key

string with a user's string. Keys 1-14 are F1-F14, 15 is the HELP key,

and  16  is  the  <shift>RUN  string.  The  example above replaces the

'help<cr>'  string  assigned  at system initialization to the HELP key

with   the   string  'string'.  Both  the  key  length  table,  PKYBUF

($1000-$100F),  and  the  definition  area,  PKYDEF  ($1010-$10FF) are

compressed  and  updated.  The maximum length of all 16 strings is 240

characters.  No change is made if there is insufficient room for a new

definition.

11. $FF6B SETBNK  ;set bank for I/O operations

;and filename

Preparation:

Registers:  .A = BA, memory bank (0-FF)

.X = FNBANK, filename bank

Memory:     system map

Flags:      none

Calls:      SETNAM

Results:

Registers:  none

Memory:     BA, FNBANK updated

Flags:      none

Example:

see OPEN

SETBNK  is  a  prerequisite  for any memory I/O operations and must be

used  along  with  SETLFS  and  SETNAM prior to OPENing files, etc. BA

($C6)  sets  the  current  64KB  memory  bank  for  LOAD/SAVE/  VERIFY

operations.  FNBANK  ($C7)  indicates  the  bank in which the filename

string  is  found.  The Kernel routine SETBNK is often used along with

SETNAM and SETLFS calls prior to OPENs. See the Kernel OPEN, LOAD, and

SAVE calls for examples.

12. $FF6E JSRFAR  ;gosub in another bank

13. $FF71 JMPFAR  ;goto another bank

Preparation:

Registers:  none

Memory:     system map, also:

$02 --> bank (0-FF)

$03 --> PC_high

$04 --> PC_low

$05 --> .S (status)

$06 --> .A

$07 --> .X

$08 --> .Y

$09 --> .Z

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     as per call, also:

$05 --> .S (status)

$06 --> .A

$07 --> .X

$08 --> .Y

$09 --> .Z

Flags:      none

The  two  routines,  JSRFAR  and  JMPFAR, enable code executing in the

system bank of memory to call (or JMP to) a routine in any other bank.

In  the case of JSRFAR, the called routine must restore the system map

before executing a return.

JSRFAR  calls  JMPFAR. Both are RAM routines, located at $39C and $3B1

respectively.

The  user  should take necessary precautions when calling a non-system

bank  that  interrupts  (IRQs  &  NMIs)  will  be handled properly (or

disabled beforehand).

14. $FF74 LDA_FAR  ;LDA (.X),Y from bank .Z

Preparation:

Registers:  .X = pointer to base page pointer

.Y = index

.Z = bank (0-FF)

Memory:     setup indirect vector

Flags:      none

Calls:      none

Results:

Registers:  .A = data

Memory:     DMA_LIST updated

Flags:      status valid

LDA_FAR  enables  applications  to  read  data from any other bank. It

builds  a  DMA_LIST to fetch one byte, executes the DMA, and reads the

byte. It's a ROM routine.

15. $FF77 STA_FAR  ;STA (.X),Y from bank .Z

Preparation:

Registers:  .A = data

.X = pointer to base page pointer

.Y = index

.Z = bank (0-FF)

Memory:     setup indirect vector

Flags:      none

Calls:      none

Results:

Registers:  .X used

Memory:     DMA_LIST updated

Flags:      status invalid

STA_FAR  enables  applications  to  write  data  to any other bank. It

builds  a DMA_LIST to stash one byte, and executes the DMA. It's a ROM

routine.

16. $FF7A CMP_FAR  ;CMP (.X),Y from bank .Z

Preparation:

Registers:  .A = data

.X = pointer to a base page pointer

.Y = index

.Z = bank (0-FF)

Memory:     setup indirect vector

Flags:      none

Calls:      none

Results:

Registers:  .X used

Memory:     none

Flags:      status valid

CMP_FAR  enables  applications  to  compare data to any other bank. It

builds  calls  LDA_FAR  and  compares  the  given  byte  with the byte

fetched. It's a ROM routine.

17. $FF7D PRIMM  ;print immediate utility

Preparation:

Registers:  none

Memory:     none

Flags:      none

Calls:      none

Results:

Registers:  none

Memory:     none

Flags:      none

Example:

JSR $FF7D       ;display following text

.BYTE 'message'

.BYTE $00       ;terminator

JMP continue    ;execution resumes here

PRIMM is a Kernel utility used to print (to the default output device)

a  PETSCII string which immediately follows the call.  The string must

be  no  longer  than  255 characters and be terminated by a null ($00)

character.  It  cannot  contain  any embedded null characters. Because

PRIMM  uses  the system stack to find the string and a return address,

you MUST NOT JMP to PRIMM. There must be a valid address on the stack.

3.4.6. BASIC 10.0 MATH PACKAGE

This document details the many user-callable routines available in the

C64DX BASIC 10.0 math package.

FLOATING POINT MATH PACKAGE CONVENTIONS

In BASIC memory the number is PACKED and looks like this:

+--------+---------+--------+--------+-----+

| signed | B7=SGN  |        |        |     |

|   EXP  +---------+ M A N T I S S A | LSB |

|  +$80  |   MSB   |        |        |     |

+--------+---------+--------+--------+-----+

Because  the  mantissa  is  normalized  such that its msb is always 1,

BASIC  stores  the  SIGN  of  the  mantissa  here  to  save  a byte of

storage.  It  must be normalized when put in the FACC (see CONUPK). In

the FACC the NORMALIZED number looks like this:

$63       $64      $65      $66     $67    $68

FACEXP    FACHO    FACMOH   FACMO   FACLO  FACSGN

+--------+---------+--------+--------+-----+-------+

| signed | BIT 7=1 |        |        |     | SIGN  |

|   EXP  +---------+ M A N T I S S A | LSB |+ = $00|

| +$80   |   MSB   |        |        |     |- = $00|

+--------+---------+--------+--------+-----+-------+

Negative exponents are not stored 2's complement. The maximum exponent

is  10^38  ($FF) and the minimum is 10^-39 ($01). A zero value for the

exponent means the number is zero. Since the exponent is a power of 2,

it  can  be  described  as  the  number  of  left  (EXP>$80)  or right

(EXP<=$80) shifts to be performed on the normalized mantissa to create

the  binary  representation  of  the value. There is a second floating

accumulator called ARG which has the same layout. It is located at $6A

through $6F. Throughout the math package the floating point format is:

* the mantissa is 24 bits long.

* the binary point is to the left of the msb.

* the mantissa is always positive, and its msb is always 1.

* number = mantissa * 2^exponent, sign in FACSGN.

* the sign of the exponent is the msb of the exponent.

* the exponent is stored in excess $80 (i.e., it is a signed

8-bit number with $80 added to it.)

* an exponent of zero means the number is zero. (Note that

the rest of the accumulator cannot be assumed to be zero.)

* to keep the same number in the accumulator while shifting:

right shifts --> increment exponent

left shifts  --> decrement exponent

Arithmetic routine calling conventions:

* For one argument functions:

the argument is in the FACC.

the result is left in the FACC.

* For two argument operations:

the first argument is in MEMORY (packed) or ARG (unpacked).

the second argument is in the FACC.

the result is left in the FACC.

* Always call ROM routines with SYSTEM memory in context (BANK $FF).

A  note concerning precision. Since the mantissa is always normalized,

the  high  order  bit of the most significant byte is always one. This

guarantees  at  least  40  bits (5 byte mantissa times 8 bits each) of

precision, which is approximately 9 significant digits plus a few bits

for  rounding. In fact, there is a 'rounding' byte, FACOV ($71), which

should,  for  the greatest degree of precision, be loaded whenever you

load  the FACC. The high order bit of FACOV is utilized in most of the

math  routines.  While  some  of  the  'movement' routines 'round' the

loaded  floating  point  number  (i.e.,  FACOV = $00), others (such as

CONUPK)  do  not - assuming the value of FACOV is the useful result of

an operation in progress. In 99% of the cases you need not worry about

it,  as  its significance is virtually nil. For the greatest degree of

precision however, use it.

A few examples of normalized (FACC) floating point numbers:

VALUE          EXP          M A N T I S S A           SIGN

-------        -----    --------------------------     ----

1E38   =       FF      96      76      99      53      00

4E10   =       A4      95      02      F9      00      00

2E10   =       A3      95      02      F9      00      00

1E10   =       A4      95      02      F9      00      00

10   =       84      A0      00      00      00      00

1   =       81      80      00      00      00      00

.5   =       80      80      00      00      00      00

.25   =       7F      80      00      00      00      00

.6   =       80      99      99      99      9A      00

1E-04   =       73      D1      B7      59      59      00

1E-37   =       06      88      1C      EA      15      00

1E-38   =       02      D9      C7      DC      EE      00

3E-39   =       01      82      AB      1E      2A      00

0   =       00      xx      xx      xx      xx      00

-1   =       81      80      00      00      00      FF

-5   =       83      A0      00      00      00      FF

Now for a simple example of deriving the actual binary from the FACC:

5   =       83      A0      00      00      00      00

|        \

|         \

($83-$80)   ($A0)

|

which means:   2^3   *  .10110000, or shift mantissa LEFT 3,

which gives:    101.00000 (binary) or 5.0 (hex)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           AYINT

FUNCTION:       CONVERT FLOATING POINT TO INTEGER

PREPARATION:    FACC contains floating point number (-32768<=n<=32767)

RESULT:         FACMO ($66) contains signed integer (msb)

FACLO ($67) contains signed integer (lsb)

ERROR:          ?ILLEGAL QUANTITY ERROR if FACC too big.

EXAMPLE:        JSR AYINT      ;INT(FACC)

LDA $66        ;MSB

LDA $67        ;LSB

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           GIVAYF

FUNCTION:       CONVERT INTEGER TO FLOATING POINT

PREPARATION:    .A contains signed integer (msb)

.Y contains signed integer (lsb)

RESULT:         FACC contains floating point number

EXAMPLE:        LDA #>INTEGER

LDY #<INTEGER

JSR GIVAYF     ;FLOAT (A,Y)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FOUT

FUNCTION:       CONVERT FLOATING POINT TO ASCII STRING

PREPARATION:    FBUFFR ($100) contains ASCII string (null terminated)

.A contains pointer to string (lsb)

.Y contains pointer to string (msb)

EXAMPLE:        JSR FOUT       ;CONVERT FACC TO STRING AT $100

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           VAL_1

FUNCTION:       CONVERT ASCII STRING TO FLOATING POINT

PREPAPATION:    INDEX1 ($24,$25) contains pointer to string

.A contains length of string

SPECIAL NOTES:  String *MUST* be in var bank. Any invalid character

terminates conversion when encountered (i.e., acts

like a terminator).

RESULT:         FACC contains floating point number

EXAMPLE:        LDA #<POINTER

LDY #>POINTER

STA INDEX1     ;SET POINTER TO STRING

STY INDEX1+1

LDA #LENGTH    ;SET STRING LENGTH

JSR VAL_1      ;FACC = VAL(STRING)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           GETADR

FUNCTION:       CONVERT FLOATING POINT TO ADDRESS

PREPARATION:    FACC contains floating point number (0<=n<=65535)

RESULT:         POKER ($16,$17) contains unsigned integer address

ERROR:          ?ILLEGAL QUANTITY ERROR if FACC too big.

EXAMPLE:        JSR GETADR     ;ADR(FACC)

LDA $16        ;LSB

LDA $17        ;MSB

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FLOATC

FUNCTION:       CONVERT ADDRESS TO FLOATING POINT

PREPARATION:    FACHO  ($64) contains address (msb)

FACMOH ($65) contains address (lsb)

.X contains exponent ($90 always)

.C=1 if positive (always)

RESULT:         FACC contains floating point number

ERROR:          ?OVERFLOW ERROR if FACC too big.

EXAMPLE:        LDA #<ADDRESS

LDY #>ADDRESS

STA FACMOH     ;SET ADDRESS

STY FACHO

LDY #$90       ;EXPONENT

SEC            ;POSITIVE

JSR FLOATC     ;FLOAT ADDRESS

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FSUB

FUNCTION:       FACC = MEMORY - FACC

PREPARATION:    FACC contains floating point subtrahend

.A = pointer (lsb) to packed floating point minuend

.Y = pointer (msb) to packed floating point minuend

SPECIAL NOTES:  The minuend *MUST* be in VARBANK in packed

format. FSUB calls CONUPK to normalize it.

RESULT:         FACC contains floating point difference

ERROR:          ?OVERFLOW ERROR if FACC too big.

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* MINUEND

JSR FSUB       ;SUBTRACT MEM FROM FACC, DIFF IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FSUBT

FUNCTION:       FACC = ARG - FACC

PREPARATION:    FACC contains floating point subtrahend

ARG contains floating point minuend

SPECIAL NOTES:  This routine is similar to FSUB. The only difference

is the call to CONUPK. (FSUBT assumes you have

already loaded ARG with unpacked minuend.)

RESULT:         FACC contains floating point difference

ERROR:          ?OVERFLOW ERROR if FACC too big.

EXAMPLE:        JSR FSUBT      ;SUBTRACT ARG FROM FACC, DIFF IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FADD

FUNCTION:       FACC = MEMORY + FACC

PREPARATION:    FACC contains floating point addend

.A = pointer (lsb) to packed floating point addend

.Y = pointer (msb) to packed floating point addend

SPECIAL NOTES:  The second addend *MUST* be in VARBANK in

packed format. FADD calls CONUPK to normalize it.

RESULT:         FACC contains floating point sum

ERROR:          ?OVERFLOW ERROR if result too big

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* ADDEND

JSR FADD       ;ADD MEMORY TO FACC, SUM IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FADDT

FUNCTION:       FACC = ARG + FACC

PREPARATION:    FACC contains floating point addend

ARG  contains floating point addend

ARISGN  ($70) contains EOR(FACSGN,ARGSGN)

.A   contains FACEXP

SPECIAL NOTES:  This routine is similar to FADD. The only

difference is the call to CONUPK.

*********************************************

* You *MUST* put resultant sign in ARISGN.  *

* You *MUST* load FACEXP ($63) immediately  *

* before call so that status flags are set! *

*********************************************

RESULT:         FACC contains floating point sum

ERROR:          ?OVERFLOW ERROR if result too big

EXAMPLE:        LDA FACSGN

EOR ARGSGN

STA ARISGN     ;SET RESULTANT SIGN

LDA FACEXP     ;SET STATUS FLAGS PER FACEXP

JSR FADDT      ;ADD ARG TO FACC, SUM IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FMULT

FUNCTION:       FACC = MEMORY * FACC

PEPARATION:     FACC contains floating point multiplier

.A = pointer (lsb) to packed float. point multiplicand

.Y = pointer (msb) to packed float. point multiplicand

SPECIAL NOTES:  The multiplicand *MUST* be in VARBANK in

packed format. FMULT calls CONUPK to normalize it.

RESULT:         FACC contains floating point product

ERROR:          ?OVERFLOW ERROR if result too big

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* MULTIPLICAND

JSR FMULT      ;MULTIPLY MEM BY FACC, PRODUCT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FMULTT

FUNCTION:       FACC = ARG * FACC

PREPARATION:    FACC contains floating point multiplier

ARG  contains floating point muitiplicand

SPECIAL NOTES:  This routine is similar to FMULT. The only difference

is the call to CONUPK. (FMULTT assumes you have

already loaded ARG with unpacked multiplicand.)

RESULT:         FACC contains floating point product

ERROR:          ?OVERFLOW ERROR if result too big

EXAMPLE:        JSR FMULTT     ;MULTIPLY ARG BY FACC, PRODUCT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FDIV

FUNCTION:       FACC = MEMORY / FACC

PREPARATION:    FACC contains floating point divisor

.A = pointer (lsb) to packed floating point dividend

.Y = pointer (msb) to packed floating point dividend

SPECIAL NOTES:  The dividend *MUST* be in VARBANK in packed

format. FDIV calls CONUPK to normalize it.

RESULT:         FACC contains floating point quotient

ERROR:          ?DIVISION BY ZERO ERROR if FACC zero

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* DIVIDEND

JSR FDIV       ;DIVIDE MEM BY FACC, QUOTIENT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FDIVT

FUNCTION:       FACC = ARG / FACC

PREPARATION:    FACC contains floating point divisor

ARG  contains floating point divldend

ARISGN ($70) contains EOR(FACSGN,ARGSGN)

.A contains FACEXP

SPECIAL NOTES:  This routine is similar to FDIV. The only difference

is the call to CONUPK. (FDIVT assumes you have

already loaded ARG with unpacked dividend.)

*********************************************

* You *MUST* put resultant sign in ARISGN.  *

* You *MUST* load FACEXP ($63) immediately  *

* before call so that status flags are set! *

*********************************************

RESULT:         FACC contains floating point quotient

ERROR:          ?DIVISION BY ZERO ERROR if FACC zero

EXAMPLE:        LDA FACSGN

EOR ARGSGN

STA ARISGN     ;SET RESULTANT SIGN

LDA FACEXP     ;SET STATUS FLAGS PER FACEXP

JSR FDIVT      ;DIVIDE ARG BY FACC, QUOTIENT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           LOG

FUNCTION:       FACC = LOG(FACC)      natural logarithm (base e)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point logarithm

ERROR:          ?ILLEGAL QUANTITY ERROR if FACC negative or zero

EXAMPLE:        JSR LOG        ;FACC = LOG(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           INT

FUNCTION:       FACC = INT(FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point greatest integer

EXAMPLE:        JSR INT        ;FACC = INT(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           SQR

FUNCTION:       FACC = SQR(FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point square root

ERROR:          ?ILLEGAL QUANTITY ERROR if FACC negative

EXAMPLE:        JSR SQR        ;FACC = SQR(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           NEGOP

FUNCTION:       FACC = -FACC          (invert sign of FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point number with sign inverted

EXAMPLE:        JSR NEGOP       ;FACC = -FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FPWR

FUNCTION:       FACC = ARG ^ MEMORY

PREPARATION:    ARG contains floating point number

.A = pointer (lsb) to packed floating point power

.Y = pointer (msb) to packed floating point power

SPECIAL NOTES:  The power *MUST* be in ROM or SYSTEM RAM in packed

format as FPWR calls MOVFM to unpack it into FACC.

RESULT:         FACC contains floating point result

ERROR:          ?ILLEGAL QUANTITY ERROR if ARG negative

?OVERFLOW ERROR if result too big

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* POWER

JSR FPWR       ;COMPUTE ARG ^ MEM, RESULT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FPWRT

FUNCTION:       FACC = ARG ^ FACC

PREPARATION:    ARG  contains floating point number

FACC contains floating point power

.A   contains FACEXP

SPECIAL NOTES:  This routine is similar to FPWR. The only difference

is the call to MOVFM. (FPWRT assumes you have already

loaded FACC with unpacked power.)

*********************************************

* You *MUST* load FACEXP ($63) immediately  *

* before call so that status flags are set! *

*********************************************

RESULT:         FACC contains floating point result

ERROR:          ?ILLEGAL QUANTITY ERROR if ARG negative

?OVERFLOW ERROR if result too big

EXAMPLE:        LDA FACEXP     ;SET STATUS FLAGS PER FACEXP

JSR FPWRT      ;COMPUTE ARG ^ FACC, RESULT IN FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           EXP                   (compute e ^ FACC)

FUNCTION:       FACC = EXP (FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains fIoating point result

ERROR:          ?OVERFLOW ERROR if FACC too big

EXAMPLE:        JSR EXP        ;FACC = EXP(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           COS

FUNCTION:       FACC = COS(FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point cosine (in radians)

EXAMPLE:        JSR COS        ;FACC = COS(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           SIN

FUNCTION:       FACC = SIN(FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point sine (in radians)

EXAMPLE:        JSR SIN        ;FACC = SIN(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           TAN

FUNCTION:       FACC = TAN(FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point tangent (in radians)

EXAMPLE:        JSR TAN        ;FACC = TAN(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           ATN

FUNCTION:       FACC = ATN(FACC)

PREPARATION:    FACC contains floating point number

RESULT:         FACC contains floating point arctangent (in radians)

EXAMPLE:        JSR ATN        ;FACC = ATN(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           ROUND            (round to 40 bits of precision)

FUNCTION:       FACC = FACC + FACOV(msb)

PREPARATION:    FACC contains floating point number

FACOV(msb) contains 'extra' precision

RESULT:         none if FACC zero or FACOV(msb) zero

one extra bit ADDED to FACC lsb if FACOV(msb) is set

EXAMPLE:        JSR ROUND      ;ROUND FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           ABS                     (make FACSGN(msb) = $00)

FUNCTION:       FACC = ABS(FACC)

PREPARATION:    FACC contains (SIGNED) floating point number

RESULT:         FACC contains (POSITIVE) floating point

EXAMPLE:        JSR ABS        ;FACC = ABS(FACC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           SGN

FUNCTION:       .A = SGN(FACC)

PREPARATION:    FACC contains floating point number

RESULT:         .A --> $FF if FACC negative (FACC < 0)

$00 if FACC zero     (FACC = 0)

$01 if FACC positive (FACC > 0)

(status flags reflect contents of .A, carry invalid)

EXAMPLE:         JSR SGN       ;SGN(FACC)

; BEQ will trap =0

; BNE will trap <>0

; BMI will trap <0

; BPL will trap >=0     etc.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           FCOMP                 (compare FACC with MEMORY)

FUNCTION:       .A = FCOMP(FACC,MEMORY)

PREPARATION:    FACC contains floating point number

.A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:  The number *MUST* be in ROM, or RAM currently in

context below ROM, in PACKED format.

*** FACOV is significant!

RESULT:         .A --> SFF if FACC < MEMORY

$00 if FACC = MEMORY

$01 if FACC > MEMORY

(status flags reflect contents of .A, carry invalid)

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* NUMBER

JSR FCOMP      ;COMPARE FACC WITH MEMORY

; BEQ will trap FACC =  MEM

; BNE will trap FACC <> MEM

; BMI will trap FACC <  MEM

; BPL will trap FACC >= MEM    etc.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           RND0

FUNCTION:       FACC = random floating point number (0<n<1)

PREPARATION:    .A --> $00 to generate a 'true' random number

$01 to generate next random number in sequence

$FF to start a new sequence of random numbers

based upon current contents of FACC.

SPECIAL NOTES:  *MUST* be called with the system bank in context.

*MUST* load .A immediately before call so that status

flags reflect contents of .A

RESULT:         FACC = floating point random number

EXAMPLE:        LDA #$FF  ;START REPRODUCEABLE SEQUENCE BASED ON FACC

JSR RND0

LDA #$01

JSR RND0  ;GENERATE (FIRST) RANDOM NUMBER IN SEQUENCE

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           CONUPK

FUNCTION:       ARG = UNPACK(RAM_CONSTANT)

PREPARATION:    .A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:  The number *MUST* be in VARBANK or SYSTEM RAM in

packed format.

RESULT:         ARG    loaded with normalized floating point number

ARISGN ($6F) contains EOR(FACSGN,ARGSGN)

.A   contains FACEXP (status reflects contents of .A)

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* NUMBER

JSR CONUPK     ;LOAD ARG

; BEQ traps ARG = $00

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           ROMUPK

FUNCTION:       ARG = UNPACK(ROM_CONSTANT)

PREPARATION:    .A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:  The number *MUST* be in ROM or SYSTEM RAM currently

in context (otherwise identical to CONUPK).

RESULT:         ARG    loaded with normalized floating point number

ARISGN ($6F) contains EOR(FACSGN,ARGSGN)

.A   contains FACEXP (status reflects contents of .A)

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* NUMBER

JSR ROMUPK     ;LOAD ARG

; BEQ traps ARG = $00

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           MOVFRM

FUNCTION:       FACC = UNPACK(RAM_CONSTANT)

PREPARATION:    .A = pointer (lsb) to packed floating point number

.Y = pointer (msb) to packed floating point number

SPECIAL NOTES:  The number *MUST* be in VARBANK or SYSTEM RAM in

packed format.

RESULT:         FACC  loaded with normalized floating point number

FACOV ($71) cleared

EXAMPLE:        LDA #<POINTER

LDY #>POINTER  ;SET POINTER TO *PACKED* NUMBER

JSR MOVFRM     ;LOAD FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           MOVFA

FUNCTION:       FACC = ARG

PREPARATION:    ARG contains floating point number

RESULT:         FACC  contains same number as ARG

FACOV ($71) cleared

.A    contains FACEXP (but status invalid!)

EXAMPLE:        JSR MOVFA      ;COPY ARG TO FACC

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NAME:           MOVAF

FUNCTION:       ARG = FACC

PREPARATION:    FACC contains floating point number

RESULT:         FACC will be ROUNDed and FACOV cleared.

ARG  contains same number as FACC

.A   contains FACEXP (but status invalid!)

EXAMPLE:        JSR MOVAF      ;COPY FACC TO ARG

3.5. C65 DOS Documentation

DIRECTORY HEADER DEFINITION

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0      TRACK mumber which points to the 1st dir. sector

1      SECTOR number which points to the 1st dir. sector

2      Disk format version number, which is currently 'D'

512 byte sectors 20 per track

20 Sectors per track

40 Tracks per side

2 sides (note they're inverted from normal MFM disk)

3      Must = 0

4      Bytes 4 thru 21 contain the volume name (label)

22      Bytes 22 and 23 contain the disk id (fake)

24      Must contain an $A0

25      DOS version number (CBMDOS = 1, 1581 = 3)

26      Format version number (currently = 'D' (fake))

27      Bytes 27 thru 28 = $A0

29      NOT USED AT THIS TIME

30      NOT USED AT THIS TIME

31      NOT USED AT THIS TIME

32      NOT USED AT THIS TIME

33      NOT USED AT THIS TIME

34      Track number which points to this directory header

35      Sector number which points to this directory header

36      Bytes 36 thru 255 are not used at this time

NOTE:  If  this  is a subdirectory header then BYTES 32 and 33 contain

the  TRACK & SECTOR number of the DIRECTORY SECTOR that points to this

DIRECTORY HEADER.  See the partition command for a better description.

If this is the ROOT header then they will contain a $00.

BAM DEFINITION

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0      Track link for next bam sector, if last then end of BAMs

1      Sector link

2      Format type this disk was formatted under

3      Compliment version number of byte 2 above

4-5     Disk ID used when this disk was formatted

6      I/O byte used as follows;

BIT 7- When set Verify is performed after each disk write

BIT 6- Perform CRC check (not used by CBDOS)

BIT 1- Huge relative files disabled

7      Auto loader flag (not used by CBDOS)

8-15    Not used at this time by any CBM DOS versions

16-255   BAM image

BAM IMAGE

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0      Number of free sectors on this track

1      MSB flag for sector  7, LSB flag for sector  0

2      MSB flag for sector 15, LSB flag for sector  8

3      MSB flag for sector 23, LSB flag for sector 16

4      MSB flag for sector 31, LSB flag for sector 24

5      MSB flag for sector 39, LSB flag for sector 32

DIRECTORY SECTOR DEFINITION

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0      TRACK  -- Points to the next directory track.

1      SECTOR -- Points to the next directory sector.

(IF TRACK = 0 THEN THIS IS THE LAST DIRECTORY SECTOR)

FILE ENTRY DESCRIPTION

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0      File status byte which is used as follows:

BIT 7- Set indicates properly closed file

BIT 6- File is locked (read only)

BIT 5- Save with replace is CURRENTLY in effect,

when file is closed this bit is deleted.

BIT 4- NOT USED AT THIS TIME

BIT 3- Bits 3 thru 0 are used to indicate the filetype:

0=DEL, 1=SEQ, 2=PRG, 3=USER, 4=REL, 5=CBM, 6=not used

7=used by dos to represent DIRECT type of file access

1      TRACK - link to the 1st sector of data for this file.

2      SECTOR - link to the 1st sector of data for this file.

3      Bytes 3-18 contain the filename in ASCII, padded with $A0

19      Side Sector TRACK link for relative files

GEOS - Track number of GEOS file header

20      Side Sector SECTOR link for relative files

GEOS - Sector number of GEOS file header

21      Record size for relative files

GEOS - File structure type 0=SEQ, 1=VLIR

22      GEOS - FILE TYPES:

13 = Swap file     12 = System boot   11 = Disk device

10 = Input device  09 = Printer       08 = Font

07 = Appl. data    06 = Applications  05 = Desk Acc.

04 = System        03 = Basic data    02 = Assembly

00 = Not GEOS

23      Not used by CBM DOS previous to CBDOS

GEOS - DATE: Year last modified (offset from 1990)

CBDOS- Bits 7-4 contain the upper 4 bit's from the

file type byte (see byte 0 above) for the

UNNEW, UNSRATCH commands used by CBDOS

24      Not used by CBM DOS previous to CBDOS

GEOS - DATE: Month last modified (1 thru 12)

CBDOS- Bit's 7 thru 4 contain the lower 4 bit's from the

file type byte (see byte 23 above)

25      GEOS - DATE: Day last modified (1 thru 31)

26      TRACK (from 1) for the save with replace file

GEOS - DATE: Hour last modified (0 thru 23)

27      SECTOR (from 2) for the save with replace file

GEOS - DATE: Minute last modified (0 thru 59)

28      LSB of the # of sectors used by this file

29      MSB of the # of sectors used by this file

NOTE: Each sector in the directory contains 8 entries of 32 bytes each.

SIDE SECTOR FORMAT DEFINITION

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0      Next Side Sector TRACK  link ($FF if last)

1      Next Side Sector SECTOR link

2      Side Sector number

If this is a SUPER SIDE SECTOR then this contains an $FE

(see the description of the SUPER SIDE SECTOR below)

3      Record Size

4-5     TRACK & SECTOR link of Side Sector number 0

6-7     TRACK & SECTOR link of Side Sector number 1

8-9     TRACK & SECTOR link of Side Sector number 2

10-11    TRACK & SECTOR link of Side Sector number 3

12-13    TRACK & SECTOR link of Side Sector number 4

14-15    TRACK & SECTOR link of Side Sector number 5

16-17    TRACK & SECTOR link of the DATA BLOCK #0

18-19    TRACK & SECTOR link of the DATA BLOCK #l

etc...

NOTE: There are 91 groups to the largest file that this DOS can handle.

SUPER SIDE SECTOR FORMAT DEFINITION

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0      Next Side Sector TRACK link (SFF if last)

1      Next Side Sector SECTOR

2      Contains an SFE to indicate this is a SUPER SIDED SECTOR

3-4     TRACK & SECTOR link of Side Sector number 0

5-6     TRACK & SECTOR link of Side Sector number 1

7-8     TRACK & SECTOR link of Side Sector number 2

9-10    TRACK & SECTOR link of Side Sector number 3

11-12    TRACK & SECTOR link of Side Sector number 4

13-14    TRACK & SECTOR link of Side Sector number 5

.....    ...........................................

253-254   TRACK & SECTOR link of Side Sector number 125

NOTE:There are 91 groups to the largest file that this DOS can handle.

DATA SECTOR DEFINITION

----------------------------------------------------------------------

BYTE                           DESCRIPTION

------ --------------------------------------------------------------

0-1     TRACK and SECTOR link to the next data block. If track=0

then sector contains the number of bytes used in this

sector (which will always be at least 2 on the last block

for the T&S link bytes).

NOTE: Used by DEL, SEQ, PRG, REL (data blocks) and USR.

;*-------------------------------------------------------------------*

;* Format a track                                                    *

;* 10 sectors per track numbered 1-10, 512 byte sectors              *

;*-------------------------------------------------------------------*

;*  12 Sync marks                            00                      *

;*   3 Header ID marks w/missing clock       A1                      *

;*   1 Header ID                             FE                      *

;*   4 Header bytes                          Track                   *

;*                                           Side                    *

;*                                           Sector                  *

;*                                           Sector size             *

;*   2 Header CRC bytes                      xx,xx                   *

;*  22 Data gap bytes                        4E                      *

;*  12 Sync marks                            00                      *

;*   3 Data block ID marks w/missing clock   A1                      *

;*   1 Data block ID                         FB                      *

;* 512 Data block fill bytes                 00                      *

;*   2 Data block CRC bytes                  xx,xx                   *

;*  24 Sector gap bytes                      4E                      *

;*-------------------------------------------------------------------*

;*-------------------------------------------------------------------*

;* Calculate the 2 bytes CRC for each sector header of an entire     *

;* track of 10 sectors. AXYZ are trashed.                            *

;*                                                                   *

;*   This routine is based on the Cyclical Redundancy Check          *

;*   on the polynomial: A^16+A^12+A^5+1.                             *

;*                                                                   *

;*     HEADER contains TRACK,SIDE,SECTOR,2 [sector size]             *

;*                                                                   *

;*     DO WHILE ne = 0                                               *

;*       DO FOR each bit in the data byte (.a) [from lsb to msb]     *

;*         IF (LSB of crc) eor (LSB or data)                         *

;*           THEN CRC = (CRC/2) EOR polynomial                       *

;*         ELSE CRC = (CRC/2)                                        *

;*         END IF                                                    *

;*       LOOP                                                        *

;*     LOOP                                                          *

;*-------------------------------------------------------------------*

;*-------------------------------------------------------------------*

;*     SIDE           = (LogicalSector >= 20) AND 1                  *

;*     TRACK          = LogicalTrack - 1                             *

;*     StartingSector = SIDE * 20                                    *

;*     SECTOR         = (LogicalSector - StartingSector) / 2 + 1     *

;*     HALF           = (LogicalSector - StartingSector) AND 1       *

;*-------------------------------------------------------------------*

C65 Partition and Subdirectory Syntax

This  specification  describes a _proposed_ C65 partition/subdirectory

parser.

OPEN la,fa,sa, "[#]/path/:filename"

OPEN la,fa,15, "<cmd>#/path/:[cmd_string]"

where: #             is an optional "drive" number, 0-3.

/path/        is a partition or subdirectory name

:             delimits the path from the filename

and:   <cmd>         is a DOS command (such as I,N,S,C, etc.)

(cmd_string)  is an optional string required by some commands.

The  first  example  illustrates a typical filename specification, the

second example illustrates a command channel instruction.

OPEN la,fa,sa, "0/SUBDIR1/SUBDIR2/:FILE,S,W"

Action taken                           Why

------------------------------------ -------------------------------

1. Select the "root"                 0

2. Find & enter two subdirectories   /SUBDIR1/SUBDIR2/:

(the trailing "/" is required

to be compatible with CMD?)

3. Create & open file for writing    FILE,S,W

The "root" or "drive number", path, and ":" are all optional.  If they

are  omitted,  the  file  is  opened  in  the current partition.  Some

similar, and legal, syntaxes are:

OPEN la,fa,sa, "FILE,S,W"           (create "FILE" in current part)

OPEN la,fa,sa, ":FILE,S,W"          (create "FILE" in current part)

OPEN la,fa,sa, "0:FILE,S,W"         (create "FILE" in current part)

OPEN la,fa,sa, "/SUBDIR/ FILE,S,W"  (from current partition, enter

"SUBDIR" and create "FILE")

OPEN la,fa,sa, "//SUBDIR/:FILE,S,W" (from Root partition, enter

"SUBDIR" and create "FILE")

OPEN la,fa,sa, "@0/SUBDIR/:FILE"    (open "FILE" in "SUBDIR" for

writing)

Some questionable syntaxes, and their affect, are:

OPEN la,fa,sa, "0FILE,S,W"          (this would create file "0FILE")

OPEN la,fa,sa, "/SUBDIR/FILE,S,W"   (creates file "/SUBDIR/FILE"

OPEN la,fa,sa, "@0:FILE,S,W"        (open filr "FILE" in current

partition for writing)

OPEN la,fa,sa, "/0:FILE,S,W"        (? should create file "0:file",

note this is not the cmd chnl)

Some legal commands:

OPEN la,fa,sa,"I0"                  (initialize current partition)

OPEN la,fa,sa,"I//"                 (initialize Root)

OPEN la,fa,sa,"I0/SUBDIR/:"         (enter "SUBDIR" and initialize)

OPEN la,fa,sa,"N0/SUBDIR/:NAME,ID"  (enter "SUBDIR" and "new" it)

OPEN la,fa,sa,"S0/SUBDIR/:FILE"     (delete "FILE" in "SUBDIR")

OPEN la,fa,sa,"/0:SUBDIR"           (1581 partition select, "/" in

this context is a command

itself)

Some  proposed general rules,  designed to be compatible with both the

1581 subpartitioning syntax and CMD syntax:

1. The  name  of  a  subdirectory  must always be separated from the

filename by a colon (":").

2. Each subdirectory name must be delimited by a slash ("/").

3. To select Root directory (partiton),  specify two slashes ("//").

This allows older applications specifying the drive number ("0:")

to be run in a partition.

CURRENT PARTITION ROUTINES

Create Partition:

"/0:PAR_NAME,"+(START-TRK)+(START-SECTOR)+(LO-BLKS)+(HI-BLKS)

Select Partition:

"/0:PAR_NAME"  will select given filname as subdirectory

"/0"           will select root directory

SELECT PARTITION

This  routine  will  allow  the user to quickly select partition paths

using  the normal SA values other than 15.  To use this new method the

user  opens  the  file  using  a  normal  SA  and the filename MUST be

structured as follows:

"/<drive>:PATH_1/PATH_2/PATH_3..... ETC"

If the dos does not find one of the filenames in the file path  stream

it will check to see if the file exists in the current  directory  and

if it does it will open the file in the normal method as it does now.

;*********************************************************************

;*                           FILE_COMMANDS                           *

;*                                                                   *

;* The following set of command channel routines were added to allow *

;* the user a graceful way of manipulating files:                    *

;*                                                                   *

;*      "F-L"  Locate a file to prevent it from being scratched      *

;*      "F-U"  Unlock a file and allow it to be scratched            *

;*      "F-R"  Restore a file after it has been scratched            *

;*                                                                   *

;* Following  each command above is the drive number,  followed by a *

;* colon then followed by the filename(s).  For example, to lock all *

;* the files on drive 0 you would send the following file command:   *

;*                                                                   *

;*             OPENXX,XX,15,"F-L0:*"                                 *

;*                                                                   *

;*             OPENXX,XX,15,"F-L0:FNAME,FNAME1,FNAME2,... ETC"       *

;*********************************************************************

;*********************************************************************

;*                           BLOCK STATUS                            *

;*                                                                   *

;* Syntax: "B-S:CHANNEL NUMBER, DRIVE NUMBER, TRACK, SECTOR"         *

;*                                                                   *

;* Then check error channel for normal errors then get one byte from *

;* from  the  channel number.  If it is a 0 then the sector is free. *

;* 1 indicates the sector is in use.                                 *

;*                                                                   *

;* This command was added to enable an easy method of finding out if *

;* a  given  track  or sector is currently marked as being used in a *

;* drive's BAM or not.                                               *

;*                                                                   *

;*                            CBDOS CHGUTIL                          *

;*                                                                   *

;*     COMMAND                  COMMENTS              DRIVES USED ON *

;* "U0>B"+chr$(n)  b  =  set fast/slow serial bus          1581      *

;* "U0>D"+chr$(n)  d  =  set dirsecinc                     CBDOS     *

;* "UO>H"+chr$(n)  h  =  set head selection 0,1            1571      *

;* "U0>M"+chr$(n)  m  =  set dos mode                      1571      *

;* "U0>R"+chr$(n)  r  =  set dos retries on errors      1571,1581    *

;* "UO>S"+chr$(n)  s  =  set secinc                  1571,1581,CBDOS *

;* "U0>V"+chr$(n)  v  =  set verify on/off              1581,CBDOS   *

;* "U0>?"+chr$(n)  ?  =  set device number                 CBDOS     *

;* "U0>L"+chr$(n)     =  set large rel files on/off        CBDOS     *

;* "U0>MR"+ xx        =  perform memory read               1581      *

;* "U0>MW"+ xx        =  perform memory write              1581      *

;*  12345                                                            *

;*      ^--------------- CMDSIZ points to end of starting string @1  *

;*********************************************************************

FLOPPY DISK CONTROLLER ERRORS

IP  FDC  DESCRIPTION

--  ---  -----------

0   (0)  no error

20  (2)  can't find block header

23  (5)  checksum error in data

25  (7)  write-verify error

26  (8)  write w/ write protect on

27  (9)  crc error in header

Information description

-----------------------

1   files scratched

2   selected partition

3   files locked

4   files unlocked

5   files restored

Parameter errors

----------------

30  general syntax

31  invalid command

32  long line

33  invalid filname

34  no filenames given

Relative file errors

--------------------

50  record not present

51  overflow in record

52  file too large

53  big relative files disabled

Open routine errors

-------------------

60  file open for write

61  file not open

62  file not found

63  file exists

64  file type mismatch

Sector management errors

------------------------

65  no block

66  illegal track or sector

67  illegal system t or s

General channel/block errors

----------------------------

02  channel selected

70  no channels available

71  bam corrupted error

72  disk full

73  cbdos v1.0

74  drive not ready

75  format error

76  controller error

77  selected partition illegal

78  directory full

79  file corrupted

3.6. C64DX RS-232 DRIVER

00A7   rs232_status             - UART status byte

00A8   rs232_flags              - open flag, xon/xoff status

- b7: channel open      (reset)

- b6: flow control      (1=x-line)

- b5: duplex            (1=half)

- b1: XOFF received

- b0: XOFF sent

00A9   rs232_jam                - system character to xmit

00AA   rs232_xon_char           - XON character (null=disabled)

00AB   rs232_xoff_char          - XOFF character (null=disabled)

00B0   rs232_xmit_empty         - xmit buffer empty flag (0=empty)

00B1   rs232_rcvr_buffer_lo     - lowest page of input buffer

00B2   rs232_rcvr_buffer_hi     - highest page of input buffer

00B3   rs232_xmit_buffer_lo     - lowest page of output buffer

00B4   rs232_xmit_buffer_hi     - highest page of output buffer

00B5   rs232_high_water         - point at which receiver XOFFs

00B6   rs232_low_water          - point at which receiver XONs

00C4   rs232_rcvr_head          - pointer to end of buffer

00C6   rs232_rcvr_tail          - pointer to start of buffer

00C8   rs232_xmit_head          - pointer to end of buffer

00CA   rs232_xmit_tail          - pointer to start of buffer

RS-232 interrupt-driven handler

How  it  works:  when an RS232 channel is OPENed, buffers are flushed,

all flags and states are reset, and the receiver IRQ is enabled.  When

a byte is put into the xmit buffer by BSOUT,  the xmit IRQ is enabled.

The xmit IRQ is disabled whenever the xmit buffer is found to be empty

or  an  XOFF is received  (it is enabled whenever an XON is received).

CLOSE  will  hang until the xmit buffer is empty,  and BSOUT will hang

when the xmit buffer is full.  IRQs must be allowed by the user at all

times  (and especially during BSOUT calls) for proper operation.  (The

RS232  channel  will  work even if IRQs are disabled by the user,  but

thoughput will be reduced to the frame rate (normal system raster IRQ)

and the system can hang forever should the xmit buffer become full and

BSOUT is called with a byte to xmit).  A successful CLOSE will disable

all RS232 interrupts and re-init everything.

Note  that DOS calls disable both IRQ and NMI interrupts while the DOS

code is in context. The remote should be XOFFed to avoid loss of data.

Refer to the UART specification for register description  &  baud rate

tables.

Open an RS-232 channel

This is different from the usual C64/C128 command string.

1    2     3        4          5     6

Command string bytes:    baud|word|parity|stop(unused)|duplex|xline

4.0. C64DX DEVELOPMENT SOPPORT

Please  photocopy  the  attached  'C64DX PROBLEM REPORT' and use it to

report any problems.

If  you  have  any  requests  or recommendations,  please  send a good

description of it and explain why you want it.

+----------------------------------------+-------------------------------------+

|        C64 DX PROBLEM REPORT           | Date                                |

+----------------------------------------+-------------------------------------+

|Please complete this form as completely as possible and mail or express it to:|

|                                                                              |

| Commodore Business Machines, Inc.       Telephone: 215-431-9427              |

| 1200 Wilson Drive                       Fax:       215-431-9156              |

| West Chester,  PA    I9380              Email:     fred@cbmvax.commodore.com |

|                                                                              |

|                      Attention: Fred Bowen, Engineering                      |

+------------------------------------------------------------------------------+

|Company Name                                                                  |

+------------------------------------------------------------------------------+

|Company Address                                                               |

|                                                                              |

|                                                                              |

|                                                                              |

|                                                                              |

|                                                                              |

|                                                                              |

|                                                                              |

+----------------------------------------+-------------------------------------+

|Your Name                               |Your Phone                           |

+----------------------------------------+-------------------------------------+

|Your system                                                                   |

|                                                                              |

| Serial No.________ PCB rev_________ Software ver________ ROM Cksum__________ |

|                                                                              |

| 4510 rev__________ 4567 rev________ F011(DOS)___________ F018(DMA)__________ |

|                                                                              |

| Peripherals:                                                                 |

+---------------------+--------------------------------------------------------+

|Your problem         | Explain problem here and show how to cause it.  Attach |

|                     | sample program.                                        |

|  ____ C64 mode      |                                                        |

|  ____ C64DX mode    |                                                        |

|  ____ Hardware      |                                                        |

|  ____ Software      |                                                        |

|  ____ Mechanical    |                                                        |

|  ____ Documentation |                                                        |

|  ____ Compatibility |                                                        |

|                     | It happens:                                            |

|                     |  ____ all the time  ____ frequently  ____ occasionally |

+---------------------+--------------------------------------------------------+

|In your opinion, how I bad is the problem?      ____ Must fix, no workaround  |

|                                                                              |

|                                                ____ I can work around it     |

|                                                                              |

|  ____ Check here if you need to be contacted   ____ Minor problem            |

+------------------------------------------------------------------------------+

|Please leave this space blank                                                 |

|                                                                              |

|                                                                              |

|                                                                              |

|                                                                              |

|                                                                              |

|Number               Received               Contacted               Completed |

+------------------------------------------------------------------------------+

C64DX System Specification UPDATE

*   The Monitor parser now allows PETSCII input/conversion:

'A                prints ASC() value of character

>1800 'text       puts text into memory

LDA #'A

*   IRQ  runs  during  graphics  (Kernel finds its own base page). IRQ

still  does  not  run  during  DOS activity (not sure if they ever

will).

*   The  following Kernel Jump Table Entries have moved (and are still

subject to further changes):

FF05    nirq              ;IRQ handler

FF07    monitor_brk       ;BRK handler (Monitor)

FF09    nnmi              ;NMI handler

FF0B    nopen             ;open

FF0D    nclose            ;close

FF0E    nchkin            ;chkin

FF11    nckout            :ckout

FF13    nclrch            ;clrch

FF15    nbasin            :basin

FF17    nbsout            ;bsout

FF19    nstop             ;stop key scan

FF1B    ngetin            ;getin

FF1D    nclall            ;clall

FF1F    monitor_parser    ;monitor command parser

FF21    nload             ;load

FF23    nsave             ;save

FF25    talk

FF27    listen

FF29    talksa

FF2B    second

FF2D    acptr

FF2F    ciout

FF31    untalk

FF33    unlisten

FF35    DOS_talk

FF37    DOS_listen

FF39    DOS_talksa

FF3B    DOS_second

FF3D    DOS_acptr

FF3F    DOS_ciout

FF41    DOS_untalk

FF43    DOS_unlisten

FF45    Get_DOS

FF47    Leave_DOS

FF49    ColdStartDOS      <<<new

FF4B    WarmStartDOS      <<<new

2.1.2. German/Austrian Reyboard Layout

+----+   +----+----+----+----+   +----+----+----+----+   +----+----+----+----+

|RUN |   |ESC |ALT |ASC | NO |   | F1 | F3 | F5 | F7 |   | F9 | F11| F13|HELP|

|STOP|   |    |    |DIN |SCRL|   | F2 | F4 | F6 | F8 |   | F10| F12| F14|    |

+----+   +----+----+----+----+   +----+----+----+----+   +----+----+----+----+

+----+----+----+----+----+----+----+----+----+----+----+----+----+----+----+----+

|   <| !  | "  | # §| $  | %  | &  | ' /| (  | )  |  " |  ? |  ` |  ^ |CLR |INST|

|  >| 1  | 2  | 3  | 4  | 5  | 6  | 7  | 8  | 9  | 0  | + á| - '| ś [|HOME|DEL |

+----+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+----+

|  TAB  |    |    |    |    |    |  Z |    |    |    |    |  š |    |  \ | RSTR |

|       | Q  | W  | E  | R  | T  | Y  | U  | I  | O  | P  | @  | * +| ^ ]|      |

+----+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+------+

|CTRL|SHFT|    |    |    |    |    |    |    |    |    |  ™ |  Ž |  ' |  RETURN |

|    |LOCK| A  | S  | D  | F  | G  | H  | J  | K  | L  | : [| ; ]| = #|         |

+----+----+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+--+-+----+----+----+

| C= | SHIFT |  Y |    |    |    |    |    |    | < ;| > :| ? _| SHIFT|    |

|    |       | Z  | X  | C  | V  | B  | N  | M  | ,  | .  | / -|      |   |

+----+-------+-+--+----+----+----+----+----+----+----+----+-+--+-+----+----+----+

|                                            |    |    |    |    |

|                                            |    |   |   |   |

+--------------------------------------------+    +----+----+----+

Notes:

1/ The   operation   of   national  keyboards  is  identical  to  C128

implementation.  The ASCII/DIN key replaces the CAPS LOCK key,  and

can  be  toggled anytime to switch keyboard modes and automatically

change the display.

2/ The  national  keyboard  contains key legends for both national and

ASCII modes. The national legends appear on the right top/bottom of

the keys.

3/ The  German  keyboard  has  three  (3) "deadkeys".  They are accent

d'aigue,   accent  grave,  and  accent  circonflex.   Pressing  the

"deadkey"  followed  by  a  valid  vowel  or  accent character will

'build' the desired character:

accent d'aigue:      ‚

accent grave:        …, Š, —

accent circonflex:   , , Ś, ", –

4/ National  character  ROM graphic characters differ from the C64 and

ASCII (English) graphic character sets.

PAINT x, y [,color]

Working,  but  not  completely  to spec.  Uses draw pen

color and fills emptyness to any border.

RND(0)         Improved  for  better "randomness".  Uses unused POT of

second SID chip. PCB must allow lines to float.

SET DISK #     (without [TO #] parameter)  allows  user  to  clear DS$

message and specify which drive next DS$ comes from.

SET VERIFY <ON/OFF>

The new DOS65 defaults to verify-after-write OFF.  This

command works with 1581 drive, too.

*   Negative  coordinates  are  now allowed for all graphics commands.

Some  commands  require  their  arguments to be "onscreen" such as

PAINT.

*   BASIC errors now force text mode,  and  TYPE, LIST, DISK, KEYLOAD,

LOADIFF now catch all DOS errors. Autoboot filename=AUTOBOOT.C64DX.*

*   Opening  an  RS-232  channel,  command  string  allows setting new

features:

1      baud (0-16, where 16=MIDI rate)

2      word len

3      parity

4      stop bits (not used)

5      duplex

6      xline

7      xon char (0=incoming flow control disabled)

8      xoff char (0=outgoing flow control disabled)

9,10   input buffer pointer (page lo, hi)

11,12  output buffer pointer (page lo, hi)

13     high water mark (point at which xoff is xmitted)

14      low water mark (point at which xon is xmitted)

For  debug  purposes,  the border color will change if an RS232 buffer

overflow  occurs.  To  differentiate  between  a  GET# of a null and a

'no data' null, test bit 3 of STatus (same as C64).

*   Support for latest DOS controller chip,  F011D, includes error LED

blink  (border  color  still  changes  too,  for now).  Changes to

improve  FASTLOAD  speed  and  improve SAVE speed.  Will work with

F011C chip,  but error LED does not blink. Requires latest 'ELMER'

PAL  for  disk  LED  to work correctly for either controller Chip.

External  drive  LED  will not work correctly until new PCB & F016

chip  are  designed.  New  DOS  functions  include  COPY D0 TO D1,

ability  to  change  sector  skews for files (U0>S#) and directory

(U0>D#),  and  directory  compress  (i.e.,  empty trash)  via  "E"

command. Physical interleave is now 7.

*   The  DOS  COPY/CONCAT  bugs  have been fixed,  and COPY now allows

forms such as COPY D0,"*.SRC" TO D1,"*" and COPY D0,"*=SEQ" TO D1,

"*". Directory/partition paths not yet implemented, but will be.

The  following  changes/updates/fixes  have been made to the C64DX ROM

code  since the March 1,  1991 C64DX System Specification was printed.

Please make note of them. Current ROM as of this update is 910501.

CHAR       Now  works  to  spec.  and  supports the following imbedded

control  characters  (although some are buggy;  others  are

planned):

^F       6      flip

^I       9      invert

^O      15      overwrite

^R      18      reverse field on

146      reverse field off

^U      21      underline

^Y      25      tilt

^Z      26      mirror

When  specifying  a  character  set  from  ROM,  note  that

national  versions  of  the  C64DX  will  have the national

character  set  at  $39000  and  the  C64  character set at

$3DC00.  In  US/English  systems,  the  default  C64DX-mode

character set will be at $39000.

CLR ERR$   Clears BASIC error stuff, useful after a TRAP.

CURSOR [<ON/OFF>,] [column] [,row] [,style]

where: column,row = x,y logical screen position

style      = flashing (0) or solid (1)

ON, OFF    = to turn the cursor on or off

LINE x0, y0 [, [x1] [,y1]] ...

where: (x1,y1)=(x0,y0) if not specified, drawing a dot.

Additional coordinates (x2,y2), etc. draw a line

from the previous point.

LOADIFF "file" [,U#,D#]

Loads  an IFF picture from disk.  Requires a suitable graphic

screen to be already opened (this may change).  The file must

contain std IFF data in PRG file type. IFF pics can be ported

directly from Amiga (eg., using XMODEM).  Returns  'File Data

Error' if it finds data it does not like.

MOD (number, modulus)

New function.

MOUSE ON [,[port] [,[sprite] [,[hotspot] [,X/Yposition] ]]]

MOUSE OFF

where: port     = (1...3) for joyport 1, 2, or either (both)

sprite   = (0...7) sprite pointer

hotspot  = x, y offset in sprite, default 0,0

position = normal, relative, or angular coordinates

Defaults to sprite 0, port 2, last hotspot (0,0), and

position. Kernel doesn't let hotspot leave the screen.